3GPP TSG-RAN WG4 Meeting #116	R4-2510385

Bengaluru, India, 25th – 29th August 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	7.14.3.2

Document for:	Discussion

# Introduction

As per WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, In last meeting 3GPP TSG RAN WG4 Meeting #114, the outcome of the debate, encompassing consensuses and unresolved issues, were documented in WF [2]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for CSSF outside gap in CA/DC

Issue 1-2-1: revision on solution 1 (measure one serving CC per band)

Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618.

- Option 1 (vivo, OPPO): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Option 2 (Ericsson, ZTE, Nokia): Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’. - (ZTE)NW indication of interested SCC is meaningful even only multiple SCCs without SSB based reporting are configured in the band. Under such case, the NW indication is used to indicate which serving cell to be measured and reported at least. - (Nokia): UE shall follow network indication to measure the SCC if there is no SCCs on which UE is configured to report SSB based measurements in the band - Option 3 (Huawei): Discuss this revised Solution 1 update after the indication design is finalized. - Recommended WF - [Moderator]: Discuss between option 1 and option 2. If no agreement can be made, then try option 3 as compromise.

Firstly, regarding the concern about removing the phrase “if there are no SCCs on which the UE is configured to report SSB‑based measurements in band, or”, following offline discussions in the last meeting, we note that the viewpoint assumes that a UE does not need to perform measurements when no reporting configuration is valid.

Thee phrase actually refers to the below statements in Clause 9.2.3.2 of TS38.133.

- One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

We understand that the existing statements indicate that the selected SCC is determined by UE implementation when no SCC is configured for SS-RSRP measurement reporting. In other words, the UE is capable of performing measurements on one SCC when there is no PCC, no PSCC, and no SCC with reporting configuration in the same band.

However, it should be noted that Clause 9.2.3.2 defines the UE’s capability: “the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least …”, rather than prescribing the action that the UE must actually perform. Without the reporting configuration, the UE measurement is an agnostic assumption and cannot be verified.

In alignment with the above existing statements, the necessity of precluding configurations where no SCC measurement reporting is configured is unclear.

Secondly, regarding concerns from some companies in RAN2 and RAN4 that network indication is unnecessary, in our view:

- The motivation of this WI is to optimize (i.e., reduce) measurement delay in scenarios where multiple CCs are configured by MO/servingCellMO, each associated with reporting configurations.

- While in theory the network could configure MO/servingCellMO and/or reporting configurations only for the intended CC rather than all CCs, this approach is outside the scope of the WI, as it is contrary to the WI’s intention.

Given the above, the relevant question is: should the network be able to indicate which CC is to be measured, when multiple CCs are configured by MO/servingCellMO? Our answer: Yes.

Observation 1: In legacy, the UE is capable of performing measurements on one SCC when there is no PCC, no PSCC, and no SCC with reporting configuration in the same band. It defines a requirement on UE’s capability instead of the action that the UE must actually perform. Without the reporting configuration, the UE measurement is an agnostic assumption and cannot be verified.

Proposal 1: Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’, since the requirement only specifies the UE’s capability, consistent with the legacy behavior.

Observation 2: Any discussion should assume, as the prerequisite of this WI, that multiple CCs are configured by MO/servingCellMO, and that measurement report configurations associated with all MOs are valid.

Proposal 2: Under the prerequisite that of this WI, NW shall be able to indicate which CC to be measured provided that multiple CCs are configured by MO/servingcellMO. The signaling design may be lower layers signaling but up to RAN2 design.

Issue 1-2-2: Expand solution 1 to FR1 case

[Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

Recall Clause 7.6.4 in TS 38.133 as follows.

### 7.6.4 Minimum requirements for NR Carrier Aggregation For intra-band contiguous CA, only co-located deployment is applied. For intra-band non-contiguous NR carrier aggregation, the UE not capable of intraBandNR-CA-non-collocated-r18 shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below. For FR1 intra-band non-contiguous NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference as shown in table 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the UE shall be capable of handling at least a relative receive timing difference between slot timing of different carriers to be aggregated at the UE receiver as shown in table 7.6.4-1 below if UE indicates that it is capable of intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is provided. Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation Frequency Range; Maximum receive timing difference (µs). FR1; 31. FR2-1; 0.26. NOTE 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot.. For inter-band NR carrier aggregation, - the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers in FR1 and FR2-1 to be aggregated at the UE receiver as shown in table 7.6.4-2 below. - the UE shall be capable of handling at least a relative receive timing difference between subframe timing of all pairs of carriers in FR1 and FR2-2 to be aggregated at the UE receiver as shown in table 7.6.4-2 below. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2-1; 8 note1. Between FR1 and FR2-1; 25. Between FR1 and FR2-2; 25. NOTE1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA..

To our understanding, Clause 7.6.4 in TS 38.133 only interprets that , for FR1:

- Intra‑band contiguous CA is assumed to be co‑located.

- Intra‑band non‑contiguous CA may be either co‑located or non‑co‑located.

So, for serving component carriers, e.g., CC1 and CC2, if they are intra-band contiguous CA, CSSF enhancement can be applied in essence, since NW can assess the coherent relationship of two FR1 carriers to evaluate the feasibility of applying solution 1. Considering neighbour cell measurements under the following example configuration:

- Serving gNB: CC1(PCC) and CC2

- Neighbour gNB1: CC1

- Neighbour gNB2: CC2

If only CC1 is measured, following CSSF enhancement, the UE might not always detect gNB2 unless the network is aware of neighbour cell configurations and updates measurement configurations accordingly. This approach, however, could lead to increased signaling overhead.

In FR2, CC configuration variants are relatively limited from a network configuration perspective, and the above issue is unlikely in most cases. However, FR1 may encounter more complex CC configurations/combinations due to the greater variety of band/carrier implementations.

Observation 3: Clause 7.6.4 in TS 38.133 interprets the co‑location relationship for serving cells. However, neighbour cells may present more complex situations, particularly in FR1.

Proposal 3: While we have concerns regarding the practical applicability, we can compromise to support extension to FR1 if it is the majority view. However, the configuration for enabling/disabling the CSSF enhancement solution shall be separated for different FRs (e.g., enabling for FR2 shall not impact disabling for FR1).

Another underlying issue is inter-frequency layer measurement, which may have an intra-band relationship with the serving cells’ intra-frequency.

According to the agreements, measurements of neighbor cells that are intra-frequency with respect to the serving cells may follow CSSF Solution 1. That is, the UE can measure only one of those neighbor cells.

This raises the question: for an inter-frequency cell that is also intra-band relationship with the serving cells, can the measurements of the inter-frequency cell also follow CSSF Solution 1?

Proposal 4: RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency cell that is intra-band with serving cells.

# Conclusion

Observation 1: In legacy, the UE is capable of performing measurements on one SCC when there is no PCC, no PSCC, and no SCC with reporting configuration in the same band. It defines a requirement on UE’s capability instead of the action that the UE must actually perform. Without the reporting configuration, the UE measurement is an agnostic assumption and cannot be verified.

Observation 2: Any discussion should assume, as the prerequisite of this WI, that multiple CCs are configured by MO/servingCellMO, and that measurement report configurations associated with all MOs are valid.

Observation 3: Clause 7.6.4 in TS 38.133 interprets the co‑location relationship for serving cells. However, neighbour cells may present more complex situations, particularly in FR1.

Proposal 1: Not to remove the phrase ‘if there is no SCCs on which UE is configured to report SSB based measurements in band, or’, since the requirement only specifies the UE’s capability, consistent with the legacy behavior.

Proposal 2: Under the prerequisite that of this WI, NW shall be able to indicate which CC to be measured provided that multiple CCs are configured by MO/servingcellMO. The signaling design may be lower layers signaling but up to RAN2 design.

Proposal 3: While we have concerns regarding the practical applicability, we can compromise to support extension to FR1 if it is the majority view. However, the configuration for enabling/disabling the CSSF enhancement solution shall be separated for different FRs (e.g., enabling for FR2 shall not impact disabling for FR1).

Proposal 4: RAN4 to confirm CSSF Solution 1 can be applied to an inter-frequency cell that is intra-band with serving cells.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2508264, WF for NR_RRM_Ph5, Apple