3GPP TSG-RAN WG4 Meeting #116	R4-2510398

# Bangalore, India, August 25th – 29th 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 NR-NTN Phase 3 SAN demodulation requirements

Agenda item:	7.27.7

Document for:	Information

# Introduction

In RAN4#115 meeting, companies had agreement on few simulation assumptions for Rel-19 NR-NTN Phase 3 SAN demodulation requirement [1].  In this contribution, simulation results are delivered for comparison between OCC length-2 and length-4 for both NTN-TDLA and NTN-TDLC channel.

# Simulations

2.1	Simulation assumptions

Table 2.1-1: Test parameters

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| Channel bandwidth |  | 15 kHz SCS: 5MHz |
| HARQ | Maximum number of HARQ transmissions | 0 |
|  | RV sequence | 0,2,3,1 for FDD |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 and n4 for FDD |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Test Metric |  | SNR@70% Normalized throughput |

2.2	Simulation results

Table 2.2-1: Minimum requirements for PUSCH, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| TX | RX | Propagation conditions and correlation matrix | Number of UEs in OCC group with RV 0 | Slot aggregation | Fraction of max throughput | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | No OCC | OCC length-2 and OCC length-4 |
| 1 | 1 | NTN-TDLA100-200 Low | 2 | 2 | 70% | 2 | -4.12 | -4.0 |
|  |  |  | 4 | 4 |  |  | -7.48 | -7.63 |
| 1 | 2 | NTN-TDLA100-200 Low | 2 | 2 | 70% | 2 | -7.77 | -7.32 |
|  |  |  | 4 | 4 |  |  | -10.81 | -9.82 |
| 1 | 2 | NTN-TDLC100-200 Low | 2 | 2 | 70% | 2 | -9.55 | -9.53 |
|  |  |  | 4 | 4 |  |  | -12.21 | -11.85 |

# Conclusions

The initial simulation results are delivered.

# References

[1] R4-2508755, “Way Forward for [115][333] NR_NTN_Ph3_demod”, Ericsson, RAN4#115, Malta.