3GPP TSG-RAN4 Meeting #116	R4-2510417

# Bengaluru, India 25th – 29th August 2025

Title:	Views on Inter-UE CLI aspects and potential solutions for SBFD deployments

Source:	Qualcomm Incorporated

Agenda item:	7.21.2.2

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

Moreover, in RAN4#115, there were discussions on potential impact of inter-UE CLI adjacent channel interference and potential solutions when a legacy TDD is operating adjacent to SBFD, with specific focus on urban hotspot deployments. Accordingly, a WF was agreed in [2] entailing several the problem statement, discussion points, potential solutions and framework to assess each solution. In this contribution we share our views on the aspects raised in [2] and we conduct a feasibility analysis to each of the identified solutions.

# Problem statement

In [2], the following was captured regarding the coexistence between SBFD DU as an aggressor and NR TDD DL as a victim.

Based on Rel-18 study, the following conclusion is provided for Case 1 (aggressor SBFD DU victim NR TDD DL) in the conclusion part of Chapter 11 in TR 38.858: ### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1. Table 11.3.1-1: Case 1 co-existence conclusions Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations.. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (46 dBm to 53 dBm).. Urban Macro -> Urban Micro Scenario 4; FR1; No DL throughput degradation for both average throughput and cell edge throughput is observed.. Indoor -> Indoor Scenario 3 and Scenario 9; FR1 and FR2-1. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8; FR1 and FR2-1..

For the urban hotspot scenario, the observed throughput degradation is a resultant of several interference sources. One of those interference sources is coming from the geographically close UEs such that one SBFD-aware UE within a cluster (i.e., inside a building or same conference room) might be transmitting in its UL subband to its serving SBFD-capable BS, while another TDD legacy UE is receiving its downlink transmission in a DL slot.

Observation 1: One of the interference sources of the Urban Hotspot scenario is the inter-UE CLI from SBFD-aware UEs transmitting in the UL subband towards the adjacent legacy TDD UEs receiving DL transmissions.

# Adjacent channel coexistence simulations

The deployment scenario is the urban hotspot scenario, which is simply an urban macro scenario but instead of random UE deployment, it considers clustered UE deployment as given in Table E.2.1-3 in TR 38.858. It is noteworthy that Annex E assumes that cells fully or partly overlapping between the 2 networks share the same cluster. Thus, the aggressor UEs interfering on the geographically close victim UEs might have an impact on the throughput degradation of the victim legacy TDD network due to the inter-UE CLI. A visualization of such scenario is highlighted in Figure 1, where a single snapshot depicts the different scenarios that might lead to different interference statistics.

Figure 1 Visualization of possible urban hotspot scenario for legacy TDD and SBFD deployments.

To consider scenarios where the inter-UE interference would be impactful, we have considered 0% GS between the victim and aggressor network, which means that the clusters between the victim and aggressor network are always shared. In our view, such an assumption aligns from what is given in Note 1 in Table E.2.1-3 as shown below and as shown in Figure 2.

Table E.2.1-3: Cluster-based UE dropping parameters Parameter; Urban Hotspot. Cluster number per macro cell; 1 (Note 1). Cluster area; Circular area with 25 m radius. UE distribution; UEs dropped within a cluster are indoor and UEs dropped outside a cluster are outdoor. Indoor and outdoor ratio; 80% indoor and 20% outdoor (Note 2). Minimum (2D) distance; UE-UE: 1 m Cluster center – cluster center: 50 m Macro BS – cluster center: 60 m (Note 3). UE dropping; Evenly random dropping into the cluster with minimum UE-UE distance limitation. Note 1: Cells fully or partly overlapping between the 2 networks share the same cluster. Note 2: O-to-I penetration losses are not calculated for pairs of indoor UEs deployed in the same cluster area. For pairs of indoor UEs in different cluster areas, a single penetration loss component is calculated. Note 3: Consider the hexagonal grid of one of the two operators as the reference when dropping the cluster. The minimum distance between macro BS to cluster centre should be respected also for BSs belonging to the other operator.

Observation 2: To investigate the inter-UE CLI case in an extreme scenario, we consider each victim shares the cluster, in which UEs are dropped, with an aggressor cell.

Figure 2 Victim and aggressor deployments with different grid shift values.

In our simulation, we have aligned with Annex E assumptions with the following aspects being highlighted:

- 100 MHz TDD legacy network while DU SBFD aggressor network with 80 MHz and 20 MHz for downlink and uplink subband respectively.

- UE power class 3 (i.e., UE maximum conducted power = 23 dBm).

- Flat ACIR model such that BS ACLR/ACS = 45/46 dB & UE ACLR/ACS = 30/33 dB

- 80% of dropped UEs are indoor and dropped within the cluster.

- No Penetration loss modeled between UEs within the same cluster or outdoor UEs.

Before discussing the results, it is important to note several aspects that have an impact on the results. First, Annex E in TR 38.858 assumes that victim and aggressor cells share the same cluster if their geographical hexagonal boundaries cross. Moreover, no penetration loss is assumed between the UEs falling within a cluster. Such a pessimistic assumption might lead to overestimation of the interference, since there will be additional losses due to walls or UEs being scheduled that are in different rooms/ floors.

Observation 3: Based on Annex E assumptions, UEs falling in the same cluster will not experience any penetration losses, which ignores the fact that there might be additional losses due to wall penetration or UEs scheduled being in different rooms/ floors.

Another important factor impacting the inter-UE CLI interference is the aggressor UE ACLR and victim UE ACS. For the baseline case, we assume flat ACIR model adopting the minimum RAN4 RF requirements given in TS 38.104 and TS 38.101-1 for the BS and UE, respectively. This was considered as a worst-case assuming DU SBFD configuration. It is viable to consider DUD SBFD configuration as a more practical option, especially where there is an adjacent TDD network deployed. In that case, the flat ACIR model with legacy RAN4 requirements is unrealistic and yields over-estimated adjacent channel interference. More analysis of different ACLR/ACS/ACIR models is given in section 4.2.

Observation 4: Flat ACIR model with minimum RF requirements for UE ACLR/ACS can lead to over-estimation of the inter-UE CLI.

In Figure 3, the CDF of the throughput (considering intra-cell and inter-cell interference) for the urban hotspot scenario assuming the abovementioned parameters for the case of having a legacy TDD DL in the adjacent channel (red curve) and SBFD in the adjacent channel (black curve). It can be observed that the impact of inter-UE CLI is observed leading to a lower achieved throughput by the victim network.

Figure 3 Throughput CDF investigating legacy and SBFD deployments.

Observation 5: Based on assumptions given in Annex E TR 38.858, along with flat baseline ACLR/ACS with minimum RF requirements, no penetration loss between UEs in the same cluster and 0 % grid shift, we have observed throughput loss due to inter-UE CLI when SBFD is operating in the adjacent channel.

To further investigate the impact of the underlying assumptions on the experienced adjacent channel interference, we investigate the impact of UE clustering and indoor % on the victim TDD network throughput. On the left figure, the case if victim and aggressor are deployed in different clusters. As observed, if we deploy victim and aggressor UEs in different clusters, the inter-UE interference is negligible. The figure on the right considers the case if they are deployed in the same cluster but with different indoor ratios (i.e., percentage of UEs falling within the cluster).We observe the throughput for different indoor percentages assumptions and as expected, the as the indoor percentage is lower, the probability that UEs would fall in the same cluster is lower. This leads to lower inter-UE CLI, and thus higher throughput. The objective of this figure to showcase the sensitivity of the final perceived throughput to the underlying assumptions.

Figure 4 Impact of clustering and indoor % on system performance

Observation 6: The throughput degradation due to inter-UE CLI is sensitive to the UE deployment and clustering assumption as well as the % of UEs that are dropped indoor/ within a cluster.

# Solutions to address the inter-UE CLI issue

In [2], RAN4 had lengthy discussions on possible solutions to address the inter-UE CLI issue observed in urban hotspot deployments for FR1. TO summarize, RAN4 agreed on the following [2]:

- Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any.

## Solution 1: Reduced p-Max for SBFD-aware UE Tx power

This solution aims to address the inter-UE CLI issue by reducing the SBFD-aware UE p-Max via a newly introduced maximum power reduction parameter, X, that is given in dB. It is important to note that currently, there exists no methodology to identify the interfering SBFD-aware UEs to the victim TDD UEs. Thus, applying the p-Max power reduction for all SBFD-aware UEs or only a set of identified UEs is FFS. Main challenges with this solution are two-folds, a) identification of the interfering SBFD-aware UEs and b) Impact on SBFD UL network performance (i.e., UL coverage).

Observation 7: There exists no methodology to identify the set of interfering SBFD-aware UEs to the victim TDD UEs.

Nevertheless, let’s assume that an operator is able to identify such UEs. Afterwards, the p-Max of the SBFD-aware UE is decreased by a given offset, given in X dB, to avoid interfering on the victim TDD UEs in the adjacent channel. In this contribution, we assume that the identification in the clustered/ dense urban deployment is based on the dropped cluster. That means that any SBFD-aware UE that is dropped in a cluster will reduce its p-Max to adjust its maximum transmit power to avoid interfering on the victim TDD UEs coexisting in the same cluster.  Note that such identification is only for simulation purposes and in reality, such identification of clustered UEs might not be straightforward.

In figure 5, we present a holistic view of the system-wide gains and costs of max p-Max reduction solution for the SBFD network.  Green bars represent the gains % in terms of TP-loss reduction when introducing different values of X dB to the system. Note that in our contribution, all SBFD-aware UEs that are within a given distance (i.e., 25 m) to a victim legacy TDD UE will be configured with this X dB. As mentioned earlier, the methodology to identify those interfering UEs is not yet defined. As can be observed, with higher values of X dB, higher percentages  of TP-loss reduction can be achieved. On the other hand, the black bars represent the maximum TP loss percentage of the SBFD UL network. The maximum TP is evaluated based on the SNR of the scheduled SBFD-aware UEs. Intuitively, as the maximum configured p-Max value of the UE is lower, the UL SNR will be lower and thus, the maximum achievable TP will be lower. It is important to note that values given in Figure 5 are based on the set of assumptions given in Section 3.

Figure 5 System level gains and losses of p-Max reduction solution

Observation 8: With p-Max reduction, TP-loss reduction can be achieved due to lower inter-UE CLI interference, however, such reduction comes with the cost of SBFD UL network performance degradation.

Given that the value of X dB should provide a balance between sufficient inter-UE CLI mitigation and acceptable degradation for the SBFD UL network performance, it is important to note that for each power class, a unique X dB would be applied. From SBFD-aware UE point of view, no restrictions have been placed on the power class for the SBFD deployment, however, Rel-18 SI RAN4 adjacent channel coexistence work focused only on the PC3 (i.e., 23 dBm), since it was following the Rel-16 CLI study item parameters. An overview of the cost/gain of solution 1 is given in Table 1. Finally, since this solution addresses the interfering SBFD-aware UEs, this solution has to be UE-configured and not applied to all SBFD-aware UEs.

Observation 9a: If this solution is agreed by RAN4, more analysis is needed to derive meaningful X dB values for the different power classes for FR1 deployments.

Observation 9b: If this solution is agreed by RAN4, it has to be UE configured, in order not to impact all SBFD-aware UEs.

Table 1 Analysis on p-Max for SBFD-aware UE solution

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution: Reduced p-Max for SBFD-aware UE Tx power from victim channel) | Inter-UE CLI reduction | Degradation in SBFD-UL network performance | Derive max power reduction values for different UE power classes and modifications to TS 38.101-1. | FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. | FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. |

## Solution 2: DUD SBFD configuration

In this solution, the main idea is to provide protection for the adjacent TDD network from inter-UE CLI by adopting the SBFD configuration DUD in the SBFD network. By doing so, the downlink subband will act as a guard band, thus, reducing adjacent channel interference stemming from the aggressor SBFD-aware UE sending in the UL subband.

DUD configuration or UD configuration can provide protection against inter-UE CLI. However, mandating DUD configuration in Rel-19 work item will be limiting and might negatively impact the deployment of SBFD. It is best left to the operators to coordinate which SBFD configuration fits their use cases, while accommodating impact on adjacent operations, whenever feasible.

Observation 10: Mandating DUD configuration might be limiting to SBFD deployments.

In terms of quantifying the gains of such solution in the RAN4 adjacent channel coexistence framework, main impact will be on the underlying aggressor UE ACLR and victim UE ACS assumptions. As shown in Figure 6, the victim UE will fall into the ACLR3 region of the aggressor SBFD-aware UE. For sake of simplicity, we assume ACLR3 = 50 dB (following Table 12.9 in TR 36.942 for LTE-A simulations). It is important to note that ACLR3 here is just used for simulations and that minimum RAN4 ACLR requirements still hold. For the ACS, during offline discussions after RAN4#115, companies have proposed either ACS2 ~ 33.6 dB or ACS3 ~ 46 dB (following Table 12.9 in TR 36.942) as an optional value. Accordingly, the ACIR based on the different options can be summarized as follows:

- Legacy ACIR with ACLR = 30 dB / ACS  = 33 dB  ACIR ~ 28 dB

- Enhanced ACIR (a) w/ ACLR3 = 49 dB / ACS2 = 33.6 dB  ACIR ~ 33.5 dB

- Enhanced ACIR (b) w/ ACLR3 = 49 dB / ACS = 46 dB  ACIR ~ 44 dB

Figure 6 ACLR leakage visualization for DUD configuration

In Figure 7 we plot the TP-loss reduction percentage between edge TP loss with legacy ACIR and edge TP loss with ACIR2 and ACIR3. It can be observed that the throughput degradation is minimized as the ACIR is increased, which is expected. It is also important to note that the assumptions of SBFD DU configurations adjacent to TDD legacy network provide a worst-case view of the coexistence scenario and that additional protection can be achieved via the DUD configuration. The exact gains to be realized from the DUD configuration depends on the underlying assumptions of ACLR/ACS adopted. An overview of the cost/gain of solution 2 is given in Table 2.

Observation 11: Underlying assumptions on the SBFD configuration affect the experienced inter-UE CLI. Adopting ACIR2 or ACIR3 in the simulation provided gains in terms of inter-UE CLI reduction.

Figure 7 System level gains comparing ACIR2 and ACIR3 to legacy ACIR

Table 2 Analysis on mandated SBFD configuration solution

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | Inter-UE CLI reduction | Limiting deployment options for SBFD feature | Mandate a given SBFD configuration in RAN4 specs | None | None |

## Solution 3: BS implementation solutions

The third solution entails BS-implementation techniques that could be considered to address the inter-UE CLI issue. If the two networks belong to the same operator, an intuitive solution would be avoiding scheduling victim legacy TDD UEs that might be in close geographical proximity of the aggressor SBFD-aware UE. As an example, Figure 6 shows the example where in two cells, victim legacy UEs would fall in the same building/ cluster as the aggressor SBFD-aware UE. If the pair got scheduled along with the SBFD-aware UEs, inter-UE CLI would be present. Avoiding scheduling those UEs in the same slot or even allocating them to further away resource blocks could be a viable solution to address this Issue. Such action would require that the victim UE conducts the CLI measurement and reports it to its serving BS, which

Figure 8 Depiction of SBFD-aware UE identification to address inter-UE CLI

One example of such a scheduling-oriented approach was investigated by RAN1 in Section 2 in R1-2401442, where it was shown that an CLI-aware scheduling scheme shows up to 90% of DL performance degradation can be recovered as shown in Figure 8 below. For more info on the investigated inter-UE CLI handling schemes, the reader is kindly referred to Section 8.4 in TR 38.858. An overview of the cost/gain of solution 3 is given in Table 3.

Figure 9 Recovery of DL Throughput loss with simple CLI-aware scheduler [R1 2401442]

Observation 12: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI.

Table 3 Feasibility analysis on BS-implementation solution

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution: Scheduling-oriented solution (e.g., avoid victim UE close to SBFD-aware UE scheduling) | Inter-UE CLI reduction | Additional information exchange between BSs to avoid inter-UE CLI. | None | FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. | FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. |

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Observation 1: One of the interference sources of the Urban Hotspot scenario is the inter-UE CLI from SBFD-aware UEs transmitting in the UL subband towards the adjacent legacy TDD UEs receiving DL transmissions.

Observation 2: To investigate the inter-UE CLI case in an extreme scenario, we consider each victim shares the cluster, in which UEs are dropped, with an aggressor cell.

Observation 3: Based on Annex E assumptions, UEs falling in the same cluster will not experience any penetration losses, which ignores the fact that there might be additional losses due to wall penetration or UEs scheduled being in different rooms/ floors.

Observation 4: Flat ACIR model with minimum RF requirements for UE ACLR/ACS can lead to over-estimation of the inter-UE CLI.

Observation 5: Based on assumptions given in Annex E TR 38.858, along with flat baseline ACLR/ACS with minimum RF requirements, no penetration loss between UEs in the same cluster and 0 % grid shift, we have observed throughput loss due to inter-UE CLI when SBFD is operating in the adjacent channel.

Observation 6: The throughput degradation due to inter-UE CLI is sensitive to the UE deployment and clustering assumption as well as the % of UEs that are dropped indoor/ within a cluster.

Observation 7: There exists no methodology to identify the set of interfering SBFD-aware UEs to the victim TDD UEs.

Observation 8: With p-Max reduction, TP-loss reduction can be achieved due to lower inter-UE CLI interference, however, such reduction comes with the cost of SBFD UL network performance degradation.

Observation 9a: If this solution is agreed by RAN4, more analysis is needed to derive meaningful X dB values for the different power classes for FR1 deployments.

Observation 9b: If this solution is agreed by RAN4, it has to be UE configured, in order not to impact all SBFD-aware UEs.

Observation 10: Mandating DUD configuration might be limiting to SBFD deployments.

Observation 11: Underlying assumptions on the SBFD configuration affect the experienced inter-UE CLI. Adopting ACIR2 or ACIR3 in the simulation provided gains in terms of inter-UE CLI reduction.

Observation 12: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI.

Observation 13: Summary of the cost/ gain analysis of the different solutions is as follows:

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution: Reduced p-Max for SBFD-aware UE Tx power from victim channel) | Inter-UE CLI reduction | Degradation in SBFD-UL network performance | Derive max power reduction values for different UE power classes and modifications to TS 38.101-1. | FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. | FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. |
| Solution: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | Inter-UE CLI reduction | Limiting deployment options for SBFD feature | Mandate a given SBFD configuration in RAN4 specs | None | None |
| Solution: Scheduling-oriented solution (e.g., avoid victim UE close to SBFD-aware UE scheduling) | Inter-UE CLI reduction | Additional information exchange between BSs to avoid inter-UE CLI. | None | FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. | FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. |

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

[2]       R4-2504751, “Way Forward on UE-UE CLI problem statement and potential solutions ”, RAN4#115, Samsung.