# 3GPP TSG-RAN WG4 Meeting #116	R4-2510421

# Bengaluru, India, 25th – 29th August, 2025

Title: 	On Optional Channel Bandwidths Over Different Releases

Agenda item:	5.3.7

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

The discussion on whether some CBWs for some bands should become mandatory was triggered as for MSD analysis it was unclear whether 3MHz DL should have been considered or not, since 3MHz is optional. In the WF [1] it was decided not to include MSD based on 3MHz, however the discussion on whether set some other CBW to mandatory is still on going as shown below.

Agreement 1: - 3MHz CBW: - For MSD analyses, do not consider 3MHz CBW except when the DL affected band is band n106. - Companies are invited to share their views at next meeting whether 3MHz CBW should become mandatory for bands n26, n28, n31, n72, n85 and n100. - Note: For band n28, one operator can only deploy 3MHz in this band. - 5MHz CBW for band n41/n90: - For MSD analyses, consider 10MHz as the smallest DL CBW. - Companies are invited to further study whether 5MHz CBW for bands n41/n90 could be removed from Rel-19. - 35MHz CBW for band n71 - Companies are invited to further study whether 35MHz CBW for band n71 should become mandatory support.

In this tdoc, we are going to discuss every case.

# Discussion

## 3 MHz

It was requested to the 3MHz CBW as mandatory for bands n26, n28, n31, n72, n85 and n100. However, as 3MHz uses a special design of sync raster, a chip design modification would be essential for its operation. By making 3MHz mandatory on these bands, UE manufacturers will force to modify their already established designs or to un-support these bands. Therefore, it would be best to keep 3MHz as optional on these. Operators are invited to discuss the support of 3MHz in a case-by-case fashion with UE manufacturers.

Proposal 1: Keep 3MHz optional, to avoid UEs un-supporting n26, n28, n31, n72, n85 and n100.

## 5 MHz

It was requested to remove 5 MHz CBW for bands n41 and n90. As these two configurations were never deployed by any operator, their removal could be considered.

Proposal 2: Remove 5MHz CBW from n41 and n90.

## 35 MHz

It was requested to the 35MHz CBW as mandatory for bands n71. During Rel-16 , WI NR_FR1_35MHz_45MHz_BW, the large REFSENS degradation in the case of n71 with 35 MHz DL and 20MHz UL (asymmetric allocation) was studies in details[2]. Such a high UE REFSENS degradations would reduce network coverage which would be counterproductive for operators.

Proposal 3: Due to high REFSENS degradation for 35MHz DL on n71, we propose to keep the CBW as optional, unless operators strongly require it to be mandatory.

# Conclusion

Proposal 1: Keep 3MHz optional, to avoid UEs un-supporting n26, n28, n31, n72, n85 and n100.

Proposal 2: Remove 5MHz CBW from n41 and n90.

Proposal 3: Due to high REFSENS degradation for 35MHz DL on n71, we propose to keep the CBW as optional, unless operators strongly require it to be mandatory.

# Reference

[1]	« R4-2508050,WF on optional CHBW across releases, Skyworks ».

[2]	« R4-2016864  , WF on 35MHz 45MHz REFSENS AMPR, Qualcomm, Skyworks ».