3GPP TSG-RAN WG4 Meeting #116	R4-2510426

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on UE Demodulation performance requirements for LPWUS

Agenda item:	7.24.7

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

Issue 3-1-1: Whether to introduce demodulation requirements for LP-WUS - Agreement - Introduce demodulation requirements for LP-WUS Issue 3-1-2: Whether to introduce MDR test - Agreement - Introduce MDR test for LP-WUS Issue 3-1-4: Test metric for MDR test - Agreement - 1% MDR Issue 3-1-20: Whether to introduce dedicated PDSCH, PBCH and CSI reporting requirements for the main receiver - Agreement - Do not introduce dedicated PDSCH, PBCH and CSI reporting requirements for the main receiver

In the following we will discuss the remaining issues and provide observations and proposals if needed.

Discussion

Simulation analysis of testability aspects

In this section we will give an overview of the performance of a LP-WUR based on false alarm rate (FAR) based on noise and also false detection based on LP-WUS signal where wrong profile has been selected and thereby wrong wakeup occur. The FAR level influence of the MDR, with various 3GPP channel profiles, length of the sequence are all considered.

The FAR based on noise detection can be seen in Figure 1.

Figure 1 Simulation for the FAR based on noise. The simulation is done for OOK4 with M=1,2 and 4 and the sequences contains 3, 4 and 5 info bits. The sweep parameter is the length of the sequence (L)

From Figure 1 it can be seen that the FAR is below the 1% for all sequences with a length of 7 and below 0.1% for all sequences with a length of 12. The simulation is performed with AWGN, TDLC300-100 and TDLA30-10 and the results are identical for all profiles (only AWGN is illustrated in this section). If sequences with a length of 6 or below there will be a constant FAR larger than 1%.

False alarm rate detection can also be based on decoding of other codeword there is not intended t the device but other devices. This type of FAR depends highly on the simulation of the FAR based on wrong detection/estimation of the LP-WUS sequence is illustrated in Figure 2, Figure 3 and Figure 4.

The FAR simulation with AWGN channel is illustrated in Figure 2. As it can be seen occur the 1% FAR level below ­5 dB SNR for all length of the sequences with 5 info bits and OOK4 M=4.For less info bits the FAR level will be lower. For AWGN channel will a choice of sequence with the length of 12 or higher have a quite good performance (1% occur only below -7 dB)

For short sequences L<7 will the FAR based on noise be larger than >1%  for all SNR and for OOK-1 and OOK-4.

Figure 2 Simulation of the FAR based on wrong group detection based on other group signal decoding. Channel used for the simulation is AWGN channel.

The FAR simulation with TDLA30-10 channel is illustrated in Figure 3. As it can be seen occur the 1% FAR level below +5 dB SNR for all length of the sequences with 5 info bits and OOK4 M=4.For less info bits the FAR level will be lower but not lower than 0 dB SNR. Simulating and testing with the TDL-A30-10 is quite strict to the OOK signal decoding performance and it should be considered it TDLA30-10 should be used for test of the de-modulator performance as the UE would then change to “normal” idle conditions and use the MR instead of the LP-WUR for idle.

Figure 3 Simulation of the FAR based on wrong group detection based on other group signal decoding. Channel used for the simulation is TDL-A30-10 channel.

The FAR simulation with TDLC300-100 channel is illustrated in Figure 4. As it can be seen occur the 1% FAR level below +1 dB SNR for all length of the sequences with 5 info bits and OOK4 M=4.For less info bits the FAR level will be lower down to -4 dB SNR. For the TDLC300-100 profile a choice of a sequence with a length of 24 and M=4 shows quite good performance.

Figure 4 Simulation of the FAR based on wrong group detection based on other group signal decoding. Channel used for the simulation is TDL-C300-100 channel.

In Figure 5 is the MDR performance for sequences with 5 info bits and length from 6 to 32 illustrated. As it can be seen is the 1% MDR obtained between a SNR of 2-5 dB depending on the length of the sequence.

Figure 5 Illustration of MDR performance for TDL-C300-100 channel profile

Base on the simulation it can be seen that it is possible to make a test configuration there can verify the MDR and FAR performance. As an example, the setup could be following:

If the LP-WUS sequence is generated with OOK4- M=4 and sequence length of 24 the LP-WUS would fit into a NR slot as illustrated in Figure 6. In this case a target SNR test level with TDLC300-100 channel profile would be in the range from 1-5 dB SNR. This range would also be aligned well with the FAR base on both noise and signal detection there would be in the range of 0.1% FAR.

Figure 6 Illustration of a possible LP-WUS setup. In this configuration can LP-WUS with OOK4 M=2 L=12 and OOK4 M=4 L=24

Large number of sequences based on both OOK-1 and OOK-4 can fit into one NR slot.

For test of the demodulation select a sequence there can fit int one NR slot for more practical implementation.

Testability Aspects

Whether to define a test mode for LP-WUS/WUR

In RAN4#115 the testability of LP-WUS/WUR was discussed, and it was agreed to introduce MDR test, however it is still open if a test mode would be recommended for the MDR test (see [1]):

Issue 2-1-1: Whether to define a test mode for LP-WUS/WUR - Agreement - Take RF approach as a starting point to test MDR for demodulation requirements. RF agreement Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) - FFS whether test mode is recommended for MDR testing.

The test setup for LP-WUS/WUR can be as illustrated in Figure 7.

Figure 7 Illustration of a typical test setup for a device included LP-WUR. The downlink signal providing info to both the LP-WUR and the main radio receiver. Uplink is only connected to the main radio transmitter. For TDD it can be a single I/O bi-directional interface supporting both up-link and down-link.

Test time in idle based on msg. 1/ 3 to test equipment

To verify the performance of the LP-WUR in idle based on 1% MDR the number of LP-WUS detections/ responses should be in the range of 1000. (factor of 10x to achieve good accuracy of the MDR). The steps for responding to a received LP-WUS dedicated to the device is as follows:

- Perform decoding of the WUS

- Wakeup the Main Radio (MR)

- Send MSG. 1(PRACH), receive msg. 2 for time sync, transmit msg3, establish connection

- Terminate connection

- Enter LP-WUS/WUR connection

- Goto sleep

The time for this procedure can be in the range of app. 10 sec and should be repeated app. 1000 times. This gives a test time of almost 3 hours.

The similar procedure can be followed when the device is under test for FAR. In this case a failing scenario is when the device is responding when not expected.

Assuming a FAR of max 1% as with MDR, this gives a total test time of app. 6 hour. As this test is containing both LR and MR activity any given error can be due to other actions than the actual test purpose – test and verification of the demodulation of the LP-WUS it would be beneficial to have a test mode and thereby minimizing the test time and reduce the activity and errors there could come from MR activity.

MDR tests if done based on RF agreement can be implemented without introducing a specific test mode, however with the current RF agreement, such a test will likely take a long time. It is however RAN4 objective to define the tests but not necessarily decide how the test is implemented.

Introduce a test-mode for LP-WUS/WUR testing.

Whether to consider shortening the FAR test duration (if a test is introduced)

In RAN4#115 it was discussed if FAR tests shall be introduced and if a test mode would be recommended in case FAR Tests are introduced [1]:

Issue 2-1-2: Whether to consider shortening the test duration (if a test is introduced) - Agreement: - FFS whether to introduce FAR test and further study feasibility of introducing FAR test - FFS whether test mode is recommended for FAR testing.

The question of introducing FAR tests at all, is handled in section 2.3.1 of this contribution.

If FAR tests are introduced, the overall idea with the test will be to ensure, that the UE does not wake-up in situations where no LP-WUS is sent by the gNB (this would in some situations be the majority of time in normal operation) or the group ID in the LP-WUS sent by the gNB does not belong to the UE itself (FAR based on signal). Such a wake up can only happen in the case where the UE detect a LP-WUS and at the same time decode the LP-WUS to refer to the group ID of the UE.

At least two options can be considered when discussing how to implement a FAR test:

- Introduce AWGN from the TE and monitor how often the UE wakes up by UE responding to PDCCH.

- Introduce AWGN from the TE and monitor many times the UE has woken up by UE reporting the number of LPWUS it has detected.

- Introduce a setup where the TE send valid LP-WUS with different group identifiers randomly and monitor how often the UE wakes up by UE responding to PDCCH.

- Introduce a setup where the TE send valid LP-WUS with different group identifiers randomly and monitor how many times the UE has woken up by UE reporting the number of LPWUS it has detected.

As already highlighted in section 2.2.1 using the PDCCH to detect UE waking up is very time consuming, so also here we see it would most sense to introduce a new test mode.

Similar to our observation for MDR, we expect for FAR the test will also take substantial time to run unless a test mode is introduced.

If FAR test is introduced, introduce new test mode to reduce the FAR test run time.

Whether to define separate test cases for idle/inactive and connected modes

In RAN4#115 it was discussed if separate test cases should be defined for idle/inactive and connected modes (see [1]):

Issue 2-1-3: Whether to define separate test cases for idle/inactive and connected modes - WF - Option 1: Test cases should be designed separately LP-WUS operation in IDLE/INACTIVE mode and CONNECTED mode since the procedures are different. If both IDLE/INACTIVE mode and CONNECTED mode are supported by the LP-WUR, which mode is selected for test should be up to UE declaration.

The way of operation is quite different whether the device is in idle or connected mode with respect to the usages and decoding of the LP-WUS. In connected mode the length of the sequence (L) and number of info bits is assumed to be smaller than in idle mode. In idle mode the number of info bits will be high in dense areas to minimize the wakeup of larger groups of devices.

We see significant differences in usage and decoding of LP-WUS between idle and connected mode, which makes it relevant to define separate test cases.

Define separate test cases/ procedure for the modulator in idle and connected mode.

Demodulation Requirements

Whether to introduce FAR test

In RAN4#115 it was discussed whether to introduce FAR test (see [1]):

Issue 3-1-3: Whether to introduce FAR test - WF - Option 1: Yes. - Option 2: No.

As described in section 2.1 can the FAR be quite significant especially during test with channel profile like TDL-C300-100 and TDL-A30-10. As illustrated in Figure 8 is the FAR >1% up to +1 dB SNR depending on the sequence and the sequence length. This means that it can affect the power consumption performance and the MDR indirectly.

Figure 8 Simulation of the FAR based on wrong group detection based on other group signal decoding. Channel used for the simulation is TDLC300-100 channel.

In addition, the following agreement was made in RAN4#111 RF session (see [4])

Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

Based on the agreement in RAN4 RF, it is expected by RAN4 RF that RAN4 demod will introduce demodulation requirements for FAR. In case RAN4 does not introduced requirements for FAR it can be expected that companies will bring the issue up again in RAN4 RF as the agreement in RAN4 RF will not be fulfilled.

Since all companies already agreed that false alarm (FAR) requirements are needed as demodulation requirements in RAN4 RF session, we assume there has been no change in companies view since then.

It has already been agreed in RAN4#114 to introduce LPWUS requirements for MDR, however MDR requirements only secures, that the UE wakes up when needed, it does not measure if the UE wakes up by a false detection which matches the UE assigned group. The FAR requirements are therefore needed to ensure, that the threshold of the LPWUS detection is not set too low which will ensure MDR tests are passed without any failed detection of valid LPWUS but due to the low threshold setting the UE will possible also wake up on false detection even in cases where no LPWUS is send at all.

Introduction of LPWUS is a very costly feature when looking at the spectrum lost to LPWUS. It is therefore in our view critical, that the UEs are optimized to not have any/low number of FAR as well as not have too high MDR.

Previous agreement in RAN4#111 RF session is that FAR is expected to be defined as demodulation requirements, hence no FAR requirements were introduced by RAN4 RF.

Having both MDR and FAR requirements will ensure the UE implementation has been optimized with focus on securing that both LPWUS dedicated for the DUT are all correctly detected but also that no false detection of LPWUS is made by the UE.

RAN4 shall introduce requirements for FAR

Test metric for FAR test (if introduced)

In RAN4#115 it was discussed the test metric for FAR test was brought up (see [1]):

Issue 3-1-5: Test metric for FAR test (if introduced) - WF - Option 1: 1% FAR - Option 2: 0.1% FAR

Since a UE having a high FAR effectively does not utilize the LPWUS feature optimally, it is important that the FAR is as low as possible. Based on this we see selecting a limit of 0.1% FAR to be reasonable. We have however earlier demonstrated that a high number of runs are needed even for FAR at 1% if PDCCH is used to evaluate every time the UE has been scheduled for checking for LPWUS. If 0.1% is selected, the number of validations would need to be multiplied by 10 compared to 1%.

Using a FAR of 0.1% will require a very high number of validations to ensure coverage. If no testmode is introduced, the test time will be extremely high.

If a test mode is introduced to reduce test time, set FAR to 0.1%. If no testmode is introduced, set FAR to 1% as a compromise to not increase test time compared to MDR test time.

LP-WUS waveform

Issue 3-1-6: LP-WUS waveform - WF Option 1: Consider both OOK-1 and OOK-4

Seen from network load perspective it will be preferred to have the LP-WUS sequence within one slot. This means that if OOK-1 is used the sequence length can be max L=7. This would be fine for sequences with 2,3 and optional 4 info bits. For OOK- 4 with M=2 or 4 the number of info bits can be up to 5 and sequence length of L=24 (max28).

Based on this, we see it relevant to consider both OOK-1 and OOK-4 for requirement definition.

OOK-1 and OOK-4 will mandate different maximum values of L, hence to cover variation of L both are relevant for requirement definition.

Consider both OOK-1 and OOK-4

Receiver type

In RAN4#115 the receiver types for demodulation requirements were brought up with below proposed option [1]:

Issue 3-1-7: Receiver type - WF - Option 1: Consider both OOK and OFDM receivers

In the RAN4 RF sessions it has been agreed to define two types of receivers [5]:

Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

As RAN4 RF has introduced two types of receivers (OOK and OFDM) and since there is not defined any requirements about one of the receivers being mandatory for the UE, both receivers have to be considered for requirement definition.

Two receiver types have been defined by RAN4 RF (Type 1: OOK and Type 2: OFDM). Both receiver types are relevant for requirement definition as it is not given which receiver type(s) a UE will support.

Consider both OOK and OFDM receivers for requirement definition

If individual requirements are defined for both receiver types, consider if applicability rules can be introduced to only test for one receiver type in case a UE supports both receiver types.

Frequency range

In RAN4#115 the frequency range (FR1/FR2) for demodulation requirements were brought up with below proposed options [1]:

Issue 3-1-8: Frequency range - WF - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2.

To our understanding there has not been high focus on FR2 from RAN4 RF discussions, and it is not yet finalized from RAN4 RF side if FR2 are to be considered.

Prioritize FR1 to ensure requirements are defined for FR1. Keep FR2 FFS to be considered if time permits.

Duplex mode

In RAN4#115 the duplex mode for demodulation requirements were brought up with below proposed options [1]:

Issue 3-1-9: Duplex mode - WF - Option 1: Consider both TDD and FDD.

For the TDD case configuration it would be preferred to have the LP-WUS sequence within a slot to avoid larger latency for device n connected mode in parallel with UE’s in idle.

An example of the LP-WUS slot configuration could look like:

Figure 9 Illustration of a possible LP-WUS setup. In this configuration LP-WUS with OOK4 M=2 L=12 and OOK4 M=4 L=24 will be supported

This would give be best performance in case of OOK-4 with M=2 or 4 and can then have sequences of L=12 and 24.

TDD requirements can be implemented with one slot and will provide the best performance for OOK-4 with M=2 or 4 and can then have sequences of L=12 and 24. Same configuration can be used for FDD.

Consider both TDD and FDD for initial evaluation. FFS if downselection can be done to only define requirements with FDD.

SCS

In RAN4#115 the sub carrier spacing for demodulation requirements were brought up with below proposed options [1]:

Issue 3-1-10: SCS - WF - Option 1: 15kHz for FDD FR1 and 30kHz for TDD FR1. - Option 2: 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2

We do not see any reason diverting from the “normal” RAN4 configurations for LP-WUS, hence we are fine with option 2 as a starting point, which also includes FR2 if FR2 is to be considered.

As a starting point, consider 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2 (if introduced).

Bandwidth

In RAN4#115 the bandwidth for demodulation requirements were brought up with below proposed options [1]:

Issue 3-1-11: Bandwidth - WF - Option 1: For the channel bandwidth and SCS, take below assumptions for the starting points - 10MHz CHBW with SCS 15KHz for FR1 FDD - 40MHz CHBW with SCS 30KHz for FR1 TDD - 100MHz CHBW with SCS 120KHz for FR2

We do not see any reason diverting from the “normal” RAN4 configurations for LPWUS, hence we are fine with option 1 which also includes FR2 if FR2 is to be considered.

For the channel bandwidth and SCS, take below assumptions as a starting point
- 10MHz CHBW with SCS 15KHz for FR1 FDD
- 40MHz CHBW with SCS 30KHz for FR1 TDD
- 100MHz CHBW with SCS 120KHz for FR2 (if introduced)

Number of RBs

In RAN4#115 the number of RBs for demodulation requirements were brought up with below proposed option [1]:

Issue 3-1-12: Number of RBs - WF - Option 1: 11RB

In RAN1#117 and RAN1#118bis it was agreed to use 11PRBs for LP-WUS, hence also using 11PRB in RAN4 demod performance as a starting point is reasonable.

From RAN1 perspective it should support 11 PRB for both LP-WUS and LP-SS for FR1 SCS 15KHz and 30KHz and FR2 SCS 60 KHz and 120KHz.

11RB for all channel bandwidth and SCS combinations.

Antenna configuration

Issue 3-1-13: Antenna configuration - WF - Option 1: 1T1R.

As 1R is agreed between RAN1 and RAN4, using antenna configuration of 1T1R complies to the agreement. In addition, we expect that the configuration of 1T1R is the most likely configuration to be seen in actual deployment.

Use antenna configuration of 1T1R for requirement definition.

Channel model

In RAN4#115 the channel mode to be used for requirement definition was brough up [1]

Issue 3-1-14: Channel model - WF - Option 1: TDLA30-10 only - Option 2: TDLC300-100 only - Option 3: Both TDLC300-100 and TDLA30-10

As described in section 2.1 - we have shown that both TDLC300-100 and TDLA30-10 are feasible channel models for LPWUS requirement definition.

We have shown in our simulations that both TDLC300-100 and TDLA30-10 are feasible for requirement definition.

For now, consider both TDLC300-100 and TDLA30-10. Further discuss if down selection can be done when more simulation results are made available.

Time offsets

In RAN4#115 the used time offsets was brought up [1]:

Issue 3-1-15: Time offsets - WF - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4

As a starting point option 1 can, be used.

Use the following timeoffsets as starting point: 
- T= 5us for OOK-1
- T= 2us for OOK-4 with M=2
- T= 1us for OOK-4 with M=4

Frequency offsets

In RAN4#115 the used frequency offsets was brought up [1]:

Issue 3-1-16: Frequency offsets - WF - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm - Option 1: up to 20 ppm

Option 1 can be used as starting point – it would make sense to investigate f the frequency offset can be extended to cover 20ppm.

Use the following frequency offsets as starting point:
- For OFDM based receiver [5] ppm
- For OOK based receiver [10] ppm

Whether to define performance requirements for LP-SS

In RAN4#115 it was discussed if performance requirements are to be defined for LP-SS [1]:

Issue 3-1-17: Whether to define performance requirements for LP-SS - WF - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

We would like to not fully discard further study if requirements are to be introduced for LP-SS at this point in time. As LP-SS is only relevant for OOK receivers, it should first be decided if requirements are to be introduced for OOK receivers before further discussing requirements for LP-SS.

Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

Whether to define single requirement for Type 1 and Type 2 receivers

Issue 3-1-18: Whether to define single requirement for Type 1 and Type 2 receivers - WF - Option 1: Further discuss based on simulation results if one or two sets of requirements are to be defined

This topic is related to the two types of receivers defined for LP-WUR which is also discussed in section 2.3.4 of this document:

In the RAN4 RF sessions it has been agreed to define two types of receivers [5]:

Issue 2-1-5: How to map two LP-WUR types with UE capability Agreement: - RAN4 define two LP-WUR types (Type 1 and Type 2), no need 1-to-1 mapping between waveform and LP-WUR type. - Requirement applicability should be clearly defined. - RAN4 need to further discuss how to design the RMC for each type, how to ensure the performance of each type properly.

Since there are two types of receivers RAN4 shall study both receiver types and based on the outcome decide if performance requirements are to be defined for both receiver types, i.e., 2 sets of requirements, or if one set of requirements will be sufficiently independent on which receiver type is implemented in the DUT.

It has been agreed to define two LP-WUR types (Type 1 and Type 2) to match receivers capable of receiving the LP-WUS either with On-Off Keying (OOK) or OFDM.

RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined.

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

Coding/decoding algorithms for LP-WUS

Issue 3-1-19: Coding/decoding algorithms for LP-WUS - WF - Option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM coding

For code block (coded bits) length L after rate matching as defined in 38.212 section 7.4.2.1, support​

- For 1 information bit, L is in 1~32​

- For 2 information bits, L is in 2~32​

- For 3 information bits, L is in 3~32​

- For 4 information bits, L is in 5~32​

- For 5 information bits, L is in 6~32​

- For M=4, the code block (coded bits) length L is even number. ​

- Further restriction on the L value within above range is not precluded.​

As illustrated in Figure 1 should the length be of minimum L>=7 to fulfil the FAR <1% if this will be the agreed limit for FAR test.

The information bits and length L should match the required FAR and MDR – if FAR<1% L>=7.

Conclusion

The following Observations and Proposals were made:

Simulation analysis of testability aspects

- For short sequences L<7 will the FAR based on noise be larger than >1%  for all SNR and for OOK-1 and OOK-4.

Large number of sequences based on both OOK-1 and OOK-4 can fit into one NR slot.

- For test of the demodulation select a sequence there can fit int one NR slot for more practical implementation.

Testability Aspects

Whether to define a test mode for LP-WUS/WUR

MDR tests if done based on RF agreement can be implemented without introducing a specific test mode, however with the current RF agreement, such a test will likely take a long time. It is however RAN4 objective to define the tests but not necessarily decide how the test is implemented.

Introduce a test-mode for LP-WUS/WUR testing.

Whether to consider shortening the FAR test duration (if a test is introduced)

Similar to our observation for MDR, we expect for FAR the test will also take substantial time to run unless a test mode is introduced.

If FAR test is introduced, introduce new test mode to reduce the FAR test run time.

Whether to define separate test cases for idle/inactive and connected modes

We see significant differences in usage and decoding of LP-WUS between idle and connected mode, which makes it relevant to define separate test cases.

Define separate test cases/ procedure for the modulator in idle and connected mode.

Demodulation Requirements

Previous agreement in RAN4#111 RF session is that FAR is expected to be defined as demodulation requirements, hence no FAR requirements were introduced by RAN4 RF.

Having both MDR and FAR requirements will ensure the UE implementation has been optimized with focus on securing that both LPWUS dedicated for the DUT are all correctly detected but also that no false detection of LPWUS is made by the UE.

RAN4 shall introduce requirements for FAR

Test metric for FAR test (if introduced)

Using a FAR of 0.1% will require a very high number of validations to ensure coverage. If no testmode is introduced, the test time will be extremely high.

If a test mode is introduced to reduce test time, set FAR to 0.1%. If no testmode is introduced, set FAR to 1% as a compromise to not increase test time compared to MDR test time.

LP-WUS waveform

OOK-1 and OOK-4 will mandate different maximum values of L, hence to cover variation of L both are relevant for requirement definition.

Consider both OOK-1 and OOK-4

Receiver type

Two receiver types have been defined by RAN4 RF (Type 1: OOK and Type 2: OFDM). Both receiver types are relevant for requirement definition as it is not given which receiver type(s) a UE will support.

Consider both OOK and OFDM receivers for requirement definition

If individual requirements are defined for both receiver types, consider if applicability rules can be introduced to only test for one receiver type in case a UE supports both receiver types.

Frequency range

Prioritize FR1 to ensure requirements are defined for FR1. Keep FR2 FFS to be considered if time permits.

Duplex mode

TDD requirements can be implemented with one slot and will provide the best performance for OOK-4 with M=2 or 4 and can then have sequences of L=12 and 24. Same configuration can be used for FDD.

Consider both TDD and FDD for initial evaluation. FFS if downselection can be done to only define requirements with FDD.

SCS

As a starting point, consider 15kHz for FDD FR1, 30kHz for TDD FR1 and 120KHz for FR2 (if introduced).

Bandwidth

For the channel bandwidth and SCS, take below assumptions as a starting point
- 10MHz CHBW with SCS 15KHz for FR1 FDD
- 40MHz CHBW with SCS 30KHz for FR1 TDD
- 100MHz CHBW with SCS 120KHz for FR2 (if introduced)

Number of RBs

11RB for all channel bandwidth and SCS combinations.

Antenna configuration

Use antenna configuration of 1T1R for requirement definition.

Channel model

We have shown in our simulations that both TDLC300-100 and TDLA30-10 are feasible for requirement definition.

For now, consider both TDLC300-100 and TDLA30-10. Further discuss if down selection can be done when more simulation results are made available.

Time offsets

Use the following timeoffsets as starting point: 
- T= 5us for OOK-1
- T= 2us for OOK-4 with M=2
- T= 1us for OOK-4 with M=4

Frequency offsets

Use the following frequency offsets as starting point:
- For OFDM based receiver [5] ppm
- For OOK based receiver [10] ppm

Whether to define performance requirements for LP-SS

Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

Whether to define single requirement for Type 1 and Type 2 receivers

It has been agreed to define two LP-WUR types (Type 1 and Type 2) to match receivers capable of receiving the LP-WUS either with On-Off Keying (OOK) or OFDM.

RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined.

If agreed to introduce two sets of requirements, then re-use the definition from RAN4 RF for the receiver types.

Coding/decoding algorithms for LP-WUS

The information bits and length L should match the required FAR and MDR – if FAR<1% L>=7.

- References

- R4-2508626 - WF for [115][332] NR_LPWUS_Demod

- RP-241824 - Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- R4-2410569 - WF for [111][136] NR_LPWUS_UERF

- R4-2505234 - WF for [114bis][131] NR_LPWUS_UERF