3GPP TSG-RAN WG4 Meeting #116	R4-2510435

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.1.4

Source:	Samsung

Title:	Simulation results on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In RAN4#114bis and RAN4#115 meetings, work plan, general scopes and some test assumptions were agreed [4][5]. In this contribution, we provide our initial simulation results on 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Simulation for PDSCH demodulation performance

  - Simulation assumptions

According agreements in RAN4#114bis and RAN4#115 meetings [4][5], the simulation assumptions for PDSCH perforamnce test as Table 2-1.

Table 2-1. Simulation Assumptions for PDSCH perforamnce test

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Test case type |  |  | Only single carrier test |
| TDD pattern |  |  | 7DS2U, S: 6D+4G+4U |
| Channel bandwidth / SCS |  |  | 40 MHz / 30 kHz |
| Antenna configuration |  |  | 4 Layers: 4T6R 6 Layers: 8T6R |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| DM-RS configuration |  |  | 4 Layer: Rel-15 Type 1 DM-RS with 1+1 6 Layers: FFS |
| Overhead |  |  | 0 |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| (N1, N2), (O1, O2) |  |  | 8Tx case: (N1,N2) = (4,1), (O1, O2) = (4,1) 4Tx case: (N1,N2)=(2,1), (O1, O2) = (4,1) |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 6 layers cases |
| Number of HARQ Processes |  |  | 8 |
| HARQ ACK/NACK bundling |  |  | Not configured |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Receiver type assumption |  |  | MMSE-IRC with joint 6Rx MIMO detector |
| Tx EVM assumption |  |  | - 6% for QPSK/16QAM/64QAM - 3% for 256QAM |
| Test metric |  |  | SNR@70% max TP |

  - Simulation results

MCS for 4 MIMO layer case

For 4 MIMO layer case with TDLA30-10 ULA Low, there are two MCS options, one is MCS17 (Table 1), the other is MCS20 (Table 2). The PDSCH demodulation performance for these two MCS values as Figure 2-1, and the ideal SNR working points are summarized in Table 2-2.

Figure 2-1. Performance comparison with different MCS values for 4T6R4Layer ULA Low

Table 2-2. Ideal SNR working points for 4T6R4Layer ULA Low

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 4x6, ULA Low | 70 | 11.58 |
| 2 | 40 / 30 | MCS20 (Table 2) | TDLA30-10 | 4x6, ULA Low | 70 | 22.84 |

Since the SNR working points in Table 2-2 are ideal values, considering impairment margin, the SNR working point for MCS20 (Table 2) will be about 26dB, which seems a little bit high for demodulation tests.

MIMO correlation for 4 MIMO layers for Handheld UE testing

For the MIMO correlations, there are three proposals. In which, option 1 and option 2 introduces some correlation, while option 3 assumes zero correlation. According the properties of handheld UE and FWA UE, it seems some correlation should be introduced for handheld UE. Therefore, we evaluate the performance with different correlation values as Figure 2-2, and the ideal SNR working points are summarized in Table 2-3.

- Proposal - Option 1: with α = 0 and β = 0.07179 - Option 2: with α = 0.3 and β = 0.07179 - Option 3: with α = 0 and β = 0

Figure 2-2. Performance comparison with different correlation values for 4T6R4Layer ULA Med

Table 2-3. Ideal SNR working points for 4T6R4Layer ULA Med

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0, beta=0.07179 | 70 | 18.0 |
| 2 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0.3, beta=0.07179 | 70 | NA |
| 3 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0.3, beta=0.005154 | 70 | 26.6 |

From Table 2-3, it is clearly only Test num. 1 with (alpha=0, beta=0.07179) is workable for PDSCH demodulation test.

PDSCH Demodulation Performance for 6 Layers

According agreements for 6 layers, 8T6R6Layer with TDLA30-10 ULA Low and MCS17 (Table 1) should be used for PDSCH demodulation performance test for 6 Layers. Figure 2-3 is the simulation results for 6 Layers and the ideal SNR working points are summarized in Table 2-4.

Figure 2-3. PDSCH Demodulation Performance for 8T6R6Layer ULA Low

Table 2-4. Ideal SNR working points for 8T6R6Layer ULA Low

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 8x6, ULA Low | 70 | 18.0 |

- Simulation for SDR

  - Simulation assumptions

According agreements in RAN4#114bis and RAN4#115 meetings [4][5], the simulation assumptions for SDR requirements test as Table 3-1.

Table 3-1. Simulation Assumptions for SDR requirements test

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | wideband |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Length |  | Rank 2/4: 1 Rank 6/8: 2 |
|  | Antenna ports indexes |  | {1000, 1001} for 2 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers, 6 Layers and 8 Layers CCs |
| PTRS configuration |  |  | PTRS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  | Not configured |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| Propagation condition for 2T6R |  |  | Static propagation condition No external noise sources are applied H=111j-jj111-jj-j |
| Antenna configuration | 1 layer CCs |  | 1x1 or 1x2 or 1x4 |
|  | 2 layers CCs |  | 2x2 or 2x4 or 2x6 or 2x8 |
|  | 4 layers CCs |  | 4x4 or 4x6 or 4x8 |
|  | 6 layers CCs |  | 8x6 |
|  | 8 layers CCs |  | 8x8 |
| Physical signals, channels mapping and precoding |  |  | As specified in Annex B.4.1 |
| Es |  | dBm/Hz | -112 for MCS indexes in Table 5.5A-5: -110.5 For 1024QAM MCS indexes in Table 5.5A-6: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |  |  |  |

  - Simulation results

For 1024QAM with 2 layers, the simulation results could be summarized as Figure 3-1.

Figure 3-1. PDSCH Demodulation Performance for 1024QAM with 2 layers

Considering the assumption of TxEVM for 1024QAM is 2.5%, the maximum achievable SNR should be 32.0412dB. The ideal SNR working points for different MCS values as Figure 3-1, in which for MCS25, only when SNR is about 32dB (seems too high), maximum throughput could be achieved. Therefore, the MCS value for 1024QAM with 2 layers should be MCS24. Accordingly, the MCS values for 1024QAM with 2 layers should be as Table 3-2.

Table 3-2. MCS values for 1024QAM with 2 layers

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Conclusion

In this contribution, we provide our initial simulation results for PDSCH demodulation performance, SDR requirements on “6Rx for handheld and FWA” topic.

The ideal working point for PDSCH demodulation performance could be summarized as following.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS value | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 4x6, ULA Low | 70 | 11.58 |
| 1-2 | 40 / 30 | MCS20 (Table 2) | TDLA30-10 | 4x6, ULA Low | 70 | 22.84 |
| 2-1 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0, beta=0.07179 | 70 | 18.0 |
| 2-2 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0.3, beta=0.07179 | 70 | NA |
| 2-3 | 40 / 30 | MCS13 (Table 1) | TDLA30-10 | 4x6, ULA Med alpha=0.3, beta=0.005154 | 70 | 26.6 |
| 3-1 | 40 / 30 | MCS17 (Table 1) | TDLA30-10 | 8x6, ULA Low | 70 | 18.0 |

The SDR requirements for 1024QAM with 2 layers could be summarized as following.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[5] R4-2508727, Way Forward for Topic [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon