3GPP TSG-RAN WG4 Meeting #116	R4-2510438

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.12.3

Source:	Samsung

Title:	Simulation results on spatial channel model

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#109 in Sep 2025 [2]. The objectives are as below:

The study item includes the following objectives: - Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall be for FR1 (conducted).

In RAN4#113-RAN4#115 meetings, parameter assumptions, CDL methodologies and AAV configurations has been agreed for simulations [3][4][5][6][7].

In this contribution, we share our simulation results for CDL-based model, TDL extended model and baseline TDL channel model.

- Simulation assumptions

  - Agreed simualtion assumptions

For CDL based channel modeling, we combine the simulation assumptions argeed from RAN4#113 to RAN4#115 meetings [3][4][5][6][7] as below.

Table 2-1: Common simulation assumptions

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4 for PDSCH demodulation performance: 4T4R Rank8 for PDSCH demodulation performance: 8T8R Rank2/Rank4 for PMI performance: 8T4R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | - Rank 4 for PDSCH demodulation performance: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8 for PDSCH demodulation performance: SNR(dB)@30% and 70% of max throughput for each codeword - Rank2/Rank4 for PMI performance: SNR@90% and 70% of maximum throughput |

Table 2-2: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |

  - Agreed simualtion cases

For both purposes of alignment and comparsion, below cases are simulated in this contribution.

- CDL option 1 (A2): The model has 20 clusters which is derived based on Table 7.7.1-3 in TR 38.901 without cluster truncation.

- CDL option 2 (A4): The model has 12 clusters which is derived based on CDL option 1 with further cluster truncation.

- CDL with AAV3: Pass though AAV configuration: (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports, (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports

- CDL with AAV1Y: Sub-array Antenna Array Virtualization (AAV) configuration: (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports,  (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports

- Extended TDL: The model description is in TR38.753.

- Legacy TDL: The model description is in TS38.101-4 Annex B.2.

- Simulation results

  - PDSCH demodulation performance for 4Tx4Rx4layer

    - Legacy TDL and SCM comparison

Firstly, since SCM (different options, two CDL options and one unified extended TDL solution) are studied in this SI, it should be beneficial to fully understand the performance difference of different channel models. Hence, we simulated the PDSCH demodulation performance with legacy TDL, CDL-A2, CDL-A4 and Extended-TDL, the performance as Figure 3-1.

Figure 3-1. 4T4R4Layer PDSCH demodulation performance comparison with different solutions

From Figure 3-1, we have below observations for 4T4R4Layer PDSCH demodulation performance:

- Peak throughput is unable to achieve with legacy TDLC300-10 ULA Medium correlation configuration.

- For 38.753 based CDL with pass though AAV configuration, similar performance (SNR working points at 70% of maximum throughput) could be observed for CDL-A2 and CDL-A4, which is close to legacy TDL with XP Medium correlation case.

- For extended TDL case, similar performance (SNR working points at 70% of maximum throughput) as legacy TDL with XP Medium correlation case.

    - 38.753 CDL-based options comparison

According our agreements in last RAN4#115 meeting, pass though AAV configuration and the truncation solution A4 are down-selected for alignment purpose. At the same time, the simulation results for CDL with AAV1Y and A2 without truncation should be captured into the TR for comparison purpose. Hence, the PDSCH performance for CDL-A2-AAV1Y, CDL-A2-AAV3, CDL-A4-AAV1Y, and CDL-A4-AAV3 should be analysed. Simulation results as Figure 3-2.

Figure 3-2. 4T4R4Layer PDSCH demodulation performance for 38.753 CDL-based options comparison

From Figure 3-2, we have below observations:

- For 4T4R 4layer PDSCH demodulation performance, there are almost no performance difference observed for 38.753 CDL-based options (CDL-A2-AAV1Y, CDL-A2-AAV3, CDL-A4-AAV1Y, and CDL-A4-AAV3).

    - SNR working points summary

Based on agreed simulation assumptions, our simulation results for 4T4R4Layer PDSCH demodulation performance could be summarized as Table 3-1.

Table 3-1. SNR working points summary for 4T4R4Layer PDSCH demodulation performance

| Test cases | Metric (SNR [dB] @% Max TP) |  |
| --- | --- | --- |
|  | 30% | 70% |
| TDL, ULA-Low | 5.2 | 13.6 |
| TDL, ULA-Med | NA | NA |
| TDL, XP-Med | 6.5 | 15.5 |
| TDL, XP-High | 12.1 | 27.6 |
| CDL, A2-AAV3-3kmh | 6.5 | 15.4 |
| CDL, A2-AAV1Y-3kmh | 6.8 | 15.6 |
| CDL, A4-AAV3-3kmh | 6.7 | 15.6 |
| CDL, A4-AAV1Y-3kmh | 6.8 | 15.6 |
| Extended TDL | 6.0 | 15.4 |
| CDL, A2-AAV3-30kmh | 7.2 | 16.8 |
| CDL, A2-AAV1Y-30kmh | 6.7 | 16.1 |
| CDL, A4-AAV3-30kmh | 7.6 | 17.5 |
| CDL, A4-AAV1Y-30kmh | 7.0 | 16.1 |

  - PDSCH demodulation performance for 8Tx8Rx8layer

    - Legacy TDL performance

For 8T8R8Layer legacy TDL with ULA-Low correlation configuration, the same PDSCH demodulation performance could be observed for two codewords as Figure 3-3. While, for 8T8R8Layer legacy TDL with XP-Med correlation configuration, different PDSCH demodulation performance could be observed for two codewords as Figure 3-4, however, 70% maximum throughput could not be reached.

Figure 3-3. 8T8R8Layer PDSCH demodulation performance for TDL with ULA-Low

Figure 3-4. 8T8R8Layer PDSCH demodulation performance for TDL with XP-Med

    - 38.753 CDL-based options performance

According our agreements in last RAN4#115 meeting, pass though AAV configuration and the truncation solution A4 are down-selected for alignment purpose. At the same time, the simulation results for CDL with AAV1Y and A2 without truncation should be captured into the TR for comparison purpose. Hence, the PDSCH performance for CDL-A2-AAV1Y, CDL-A2-AAV3, CDL-A4-AAV1Y, and CDL-A4-AAV3 should be analysed. Simulation results as Figure 3-5 – Figure 3-6.

Figure 3-5. 8T8R8Layer PDSCH demodulation performance for A4 vs A2 (AAV3)

Figure 3-6. 8T8R8Layer PDSCH demodulation performance for AAV3 vs AAV1Y (A4)

From Figure 3-5 and Figure 3-6, we have below observations:

- For 8T8R 8layer PDSCH demodulation performance, different PDSCH demodulation performance could be observed for two codewords.

- For 8T8R 8layer PDSCH demodulation performance, there are obvious performance difference observed for 38.753 CDL A2 and A4, i.e., the SNR working point at 70% maximum throughput for A4-Total is 1.3 dB worse than that of A2-Total.

- For 8T8R 8layer PDSCH demodulation performance, there are almost no performance difference observed for 38.753 CDL AAV3 and AAV1Y, e.g., the performance of AAV3-Total and AAV1Y-Total on the SNR working point at 70% maximum throughput.

    - SNR working points summary

Based on agreed simulation assumptions, our simulation results for 8T8R8Layer PDSCH demodulation performance could be summarized as Table 3-2.

Table 3-2. SNR working points summary for 8T8R8Layer PDSCH demodulation performance

| Test cases |  | SNR [dB] |  |  |
| --- | --- | --- | --- | --- |
|  |  | Total | CW0 | CW1 |
| 30% @ maximum throughput | TDL, ULA-Low | 5.2 | 5.2 | 5.1 |
|  | TDL, XP-Med | 13.0 | 8.5 | 20.2 |
|  | CDL, A2-AAV3-3kmh | 9.5 | 6.5 | 11.5 |
|  | CDL, A2-AAV1Y-3kmh | 9.7 | 6.9 | 11.7 |
|  | CDL, A4-AAV3-3kmh | 10.2 | 6.3 | 12.4 |
|  | CDL, A4-AAV1Y-3kmh | 9.4 | 6.0 | 11.1 |
|  | Extended TDL | 8.9 | 5.7 | 11.0 |
|  | CDL, A2-AAV3-30kmh | 10.3 | 8.6 | 11.7 |
|  | CDL, A2-AAV1Y-30kmh | 9.4 | 7.1 | 11.0 |
|  | CDL, A4-AAV3-30kmh | 8.4 | 6.5 | 9.8 |
| 70% @ maximum throughput | TDL, ULA-Low | 14.4 | 14.4 | 14.3 |
|  | TDL, XP-Med | NA | 27.4 | NA |
|  | CDL, A2-AAV3-3kmh | 19.5 | 17.1 | 24.8 |
|  | CDL, A2-AAV1Y-3kmh | 19.4 | 17.3 | 25.9 |
|  | CDL, A4-AAV3-3kmh | 20.8 | 18.5 | 24.4 |
|  | CDL, A4-AAV1Y-3kmh | 21.0 | 18.2 | 25.2 |
|  | Extended TDL | 17.3 | 14.8 | 20.4 |
|  | CDL, A2-AAV3-30kmh | 23.9 | 21.5 | 26.6 |
|  | CDL, A2-AAV1Y-30kmh | 24.4 | 21.6 | 27.0 |
|  | CDL, A4-AAV3-30kmh | 24.0 | 21.0 | 26.4 |

  - PMI reporting performance for 8Tx4Rx2layer

    - Legacy TDL and SCM comparison

For 8T4R2Layer, firstly we’d like to understand the PMI reporting performance difference between legacy TDL and SCM channel configuration. Here we select one CDL-based SCM case for comparison as Figure 3-7.

Figure 3-7. 8T4R2Layer PMI reporting performance comparison for legacy TDL and CDL-based SCM

From Figure 3-7, the observations could be summarized as below

- For 8T4R2Layer PMI performance, obvious performance difference could be observed between legacy TDL and 38.753 CDL A2.

- For 8T4R2Layer PMI performance with legacy TDL channel model, the performance of following eType II is worse than following Type I.

- For 8T4R2Layer PMI performance with 38.753 CDL A2 channel model, the performance of following eType II is better than following Type I.

    - 38.753 options comparison

This section provides PMI reporting performance simulation results comparison for different solutions listed in 38.753, i.e., CDL A4 vs CDL A2, CDL AAV3 vs AAV1Y, and extended TDL.

38.753 CDL A4 (with truncation) and A2 (without truncation) comparison

The PMI reporting performance are compared between 38.753 A4 and A2 in Figure 3-8, for following Type I codebook, following eType II codebook and random Type I codebook.

Figure 3-8. 8T4R2Layer PMI reporting performance comparison for CDL A4 and A2

From Figure 3-8, the observations could be summarized as below

- For 8T4R2Layer PMI performance, almost no performance difference between CDL A4 and A2 could be observed for following Type I codebook case.

- For 8T4R2Layer PMI performance, almost no performance difference between CDL A4 and A2 could be observed for random Type I codebook case.

- For 8T4R2Layer PMI performance, obvious performance difference could be observed between CDL A4 and A2 for following eType II codebook case. The SNR working point @ 90% maximum throughput of CDL A4 is ~1.2dB loss than CDL A2, and The SNR working point @ 70% maximum throughput of CDL A4 is ~1.4dB loss than CDL A2.

38.753 CDL AAV3 and AAV1Y comparison

The PMI reporting performance are compared between 38.753 AAV3 and AAV1Y in Figure 3-9, for following Type I codebook, following eType II codebook and random Type I codebook.

Figure 3-9. 8T4R2Layer PMI reporting performance comparison for CDL AAV3 and AAV1Y

From Figure 3-9, the observations could be summarized as below

- For 8T4R2Layer PMI performance, almost no performance difference between CDL AAV3 and AAV1Y could be observed for following Type I codebook, following eType II codebook and random Type I codebook cases.

PMI reporting performance of 38.753 Extended TDL

The PMI reporting performance of extended TDL model with XP High correlation as Figure 3-10. From Figure 3-10, the observations could be summarized as below

- For 8T4R2Layer with extended TDL300-10 and XP High correlation, the performance of eType II codebook outperform Type I codebook.

Figure 3-10. PMI reporting performance of 38.753 extended TDL model

    - SNR working points summary

Based on agreed simulation assumptions, our simulation results for 8T4R2Layer PMI reporting performance could be summarized as Table 3-3.

Table 3-3. SNR working points summary for 8T4R2Layer PMI reporting performance

| Test cases |  | Metric (SNR [dB] @% Max TP) |  |
| --- | --- | --- | --- |
|  |  | 70% | 90% |
| TDL, XP-High | Type I | 1.1 | 2.6 |
|  | eType II | 4.5 | 6.1 |
|  | Random | 13.7 | NA |
| CDL, A2-AAV3-3kmh | Type I | 2.4 | 4.5 |
|  | eType II | 1.9 | 3.6 |
|  | Random | 6.9 | 11.2 |
| CDL, A2-AAV1Y-3kmh | Type I | 2.6 | 4.6 |
|  | eType II | 2.3 | 3.7 |
|  | Random | 7.2 | 11.5 |
| CDL, A4-AAV3-3kmh | Type I | 2.2 | 4.3 |
|  | eType II | 3.3 | 4.8 |
|  | Random | 7.3 | 11.0 |
| CDL, A4-AAV1Y-3kmh | Type I | 2.6 | 4.0 |
|  | eType II | 3.6 | 4.9 |
|  | Random | 7.4 | 11.5 |
| Extended TDL | Type I | 4.3 | 5.7 |
|  | eType II | 3.5 | 5.1 |
|  | Random | 6.7 | 9.8 |
| CDL, A2-AAV3-30kmh | Type I | 3.6 | 5.9 |
|  | eType II | 2.6 | 4.8 |
|  | Random | 7.7 | 12.1 |
| CDL, A2-AAV1Y-30kmh | Type I | 3.4 | 5.5 |
|  | eType II | 2.2 | 4.2 |
|  | Random | 7.3 | 11.9 |
| CDL, A4-AAV3-30kmh | Type I | 3.8 | 6.5 |
|  | eType II | 4.2 | 6.1 |
|  | Random | 7.2 | 11.0 |
| CDL, A4-AAV1Y-30kmh | Type I | 3.4 | 5.4 |
|  | eType II | 3.9 | 5.9 |
|  | Random | 7.3 | 11.3 |

  - PMI reporting performance for 8Tx4Rx4layer

Similar as 8Tx4Rx2layer section, in this section, the PMI reporting performance for 8Tx4Rx4layer with 38.753 SCM solutions are simulated and summarized.

    - 38.753 options comparison

This section provides PMI reporting performance simulation results comparison for different solutions listed in 38.753, i.e., CDL A4 vs CDL A2, CDL AAV3 vs AAV1Y, and extended TDL.

38.753 CDL A4 (with truncation) and A2 (without truncation) comparison

The PMI reporting performance are compared between 38.753 A4 and A2 in Figure 3-11, for following Type I codebook, following eType II codebook and random Type I codebook.

Figure 3-11. 8T4R4Layer PMI reporting performance comparison for CDL A4 and A2

From Figure 3-11, the observations could be summarized as below

- For 8T4R4Layer PMI performance, similar trends for CDL A4 and A2 could be observed, and the performance of CDL A4 is a bit worse than CDL A2.

- For 8T4R4Layer PMI performance, the performance of following eType II is worse than following Type I could be observed for both CDL A4 and A2 cases.

38.753 CDL AAV3 and AAV1Y comparison

The PMI reporting performance are compared between 38.753 AAV3 and AAV1Y in Figure 3-12, for following Type I codebook, following eType II codebook and random Type I codebook.

Figure 3-12. 8T4R4Layer PMI reporting performance comparison for CDL AAV3 and AAV1Y

From Figure 3-12, the observations could be summarized as below

- For 8T4R4Layer PMI performance, similar trends for CDL AAV3 and AAV1Y could be observed.

- For 8T4R4Layer PMI performance, the performance of following eType II is worse than following Type I could be observed for both CDL AAV3 and AAV1Y cases.

PMI reporting performance of 38.753 Extended TDL

The PMI reporting performance of extended TDL model with XP High correlation as Figure 3-13. From Figure 3-13, the observations could be summarized as below

- For 8T4R4Layer with extended TDL300-10 and XP High correlation, the performance of eType II codebook is worse than Type I codebook at some SNR range.

Figure 3-13. PMI reporting performance of 38.753 extended TDL model

    - SNR working points summary

Based on agreed simulation assumptions, our simulation results for 8T4R4Layer PMI reporting performance could be summarized as Table 3-4.

Table 3-4. SNR working points summary for 8T4R4Layer PMI reporting performance

| Test cases |  | Metric (SNR [dB] @% Max TP) |  |
| --- | --- | --- | --- |
|  |  | 70% | 90% |
| CDL, A2-AAV3-3kmh | Type I | 9.4 | 12.0 |
|  | eType II | 13.2 | 15.6 |
|  | Random | 16.3 | 19.2 |
| CDL, A2-AAV1Y-3kmh | Type I | 9.8 | 11.7 |
|  | eType II | 13.4 | 15.4 |
|  | Random | 16.4 | 19.4 |
| CDL, A4-AAV3-3kmh | Type I | 10.2 | 12.5 |
|  | eType II | 13.8 | 16.8 |
|  | Random | 16.9 | 19.6 |
| CDL, A4-AAV1Y-3kmh | Type I | 10.1 | 12.3 |
|  | eType II | 13.4 | 15.7 |
|  | Random | 16.9 | 19.8 |
| Extended TDL | Type I | 10.8 | 13.6 |
|  | eType II | 11.6 | 17.3 |
|  | Random | 16.1 | NA |
| CDL, A2-AAV3-30kmh | Type I | 10.4 | 13.6 |
|  | eType II | 14.0 | 16.3 |
|  | Random | 17.6 | 21.9 |
| CDL, A2-AAV1Y-30kmh | Type I | 9.9 | 12.7 |
|  | eType II | 14.7 | 17.9 |
|  | Random | 16.6 | 20.3 |
| CDL, A4-AAV3-30kmh | Type I | 12.1 | 15.6 |
|  | eType II | 15.0 | 17.2 |
|  | Random | 16.7 | 19.3 |
| CDL, A4-AAV1Y-30kmh | Type I | 12.9 | 14.8 |
|  | eType II | 15.1 | 16.8 |
|  | Random | 17.4 | 20.1 |

  - PMI reporting performance for 32Tx4Rx4layer

Since some performance losses are observed for eType II codebook compared with Type I codebook with 8Tx4Rx4layer configuration, we’d like to know if performance gain could be observed for eType II codebook compared with Type I codebook with 32Tx4Rx4layer. The simulation results with 38.753 CDL A4 are as Figure 3-14. From Figure 3-14., some performance gain could be achieved for eType II codebook compared with Type I codebook with 32Tx4Rx4layer.

Figure 3-14. PMI reporting performance of 38.753 CDL A4 model

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] RP-251040, Revised SID: Study on spatial channel model for demodulation performance requirements, RAN#108 meeting, Nokia, BT Plc

[3] R4-2419782, Way Forward for [113][320]NR_SCM, RAN4#113 meeting, Nokia

[4] R4-2502378, Way Forward for [114][322]NR_SCM, RAN4#114 meeting, Nokia

[5] R4-2504756, Way Forward for [114bis][320] NR_SCM, RAN4#114bis meeting, Nokia

[6] R4-2504762, Simulation assumptions for SCM, RAN4#114bis meeting, Huawei, HiSilicon, MediaTek

[7] R4-2508622, Simulation assumptions for SCM, RAN4#115 meeting, Huawei, HiSilicon, MediaTek