3GPP TSG-RAN WG4 Meeting #116	R4-2510439

Bangalore, India, 25th – 29rd August, 2025

Agenda item:	7.13.3

Source:	Samsung

Title:	Discussion on UE demodulation and CSI performance requirements for 8Rx with MMSE-IRC

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved. According to the plan, this WI is targeting to be finished in RAN#109 meeting on Sep 2025.

Generally, the enhancements for UE demodulation performance focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

RAN4#112 meeting is the first meeting for this topic, several high-level agreements are achieved [2]. Cover CQI reporting requirements under ICI for 8Rx is agreed and most parameters for simulation assumptions are agreed in RAN4#112bis [3] and RAN4#113 [4], more agreements about scenario coverage, test parameters and applicability rules are achieved in [5][6][7].

In this contribution, we provide our further analysis and views on the open issues for PDSCH demodulation performance part.

- Discussion

  - General

Definition of MMSE-IRC receiver for 8Rx UE

The discussion for the definition of MMSE-IRC receiver for 8Rx lasts from RAN4#113 meeting to RAN4#115 meeting. After thorough discussions and multiple rounds of deliberation, we have identified several options as below.

Definition of MMSE-IRC receiver capability for 8Rx UE Options: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 4: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 and 38.306 section 5.2 to the following: - Baseline 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 5: Re-use existing definition in Rel-18.

Analysis for option 1 and option 2:

Both option 1 and option 2 have the keyword ‘MMSE-IRC’, the difference is option 1 explicitly incorporates the keyword ‘MMSE-IRC’ in both the title and the body text, while option 2 restricts its usage to the content section only. Considering the following two reasons, we recommend including this keyword at least in the content section:

Firstly, the WID clearly mentioned the work scope is for MMSE-IRC as following, and related simulations are focusing on MMSE-IRC receiver. Hence, adding the keyword ‘MMSE-IRC’ in the definition could avoid scope extension.

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

Secondly, for MMSE-IRC receiver, there is no processing difference for UE implementation when handling SU-MIMO and MU-MIMO. But for some other receiver, e.g., the enhanced R-ML receiver, the UE processing is different for SU-MIMO and MU-MIMO. If we would like to use the same receiver definition for both SU-MIMO and MU-MIMO, it’s a proper solution to add the keyword ‘MMSE-IRC’.

Furthermore, we prefer to add the keyword ‘MMSE-IRC’ in the title of the definition, because we’d like to leave space for the definition of other enhanced receiver in the future. Otherwise, the naming of the receiver definition looks a little bit strange, e.g., it seems like MMSE-IRC receiver is the general definition, while another enhanced receiver is the sub definition of MMSE-IRC receiver.

However, some companies may have concern that the requirements defined for this receiver type (MMSE-IRC) could be avoided by other more advanced receiver type(s).

Analysis for option 3 and option 4:

Both option 3 and option 4 support the idea without MMSE-IRC wording, which assume the 8Rx reciever should focus on the defination for baseline or simplified. Option 3 removes ”SU-MIMO” from both title and body context, while option 4 still keep ”SU-MIMO” in the body context. Compared with option 4, it seems option 3 are targeting to define an unified 8Rx receiver for both MU-MIMO and SU-MIMO scenarios, which may save efforts for maintaining two groups of definations. However, without receiver type information (e.g., MMSE-IRC, R-ML, etc.), it is hard to determine if the same defination could be used for SU-MIMO and MU-MIMO.

In addition, for option 3, we don’t have to introduce a new 8Rx receiver defination for MU-MIMO, while for option 4, we have to introduce a new 8Rx receiver defination or applicabiliy rule for MU-MIMO.

Analysis for option 5:

Currently, the 8Rx receiver defination in 38.101-4 is as below.

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

And the 8Rx receiver defination in 38.306 as below.

SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.

Except the ”FR1”, the defination from 38.101-4 and 38.306 are the same with each other, and this defination is totally fulfilled with the WID scope when we define 8Rx demodulation performance requirements in Rel-18. Now we are facing the 8Rx receiver defination for MU-MIMO scenario. In order to keep the defination clear and easy to cross-reference, we realized that specifying 8Rx reciver(s) for SU-MIMO and MU-MIMO seperately is a suitable solution, which could help prevent problems like ambiguous definitions or overly wide-ranging scopes, and could also ensure the consistency in receiver type defination as below.

38.306 Section 5.2 UE receiver features

… Definitions for feature. MU-MIMO Interference Mitigation advanced receiver with modulation order detection R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and 4RX in FR1 when the co-scheduled UE information with DCI index 6 or 7 in Table 7.3.1.2.2-12 of TS 38.212 [10] is signalled. UE supporting the feature is required to indicate the support of advReceiver-MU-MIMO-r18 and meet the Enhanced Receiver Type 2 requirements in TS 38.101-4 [18].. MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX in FR1 when the co-scheduled UE information with DCI index 6 in Table 7.3.1.2.2-12 of TS 38.212 [10] is signalled. UE supporting the feature is required to indicate the support of advReceiver-MU-MIMO-r18 and meet the Enhanced Receiver Type 2 requirements in TS 38.101-4 [18].. SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type 1 requirements in TS 38.101-4 [18].. SU-MIMO 8Rx receiver - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector in FR1 - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors in FR1.. …

Observation 1: The WID clearly mentioned the work scope is for MMSE-IRC as “PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver”.

Observation 2: In current spec, SU-MIMO and MU-MIMO receivers for R-ML are defined separately.

Proposal 1: Keep current definition for SU-MIMO 8Rx receiver for SU-MIMO test cases, and introduce new MU-MIMO 8Rx receiver definition to prevent ambiguous definitions, overly wide-ranging scopes, and ensure the consistency in receiver type naming.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Observation 1: The WID clearly mentioned the work scope is for MMSE-IRC as “PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver”.

Observation 2: In current spec, SU-MIMO and MU-MIMO receivers for R-ML are defined separately.

Proposal 1: Keep current definition for SU-MIMO 8Rx receiver for SU-MIMO test cases, and introduce new MU-MIMO 8Rx receiver definition to prevent ambiguous definitions, overly wide-ranging scopes, and ensure the consistency in receiver type naming.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom

[3] R4-2416555, WF for [112bis][320] NR_demod_Ph5_Part2_UE, RAN4#112bis meeting, Nokia

[4] R4-2419784, Way forward for [113][322] NR_demod_Ph5_Part2_UE, RAN4#113 meeting, Nokia

[5] R4-2502275, Way forward for [114][324] NR_demod_Ph5_Part2_UE, RAN4#114 meeting, Nokia

[6] R4-2504694, Way forward for [114bis][322] NR_demod_Ph5_Part2_UE, RAN4#114bis meeting, Nokia

[7] R4-2508620, Way forward for [114bis][323] NR_demod_Ph5_Part2_UE, RAN4#115 meeting, Nokia