3GPP TSG-RAN WG4 Meeting # 116                                    R4-2510447

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.25.3.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Conditional Intra-CU LTM

Document for:	Approval

Introduction

In Rel-18, LTM was introduced to reduce handover delay and interruption time compared to L3 based mobility. As mentioned in WID [1], LTM as introduced in Rel-18 has a number of limitations compared to L3 mobility. In Rel-19, these limitations should be removed as much as possible.

Discussion

Compared to R18 LTM, conditional Intra-CU LTM should be supported in R19.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Specify RRM requirements related to the above objectives as necessary [RAN4]

LTM as introduced in Rel-18 offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures. In Rel-19, enhancements should be specified so that the system can benefit from both the high robustness and short interruption. It’s beneficial to combine LTM and conditional L3 mobility procedures and RAN4 needs to define requirements based on RAN2’s design.

From the objective, UE evaluated conditions will be specified by other groups to trigger LTM. RAN4 should define conditional cell switch delay requirement, both intra-frequency and inter-frequency need be considered. Meanwhile, the requirements are applicable to conditional cell switch to change the NR PCell to another NR cell. The relevant requirements can refer to the conditional handover of L3 in section 6.1.4.

In last meeting, RAN4 has reached agreement that CHO requirements framework can be used as baseline for Conditional LTM. As to the definiation of components, we have discussed one by one and there were still some issues need to be considered in this meeting.

Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

Tmeasure for CSI-RS based L1-RSRP measurement should be discussed based on the agreement for other part, therefore, we should study the CSI-RS based L1-RSRP measurement period first and finish the definiation of Tmeasure as soon as progress is made. In addition to measurement period, as mentioned by other companies, SFN scquisition and PSS/SSS detection may also need to be considered. For the first sub-bullet, we should further discuss the conditions for TCSI-RS_SFN_intra = 0. Similar as CSI-RS based L3 measurement, the time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled).

#### 9.10.2.5 Intra-frequency measurements without measurement gaps If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the CSI-RS based measurement shall include PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap. - PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in clause 9.2.5.1. - The time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, the time period used to acquire the SFN information is TCSI-RS_SFN_intra as shown in table 9.10.2.5-3 for FR1. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. - If the associatedSSB, which has been detectable at least for the time period Tidentify_intra_with_index defined in clause 9.2.5.1, becomes undetectable for a period ≤ 5 seconds and then the associatedSSB becomes detectable again with the same spatial reception parameter provided the timing to that cell has not changed more than  3200/2µ Tc, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3], PSS/SSS detection time and time period used to acquire the SFN information are equal to 0. The measurement period for CSI- RS based intra-frequency measurements without gaps is as shown in table 9.10.2.5-1and table 9.10.2.5-2.

Proposal 1: For CSI-RS based L1-RSRP measurement, the time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled).

In addition, the prerequisite of CSI-RS based L1-RSRP measurement is that SSB based L3 measurement or SSB based L1 measurement has been performed. If SSB index reading has been performed during L3 measurement, there is no need for additional time to obtain SFN index.

Proposal 2: If deriveSSB-IndexFromCell is not enabled, but UE has performed L1 measurement or L3 measurement with SSB index on the candidate cell, no additional time is needed to acquire the SFN information.

As to whether to include unknown cell case, we have no strong view. If we want to align with the rquirements of CHO, unknown cell case shoud be considered and PSS/SSS detection time of associatedSSB is the intra-frequency TPSS/SSS_sync_intra in clause 9.2.5.1. However, for SSB based L1-RSRP measurement, just known case has been considered and Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. When discussing in CSI-RS based L1-RSRP measurement part, we have reached consensus that only consider known cell. Based on this information, we prefer not consider unknown cell case for CSI-RS based L1-RSRP measurement.

Proposal 3: Prefer not to include unknown cell case for CSI-RS based L1-RSRP measurement.

##### Issue 1-2-4: General on Tfirst-RS and TRS-proc < Agreement > - [Tfirst-RS and TRS-proc] are included in Tinterrupt.

for this issue, we need to discuss whether T/F fine tracking can be performed before Tprocessing. TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change. Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell. Under this optimization, UE can reveive or transmit data on serving cell during the time duration of waiting for SSB which is used for fine time tracking on the target cell. For conditional LTM procedure, as network has no information of the exact instance when UE is to switch to another cell, network would keep scheduling the UE during the duration of waiting for the first SSB. According to proponent, conditional cell switch delay and interruption can be defined as follow:

For C-LTM, separate Tfirst-RS to two parts:

- Twait_RS: the time spent on waiting for the SSB occasion

- TRS: the time needed to get ready to receive SSB on target cell plus the SSB duration, which is 2 ms.

In our understanding, it is beneficial that T/F fine tracking if needed can be executed at first and then TLTM-processing during conditional LTM. However, it’s need to further discuss the value that the uncertainty in acquiring the first SSB for T/F tracking on the new cell. Maybe it can be discussed in further release.

Proposal 4: Prefer to reuse the legacy value of Tfirst-RS and TRS-proc in R18 LTM, and the further optimization on interruption time is also fine for us.

Conclusions

In this contribution, we put forward the following proposals on conditional LTM.

Proposal 1: For CSI-RS based L1-RSRP measurement, the time period used to acquire the SFN information is equal to 0 if the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled).

Proposal 2: If deriveSSB-IndexFromCell is not enabled, but UE has performed L1 measurement or L3 measurement with SSB index on the candidate cell, no additional time is needed to acquire the SFN information.

Proposal 3: Prefer not to include unknown cell case for CSI-RS based L1-RSRP measurement.

Proposal 4: Prefer to reuse the legacy value of Tfirst-RS and TRS-proc in R18 LTM, and the further optimization on interruption time is also fine for us.

References

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

- R4-2508283, WF on RRM requirements for NR_Mob_Ph4_Part2, China Telecom