3GPP TSG- WG4 Meeting # 116	R4-2510465

# Bengaluru, India, 25th – 29th August 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on LB-LB switching

Agenda Item:	7.7.3

Document for:	Approval

Introduction

In RAN4#114bis, the WF [1] for Low band carrier aggregation via switching was approved with the following issues.

Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination. Issue 2: Switching period location - Await outcome of RAN1 design Issue 3: Time mask for switching - Define the Tx on-off time mask requirements - FFS on how to define the Rx switching requirements to reflect where the switching period occurs - RAN4 has understanding that RAN1 will take switching period and timing advance into account for RAN1 scheduling pattern for low band CA via switching Issue 4: Considerations related to the Tx on-off time mask - Option 1: Specify transmit on/off time mask for FDD UL carrier/Pcell for switching between LB(FDD)-LB(SDL) - Option 2: Specify on->off time mask for switching from FDD to SDL carrier and specify off->on time mask for switching from SDL to FDD carrier - Other options are not precluded

In RAN4#115, the WF [2] was approved

Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered. Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

In RAN1 April and May meetings, the conclusion for Switching period location was concluded.

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier

This paper is to provide the left-over thinking concerning the time mask and related capability for LB-LB switching in Rel-19.

Time mask for LB-LB switching

It was agreed in the last two RAN1 meetings on switching gap location. It can be referred to determine the time mask for LB-LB switching.

Observation 1: RAN1 agreed a following for switching gap location for switching from FDD carrier to SDL carrier.

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier.

Switching gap ends at the end of the last slot on the “switch from” carrier

The specified time mask should reflect switching period location, at least for FDD-SDL switching, on which the location is on the last slot of switch-from carrier, without impact on the reception of SDL band. In the semi-static switching pattern, the transmissions configured on the switch-from band (FDD) avoid the OFDM symbols occupied by switching period. The switching period does not impact the start symbol(s) of the reception on switch-to band (SDL), as illustrated in Figure 1, and the vacant part in the carrier of the following figure is the time domain resource without channel/signal scheduling, or with the cancellation of configuration.

Figure 1 Time mask for low band CA via switching

Proposal 1: The time mask for LB-LB switching shows the switching period location for FDD-SDL switching.

Proposal 2: RAN4 capture Figure 1 as the time mask for LB CA via switching.

UE capability related to LB-LB switching

In the WID, the example band combination is CA_n5-n29. But before the approved WI, companies discussed more about CA_n12-n29. CA_n12-n29 is quite special that the frequency isolation between band n12 UL and n29 DL is only 1MHz. Thus, it is difficult to design filter for n12+n29. Con-current operation for (n12UL, n29DL) is not supported. It requires network scheduling restriction.

The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling. However, the case is different at least for CA_n12-n29 via switching.

Observation 2: The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling.

Meanwhile, con-current operation for (n12DL, n29DL) is not supported either, since Rel-19 is to specify the switching between case 1 and case 2, and there is no dualDL for the two bands.

Figure 2 Frequency range illustration for CA_n12-n29

The definition of carrier aggregation is Aggregation of two or more component carriers in order to support wider transmission bandwidths [3]. For inter-band DL CA, the simultaneous Rx operation for both the DL bands must be supported, resulting in wider bandwidth. Therefore, for Rel-19 low band FDD-SDL inter-band CA via switching that is not capable of con-current Rx operation, i.e., typical inter-band CA. It means that a band combination supports LB-LB CA does not always support the corresponding typical inter-band CA, such CA_n12-n29. Therefore, the capability for LB-LB CA should be reported separately from BandCombinationList for inter-band CA.

Observation 3: A band combination support a LB-LB CA does not always support the corresponding typical inter-band CA, given that normally for inter-band DL CA band combination, con-current Rx operation for both the DL bands is supported, while there are combinations whose LB-LB CA is possible, but the corresponding typical CA is not feasible, such as CA-n12-n29.

However, for LB CA via switching, no typical requirements for inter-band CA are needed to be specified, given that there is no con-current operation for FDD(UL/DL) and SDL.

Table 1 Requirements to be met comparison for different kinds of inter-band CA

| Requirements to be met | For typical CA | For LB CA via switching e.g.,: CA_n12-n29 | For typical CA supporting LB CA via switching e.g.,: CA_n5-n29 |
| --- | --- | --- | --- |
| Configurations and bandwidth combination sets | Yes |  | Yes |
| UE maximum output power for Inter-band CA with uplink CA configuration | Yes |  | Yes |
| ΔTIB,c | Yes |  | Yes |
| ΔRIB,c | Yes |  | Yes |
| Reference sensitivity exceptions | Yes |  | Yes |
| Time mask for LB switching |  | Yes | Yes |

Observation 4: No typical CA requirements are needed for inter-band CA band combination that only supporting LB CA via switching.

Proposal 3: For Rel-19 sub-1GHz FDD-SDL band combinations, the capability should be separately reported from BandCombinationList for inter-band CA.

The UE capability should reflect the UE RF front-end architecture. The con-current operation of Rx-Tx, Rx-Rx depends on the UE architectures.

(a) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) is not supported in Rel-19

(b) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) can be supported in future release

Figure 3 UE RF front-end architectures for inter-band FDD-SDL CA_n12-n29

The filters for band n12 and n29 are separately designed in the reference UE RF front-end architecture in Figure 3(a) The UE architecture for CA_n12-n29 that simultaneous operation (n12DL, n29DL) is not supported in Rel-19. Some companies also proposed that in the future release the concurrent DL on the two DL bands should be supported. The capability is related to scheduling restriction. The capability design should be scalable, not only reflect different UE architectures, but also apply to the requirements in different releases, and the inter-band CA with more than two bands.

Observation 5: The capability of low band CA via switching is also related to scheduling restriction.

The sub-1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration. For FDD-SDL inter-band CA:

Simultaneous (FDD, SDL) is not supported.

Proposal 4: The sub-1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration in Rel-19.

- The con-current reception on FDD band and SDL band is not supported.

- The con-current operation between FDD UL and SDL is not supported.

Conclusion

In this contribution we discussed on low band CA via switching, according to the analysis, we have the following observation and proposals:

Observation 1: RAN1 agreed a following for switching gap location for switching from FDD carrier to SDL carrier.

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier.

Switching gap ends at the end of the last slot on the “switch from” carrier

Observation 2: The capability simultaneousRxTxInterBandCA is designed for TDD-TDD and TDD-FDD inter-band NR CA. And for FDD-FDD/ FDD-SDL inter-band NR CA, simultaneous Rx-Tx is mandatorily supported without signaling.

Observation 3: A UE support a LB-LB CA does not always support the corresponding typical inter-band CA, given that normally for inter-band DL CA band combination, con-current Rx operation for both the DL bands is supported, while there are combinations whose LB-LB CA is possible, but the corresponding typical CA is not feasible such as CA-n12-n29.

Observation 4: No typical CA requirements are needed for inter-band CA band combination that only supporting LB CA via switching.

Observation 5: The capability of low band CA via switching is also related to scheduling restriction.

Proposal 1: The time mask for LB-LB switching shows the switching period location for FDD-SDL switching.

Proposal 2: RAN4 capture Figure 1 as the time mask for LB CA via switching.

Proposal 3: For Rel-19 sub-1GHz FDD-SDL band combinations, the capability should be separately reported from BandCombinationList for inter-band CA.

Proposal 4: The sub-1GHz FDD-SDL low band CA switching capability design should take scheduling restriction into consideration in Rel-19.

- The con-current reception on FDD band and SDL band is not supported.

- The con-current operation between FDD UL and SDL is not supported.

- References

[1] R4-2505223, WF on UE RF requirements for LB CA via switching, Apple, RAN4#114bis, April, 2025.

[2] R4-2508118, WF on UE RF requirements for LB CA via switching, Apple, RAN4#115, May, 2025

[3] TS 38.101-1

3GPP TSG- WG4 Meeting # 116	                                      R4-250xxxx

Bengaluru, India, 25th – 29th August 2025

Title:	Draft LS on capability of NR_LBCA_Sw

Response to:

Release:	Rel-19

Work Item:	NR_LBCA_Sw

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:

E-mail Address: xxx@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	Nil

1. Overall Description:

RAN4 has discussed and concluded on the UE capability demand of low band CA via switching:

- The capability should be separately reported from BandCombinationList for inter-band CA

- The sub-1GHz FDD-SDL low band CA switching capability design should reflect scheduling restriction of the feature in Rel-19.

  - The con-current reception on FDD band and SDL band is not supported.

  - The con-current operation between FDD UL and SDL is not supported

2. Actions:

To RAN2:

ACTION: RAN4 respectfully requests RAN2 to take into consideration of RAN4’s need on UE capability for low band CA via switching when design the capability signaling in Rel-19

.

3. Date of Next TSG WG RAN4 Meeting:

TSG-RAN4 Meeting#116bis  	 				13 – 17 October 2025		            Prague, Czech Republic

TSG-RAN4 Meeting#117  	 			    	17 – 21 November 2025		                    Dallas,USA