3GPP TSG-RAN WG4 Meeting #116	R4-2510479

Bengaluru, India, August 25th – 29th, 2025

Source:	GIST, NRRA

Title:	Discussion on Introducing Near-Field to Far-Field Transform Method for OTA TRP Measurement Method in TR 38.870

Agenda Item:		7.10.2

Document for:	Discussion

Introduction

As 5G-Advanced and upcoming 6G technologies introduce increasingly complex RF front-end architectures, the demands on OTA (Over-the-Air) TRP (Total Radiated Power) test methodologies continue to grow. In TR 38.870, two OTA TRP measurement methods are currently under discussion: Anechoic Chamber (AC) method and Reverberation Chamber (RC) method. To expand this discussion, we propose considering a third method: Near-Field to Far-Field Transform (NFTF).

NFTF methods have previously been described in TR 38.810 alongside Direct Far Field (DFF) and Indirect Far Field (IFF) approaches. While IFF (as implemented via CATR) has matured into a widely accepted standard, NFTF remains under-utilized in formal TRP validation despite its potential. Given the emergence of highly directional, beamforming, and AI-adaptive UEs, NFTF presents key advantages in measurement speed and flexibility.

This contribution outlines the justification and technical background for including the NFTF method in TR 38.870 as a study-level candidate.

NFTF Measurement Overview

NFTF reconstructs the far-field 3D radiation pattern by collecting both amplitude and phase information over a spherical surface in the near-field. The accuracy of this approach is supported by:

- Fine-grained spatial sampling using mechanically or electronically distributed probe arrays

- Accurate synchronization and calibration of probe modules

- Transform algorithms that preserve fidelity even for dense beam environments

While AC (IFF) remains dominant due to existing infrastructure and broad applicability, it suffers from limited time efficiency. RC methods have been suggested as alternatives, but they provide low spatial resolution and reduced TRP accuracy due to statistical uncertainty. In contrast, the NFTF method offers:

- High spatial resolution

- Reliable TRP accuracy

- Significant reduction in measurement time through parallelized sensing

Although NFTF does not currently support TRS efficiently, its advantages in speed, reduced DUT movement, and angular fidelity make it especially attractive for:

- Regulatory pre-testing

- Early-stage product debugging

- Testing of wearable, XR, UAV devices with complex beam patterns

- Devices with limited battery capacity

Considering these strengths, we suggest that NFTF be further discussed as a potential addition to future OTA TRP methodologies. Its inclusion may provide enhanced test flexibility and measurement fidelity for emerging device categories.

Motivation

The prototype system RRA-MAP (Rapid Radio Assessment Multi-Antenna Probing System), jointly developed by GIST, RRA, and CSU, demonstrates a full-sphere NFTF array using simultaneously active probes and GPS-synchronized timing. Key features include:

- Wideband coverage (3 – 60 GHz) via multi-arc spherical arrays

- Simultaneous V/H polarization sampling

- Rubidium-clock phase synchronization

- Drastically reduced mechanical movement for fast measurement

Earlier contributions have established:

- Theoretical foundation and calibration methodology

- Array operation with simultaneous probing

- Practical UE measurement procedures

While AC and RC remain essential for full certification, we believe it is appropriate for RAN4 to include NFTF as a study item in TR 38.870, especially in light of:

- Increasing global interest in multi-probe accelerated OTA testing

- Growing need to evaluate ISAC (Integrated Sensing and Communication) scenarios

- Demand for diverse, accurate, and rapid OTA test methodologies

Notably, Microwave Vision Group (MVG) has introduced several high-speed spherical NFTF systems such as SG Evo and SG 24, which employ multi-probe parallel sampling for fast TRP evaluation across multiple frequencies. MVG has also collaborated with Anritsu to validate Wi-Fi 7 OTA performance using multi-probe systems. These commercial deployments reflect strong industry alignment with NFTF-type measurement.

Additionally, Keysight Technologies has worked with MVG to demonstrate high-speed OTA testing systems based on NFTF concepts, further reinforcing the global momentum behind NFTF-like architectures.

Proposed Text Addition for TR 38.870

We propose the following clause be added to Section 5:

5.1.3 Definition of the Total Radiated Power (TRP) for NFTF Method

The TRP for NFTF is calculated by integrating the reconstructed 3D far-field radiation pattern, derived from near-field amplitude and phase data sampled over a spherical surface surrounding the DUT. The transformation is performed using a calibrated probe array or scanning system. Calibration and synchronization of the probes are essential to ensure accuracy.

The near-field amplitude and phase are transformed to far-field using spherical wave expansion. TRP is then calculated in the same manner as in the AC method, using equation (5.1). This method is particularly suitable for rapid prototyping, regulatory diagnostics, and measuring highly directional or dynamically adaptive devices.

Conclusion

We propose the inclusion of NFTF as a study-level OTA TRP test method in TR 38.870. While not intended to replace AC or RC methods, NFTF significantly broadens the available toolbox for future OTA testing, especially for advanced and constrained UEs. Given the ongoing development and the interest from industry and academia, we recommend initiating a formal study on the NFTF method.

- Reference

- TR 38.870: "Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)"

- TR 38.810: "NR; Study on test methods"

- R4-2307504: "CR for TR38.810 Integrating simultaneously active probe concept in the NFTF method"

- R4-2400727: "Discussion on NFTF System for UE Transmitter Characteristics"

- R4-2407599: "Discussion on the Operation of the Probe Module Array"

- R4-2411308: "UE RF Testing Procedure under NFTF Measurement Setup"

- R4-2507095: "Discussion on NFTF Method for OTA TRP Measurement"

- Microwave Vision Group SG Evo, SG 24 Data Sheets

- Anritsu and MVG collaborate to support Wi-Fi 7 OTA measurement solution, https://www.anritsu.com/en-us/test-measurement/news/news-releases/2025/2025-04-24-us01

- Keysight Technologies, “Antenna measurement using multi-probe scanning.”