3GPP TSG RAN WG4 Meeting #116	R4-2510487

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.19.3.1

Source:	vivo

Title:	Discussion on RAN4 impacts for RRM measurement prediction

Document for:	Discussion

- Introduction

In RAN#105 meeting, the revised SID [1] was approved and RAN4 work is copied as

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this contribution, we further provide our views on the RAN4 impacts for RRM measurement prediction based on the latest WF[2].

- Discussion

  - Impacts of measurement error on prediction accuracy

- Issue 2-4: Impacts of measurement error on prediction accuracy - Background: The following agreement was reached in RAN4#114 meeting: Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.. Furthermore, following WF was suggested in RAN4#114bis meeting: WF: Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.. - Proposals and Observations: - Proposal 1 (vivo): On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline: - For BB error modeling: - Link-level simulation approach is considered - For RF error, - For error modeling, the following two modelings can be the candidates. - Alt 1: Uniform distribution - Alt 2: Truncated Gaussian distribution - RF error application, each of Tx beam is independent - Proposal 2 (Ericsson): Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements. - RF error in dB for FR1 is characterized by Gaussian distribution with 〖μ=0,σ〗^2 = 2.5 truncated at ±2.5. - RF error in dB for FR2 is characterized by Gaussian distribution with 〖μ=0,σ〗^2 = 4 truncated at ±4. - Proposal 3 (Nokia): In Study Item, RAN4 to collect analysis on potential modeling methods and at least capture the collected analysis in the TR. - Proposal 4 (Nokia): RAN4 to capture the following error modeling principles in the TR: - RF and baseband measurement error modeling may be considered. - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband. - Alternative modeling approaches are not precluded and can be explored in the Work Item phase. - Proposal 5 (Qualcomm): RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g. - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach. - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction. - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction. - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions. - Proposal 6 (OPPO): Consider the following options to model the measurement error: - Option 1: For RF error, use the truncated Gaussian distribution under ±X dB; for baseband error, use the link level simulation assumptions to generate L3-RSRP difference as baseband error. - Option 2: For RF and baseband error, use the truncated Gaussian distribution under ±X dB and ±Y dB, respectively. - Option 3: For the sum of RF and baseband error, use the truncated Gaussian distribution under ±X dB. - Proposal 7 (LG Electronics): RAN4 to consider only Gaussian error model regardless what RF error model or BB error model is. - Proposal 8 (Xiaomi): RAN4 to study the factor and impact of measurement error on RRM prediction. - Proposal 9 (MediaTek): For AI mobility, in FR1, measurement error should be added as: - BB error: - Added independently per Tx beam per cell per measurement occasion according to the corresponding SINR. - The distribution of BB error under certain SINR is obtained through LLS assuming one shot measurement. - RF error: - Modelled as Gaussian distribution under ±3 dB RF error (〖μ=0,σ〗^2=3). - A random RF error is generated for all the beams of the same frequency layer per SMTC. - Proposal 10 (Apple): RAN4 to investigate the impact of measurement errors (RF and baseband) in AI/ML model input data on the accuracy of predicted measurements. Additionally, RAN4 should investigate whether RAN2 simulation results and conclusions regarding achievable prediction accuracy remain valid under such measurement errors for scenario 2. - Proposal 11 (Apple): For the multi-link RSRP temporal-prediction extension of Scenario RAN4 to investigate the impact of cell dependent measurement errors (different SINRs and channel conditions) to the prediction accuracy of the AI/ML models (scenario #4) - Proposal 12 (Apple): RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. RAN4 should validate any RAN2 conclusions in the presence of realistic measurements errors - Proposal 13 (Apple): RAN4 to investigate modelling measurement errors as a function of SINR. This analysis must account for variations in propagation environments across serving and neighboring cells, as measurement errors are inherently influenced by cell-specific SINR conditions. - Recommended WF - RAN4 to capture following factors to be considered for error models to consider the impact of measurement errors on prediction accuracy - RF and baseband measurement error modeling may be considered. - A uniform or a Gaussian distribution can be used as a model for measurement error for RF and baseband. - Alternative modeling approaches are not precluded and can be explored in the Work Item phase.

Regarding the impact of measurement errors, we believe the error modeling approach can directly leverage the existing discussions on AI/ML for NR Air Interface as a baseline. Below is the error modeling method considered during AI/ML beam simulation.

For BB error, the Link-level simulation results were selected after discussion.

For RF error, a Truncated Gaussian distribution under ±4.5 dB RF error (μ=0, σ²=4.5) is used in AI/ML. The Uniform distribution was initially excluded to better align with simulation results. The difference between this model and the Truncated Gaussian distribution in terms of BM prediction is minimal. Therefore, for AI mobility, we believe both models can be retained for now. The Fixed value model can be excluded first. From the BM simulation results, it can be observed that the prediction accuracy is not impacted by the 4 dB RF offset. A possible reason is that the trend of all datasets remains unchanged when a 4 dB offset is applied to all RSRP elements. Specifically, for RF error, "Independent for each Tx beam" is adopted.

Based on the above analysis, we propose the following for measurement error modeling for AI mobility.

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2

  - RF error application, each of Tx beam is independent

During the SI (Study Item) phase, we believe this baseline is sufficient. The table can be captured in the TR (Technical Report) to facilitate further evaluation.

Proposal 1: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2

  - RF error application, each of Tx beam is independent

On the other hand, during the previous meeting's discussion on AI/ML for NR Air Interface regarding measurement error, an important additional aspect was introduced: the phase in which measurement error is considered. This includes the following specific cases, now captured in the latest simulation assumptions for BM [3]. For the Beam Management use case in AI/ML for NR Air Interface, we observed performance variations in prediction accuracy across Case 2a, Case 2b, and Case 3. Similarly, we believe that the timing of applying measurement error (i.e., during which phase) may also impact RRM measurement prediction performance. Therefore, we recommend incorporating this factor into the analysis.

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth. - Case 2: - Case 2a No error will be considered in training dataset and ground-truth. Error will be considered in model input during inference - Case 2b No error will be considered in training dataset and ground truth for training. Error will be considered in model input during inference and ground-truth for inference - Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Specifically, since the definition of ground truth is still under discussion, whether to account for errors in the ground truth can be deferred until further conclusions are reached on its definition.

Proposal 2: The measurement error analysis can consider the impact on prediction performance

- [Case 1]: when errors are only added to the model input during the prediction phase

- [Case 2]: When errors are added to the model input both during both the training phase and prediction phase

- Whether to consider adding errors to the ground truth depends on the discussion on the definition of ground truth

- Summary

Proposal 1: On the impact of measurement errors, RAN4 to leverage the outcome of the error modeling approach in AI/ML for NR Air Interface. Specifically, the following error modelings can be as a baseline:

- For BB error modeling:

  - Link-level simulation approach is considered

- For RF error,

  - For error modeling, the following two modelings can be the candidates. Further down-selection can be done in WI phase

    - Alt 1: Uniform distribution

    - Alt 2: Truncated Gaussian distribution(μ=0, σ^2=[X]), value [X] can be further determined respectively for FR1 and FR2

  - RF error application, each of Tx beam is independent

Proposal 2: The measurement error analysis can consider the impact on prediction performance

- [Case 1]: when errors are only added to the model input during the prediction phase

- [Case 2]: When errors are added to the model input both during both the training phase and prediction phase

- Whether to consider adding errors to the ground truth depends on the discussion on the definition of ground truth

- References

- RP-242393 Revised SID on AIML for mobility in NR, OPPO

- R4-2508268 WF on FS_NR_AIML_Mob_Part1, Nokia

- R4-2508081 Updated simulation assumption for BM, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT