3GPP TSG RAN WG4 Meeting #116	R4-2510495

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.14.6

Source:	vivo

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In the last meeting, regarding the RRM test cases design for Fast SCell activation for UEs supporting Rel-18 EMR, there were some progress and the outcome was captured in WF[1]. In this document, we will provide our further analysis based on it.

- Discussion

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

In the previous meeting, the first three scenarios in the test case list were approved. For "Direct SCell activation at SCell addition for FR2," since legacy EMR does not include test cases for FR2 deployment, we can consider tests only under Configuration 1 and Configuration 2 for this scenario. The definition of the scenario "Direct SCell activation at SCell addition + FR2 + Configuration 3" should be excluded.

Proposal 1: For Direct SCell activation at SCell addition for FR2, RAN4 to define test cases under Configuration 1 and Configuration 2, excluding Configuration 3.

Regarding whether test cases should be defined for Configuration 3, the existing EMR tests aim to verify whether the UE can properly perform early measurements in idle mode and complete EMR result reporting. However, the purpose of the tests here is mainly to verify whether the UE can utilize EMR results for fast SCell activation. From this perspective, weprefer to validate this scenario and defining test cases.

Proposal 2: For UE capabilities and network configurations, RAN4 to consider

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- Summary

Proposal 1: For Direct SCell activation at SCell addition for FR2, RAN4 to define test cases under Configuration 1 and Configuration 2, excluding Configuration 3.

Proposal 2: For UE capabilities and network configurations, RAN4 to consider

- Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

- References

[1]	R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT