# 3GPP TSG-RAN WG4 Meeting #116                                                                        R4-2510500

# Bengaluru, IN, 25th-29th August, 2025

Agenda Item:	7.6.3.2.1

Source: 	CSCN

Title: 	Views on HPUE for NR-NTN

Document for:	Discussion

- Introduction

According to the  Rel-19 RAN4 NTN WID, one of major objectives is to study and specify HPUE (PC2, PC1.5, PC1) for NR-NTN/IoT-NTN with target device types as handheld UE and/or non-handheld UE [1].

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, some views on remaining open issues for NR-NTN HPUE are provided.

- Discussion

Power class and applicable device types

Supported device types for each PC depend on feasibility study and corresponding requirements. In the table below, we summarize the current progress.

Table 1 Applicable device types for different PC

| Power class | Applicable device types and number of antennas |
| --- | --- |
| PC2 | Handheld: 1Tx, 2Tx Non-handheld: 1Tx, 2Tx |
| PC1.5 | Non-handheld with 2Tx FFS for others pending on available time in Rel-19 |
| PC1 | Non-handheld only: 1Tx, [4Tx] |

For NTN operation, uplink coverage is a major bottleneck. In addition to non-handheld device, we are interested in supporting PC1.5 and PC1 for handheld device as well. For PC1.5, we understand there are some limitations on RF front-end design (e.g. PA and Duplexer). On existing TN TDD bands, PC1.5 was already supported for handheld devices with 2Tx (2 PC2 PA).  One possible way to support PC 1.5 for handheld device on NR NTN FDD bands is to reuse PC2 PA with HD-FDD mode which is also supported on TDD NR bands.

Considering the limitations of dual-antenna placement in handheld NTN terminals (with both antennas facing upward) and the dominance of Line-of-Sight (LOS) propagation in NTN channels—where achieving a 3dB gain with a 2TX configuration remains challenging—it is recommended to prioritize research on 1TX PC1.5 technologies.

In the last RAN4 meeting, several companies also proposed to study power boosting schemes to achieve high power, e.g.  requirements relaxation and RB allocation restriction [2] [3]. If supporting PC1.5 for handheld devices with FDD full-duplex is relatively difficult due to the limitations on RF front-end design at this stage, it is recommended to perform power boosting operations on some sub-carriers to achieve equivalent high power, thereby improving the uplink performance.

PC1.5

Previous agreements: Issue 1-10: PC 1.5 UE architecture assumption Agreement: - PC1.5 for non-handheld NR-NTN UE is primarily considered with 2Tx in Rel-19 - PC1.5 with 1Tx is not precluded but with low priority. It can be treated once PC1.5 with 2Tx work is done.

Issue 1-12: Power boosting for PC2 Way forward: - Further discuss evaluate the PC2 NTN power boosting option with low priority if time allows, and not target to define in Rel-19. - Further discuss following assumptions: - PC2 1Tx PA using existing TN impairment and targeting inner third allocations. - Applicable to both TN and NTN. - FFS applicability of such boosting to multi-Tx based on PC2 PAs.

Proposal 1: Support 1Tx/2Tx PC1.5 for handheld devices with HD-FDD mode.

Proposal 2:  Prioritize 1TX PC1.5 research for handheld devices.

Observation 1: It’s worth to further study power boosting schemes for both PC2 and PC1.5.

- Conclusion

In this contribution, views on remaining open issues for NR-NTN HPUE are provided.

Proposal 1: Support 1Tx/2Tx PC1.5 for handheld devices with HD-FDD mode.

Proposal 2:  Prioritize 1TX PC1.5 research for handheld devices.

Observation 1: It’s worth to further study power boosting schemes for both PC2 and PC1.5.

Reference

[1] RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”

[2] R4-2507567, “Topic summary for [115] [128] NR_IoT_NTN_HPUE_part1”, Moderator (Samsung)

[3] R4-2507568, “Topic summary for [115] [129] NR_IoT_NTN_HPUE_part2”, Moderator (vivo)

[4] R4-2508008,	 “WF on NR_IoT_NTN_HPUE_part1”, Samsung

[5] R4-2508007,	“WF on NR_IoT_NTN_HPUE_part2”,vivo