3GPP TSG-RAN4 Meeting #116	R4-2510508

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 – Regional PFD requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of the Regional PFD requirements for NTN Ku-bands |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for the NTN Ku-bands (n248 and n509) and PFD requirements from FCC and CEPT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN Ku-bands VSAT requirements will not be properly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.521-5: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[3]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[4]		3GPP TS 38.108: "NR; Satellite Node radio transmission and reception"

[5]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[6]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[7]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[8]	3GPP TS 38.331: " Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[10]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[11]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[12]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[13]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[15]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[16]	Recommendation ITU-R SM.329, "Unwanted emissions in the spurious domain".

[17]	EN 303 978, Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.2, 2016-10.

[18]	EN 301 459, Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1, 2016-05.

[19]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE.

[20]	ECC Decision(18)05,  The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz, 5 November 2021

<End of the change>

<Start of the change>

## 9.7	Additional regional requirements indicated by NS

### 9.7.1	General

Additional regional requirements can be signalled by the network. Each group of additional regional requirements is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR NTN frequency band number of the applicable FR2-NTN operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [8].

Table 9.7.1-1 specifies the additional regional requirements with their associated network signalling values, the applicable satellite orbit scenario(s) and applicable FR2-NTN operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 9.7.1-2.

Table 9.7.1-1: Additional regional requirements indicated by Network Signalling label

| Network Signalling label | Requirements (clause) | Applicable Satellite orbit scenario | NR satellite Band | Channel bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| NS_200N |  | GSO and LEO | Table 5.2.3-1 | 50, 100, 200, 400 |
| NS_201N | Clause 9.2.2.3 Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 Clause 9.6.1.2 Clause 10.8 | GSO | n512 | 50, 100, 200, 400 |
| NS_202N | Clause 9.5.3.2 Clause 9.5.3.3 Clause 9.6.1.1 | LEO | n512 | 50, 100, 200, 400 |
| NS_2XX1N | Clause 9.7.2.1 | GSO and LEO | n248, n509 | All channel bandwidth according to 5.3.5 |
| NS_2XX2N | Clause 9.7.2.2 | LEO | n248, n509 | All channel bandwidth according to 5.3.5 |
| NS_2XX3N | Clause 9.7.2.3 | LEO | n248, n509 | All channel bandwidth according to 5.3.5 |

Table 9.7.1-2: Mapping of network signalling label

| NR satellite band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n512 | NS_200N | NS_201N | NS_202N |  |  |  |  | Reserved |
| n511 | NS_200N |  |  |  |  |  |  | Reserved |
| n510 | NS_200N |  |  |  |  |  |  | Reserved |
| n509 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N | NS_2XX1N | NS_2XX2N | Reserved |
| n248 | NS_200N | NS_203N | NS_204N | NS_205N | NS_206N | NS_2XX1N | NS_2XX2N | Reserved |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of 3GPP TS 38.331 [8]. NOTE 2: For band n511 and n510, only NS_200N can be used to map. |  |  |  |  |  |  |  |  |

### 9.7.2	Power Flux Density

#### 9.7.2.1	Requirement for network signalling value “NS_2XX1N”

When "NS_2XX1N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

For aeronautical NTN VSAT type 5 and type 6 within line-of-sight of the territory of a foreign administration where fixed service networks have primary allocation in this band, the requirements specified in Table 9.7.2.1-1 apply.

Table 9.7.2.1-1: PFD requirements for aeronautical NTN VSAT type 5

| Frequency Range (GHz) | θ value | PFD value (dBW/m2) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | θ ≤ 40° | -102 + 0.5 * θ | 1 |
|  | 40° ≤ θ ≤ 90° | -82 | 1 |

#### 9.7.2.2	Requirement for network signalling value “NS_2XX2N”

When "NS_2XX2N" is indicated in the cell, the following requirements shall apply to NTN VSAT.

For NTN VSAT type 5 and type 6 transmitting towards LEO satellite and operating in the territory of administrations which authorizes FS systems in the band, the requirements specified in:

- Table 9.7.2.2-1 apply for NTN VSAT type 5 and type 6 installed on aircraft [20].

- Table 9.7.2.2-2 apply for NTN VSAT type 5 and type 6 installed on vessels [20].

- Table 9.7.2.2-3 apply for land based NTN VSAT type 5 and type 6 installed [20].

Table 9.7.2.2-1: PFD requirements for NTN VSAT type 5 installed on aircraft

| Frequency Range (GHz) | θ value | PFD value (dBW/m2) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 14.25 – 14.5 | θ ≤ 5° | -92 | 1 |
|  | 5° ≤ θ ≤ 40° | -97 + θ | 1 |
|  | 40° ≤ θ ≤ 90° | -57 | 1 |

Table 9.7.2.2-2: PFD requirements for NTN VSAT 5 installed on vessels

| Frequency Range (GHz) | Height (m) | PFD value (dBW/m2) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 14.25 – 14.5 | 80 | -86 | 1 |

Table 9.7.2.2-3: PFD requirements for land based NTN VSAT 5

| Frequency Range (GHz) | Height (m) | PFD value (dBW/m2) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 14.25 – 14.5 | 30 | -86 | 1 |

#### 9.7.2.3	Requirement for network signalling value “NS_206N”

For NTN VSAT type 5 and type 6 transmitting towards LEO satellite, the requirements specified in:

- Table 9.7.2.3-1 apply for NTN VSAT type 5 and type 6 installed on vessels [20].

- Table 9.7.2.3-1 apply for land based NTN VSAT type 5 and type 6 [20].

Table 9.7.2.3-1: PFD requirements for NTN VSAT 5

| Frequency Range (GHz) | PFD value (dBW/m2) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.47 – 14.5 | -139 | 150 |
| NOTE: This limit could not be exceeded more than 2% of the time |  |  |

<End of the change>