3GPP TSG-RAN4 Meeting #116	R4-2510510

# Bengaluru, India, 25th August 2025 – 29th August 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – A-MPR

Agenda item:		7.4.3

Document for:		Approval

# Introduction

In RAN#106, a WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

Based on the agreed WF ([2] and [3]) from last RAN4#114bis and RAN4#115 meetings, this contribution shares our further A-MPR simulation results.

# Discussion

## A-MPR simulation results

### Assumptions

The following assumptions have been considered for our A-MPR simulations:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB0

- Carrier Leakage: -28 dBc

- I/Q image: -28 dBc

- CIM3: -60 dBc

### NS_12

As agreed in the WF ([2]), we considered 7 MHz deployment in the 814-824 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-813.5 MHz frequency range.

Based on those simulations results, we would confirm the following RAN4#155 meeting’s agreements:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A2 |
|  |  | ≤1.8 | >0.72 | A1 |
|  |  | >1.8 | >1.44 | A1 |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

### NS_13

As agreed in the WF ([2]), we considered 7 MHz deployment in the 817-824 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-816 MHz frequency range.

Based on those simulations results, we would confirm the following RAN4#155 meeting’s agreements:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A1 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A7 |

| Modulation/Waveform | A1 | A2 | A7 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 4.5 |

### NS_15

As agreed in the WF ([2]), we considered 7 MHz deployment within the 824-849 MHz range and the additional requirement is -42 dBm/6.25 kHz in the 806-816 MHz frequency range.

Based on those simulations results, we would confirm the following RAN4#155 meeting’s agreements:

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A1 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A3 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A2 |
|  |  | ≥ 5.94 | ≤ 2.7 | A3 |

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 |

# Conclusion

In this contribution, we shared our additional A-MPR simulations results and confirm the following pre-agreement from last RAN4#115 meeting ([3]):

Proposal: Specify the following A-MPR requirements for 7 MHz channel bandwidth:

NS_12:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz | 817.5 ≤ Fc ≤ 820.5 | ≤1.8 | ≤0.72 | A2 |
|  |  | ≤1.8 | >0.72 | A1 |
|  |  | >1.8 | >1.44 | A1 |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 7 | ≤ 6 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 9.5 |  |

NS_13:

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7MHz |  | ≤1.8 | <1.44 | A1 |
|  | 820.5 = Fc | ≤1.8 | ≥1.44 | A2 |
|  |  | >1.8 | ≥1.8 | A7 |

| Modulation/Waveform | A1 | A2 | A7 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3.5 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 | ≤ 4.5 |

NS_15:

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 7 MHz | 841.5 < Fc ≤ 845.5 | ≥ 4.5 | > 0 | A1 |
|  |  | < 4.5, ≥ 2.16 | ≥ 1.8 | A2 |
|  |  | ≤ 1.44 | ≤ 0.36 | A3 |
|  | 838 < Fc ≤ 841.5 | > 0 | > 2.7 | A2 |
|  |  | ≥ 5.94 | ≤ 2.7 | A3 |

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 |

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2505098, WF on requirements for 7 MHz channel bandwidth, Ericsson

- R4-2507954, WF on requirements for 7 MHz channel bandwidth, Ericsson