3GPP TSG-RAN4 Meeting #116	R4-2510514

# Bengaluru, India, 25th August 2025 – 29th August 2025

Source:			Ericsson

Title:			 US 4.9 GHz band - Adjacent spectrum aspects

Agenda item:		6.14.2.1

Document for:		Discussion

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US, pending on the resolution of the litigation.

According to this WI, RAN4 should first study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band. This contribution focuses on the adjacent spectrum aspects of this new 4.9GHz US band.

# Discussion

## Background

In last RAN#108 meeting, a new WI [1] was approved to introduce the US TDD 4.9 GHz band (4940–4990 MHz) after FCC released the Amendment of Part 90 of the Commission’s Rules, WP Docket No. 07-100, Eighth Report and Order on 22 October 2024. The primary purpose of this band is to support public safety agencies in their mission-critical operations, particularly for applications that require high bandwidth.

The following objectives have been identified:

- Study coexistence issues with adjacent spectrum and other users of the 4940-4990 MHz band.

- Specify a new NR TDD operating band for the frequency range, 4940-4990 MHz.

- Specify band numbering, system parameters and RF characteristics for the channel bandwidths defined for the new band.

- Address potential BS and UE co-existence issues

- Specify the BS RF requirements.

- Specify the UE RF requirements for the new bands. Reuse NR band n79 UE hardware assumption if possible.

- Update the related technical specifications to include support for the new band.

The work in RAN4 should then start with the coexistence study scheduled for one meeting cycle.

This contribution is addressing this aspect, focusing on the coexistence with adjacent services.

## Adjacent services and CFR 47 part 90

### FCC Table of Frequency Allocation

According to the FCC's Table of Frequency Allocations, as extracted in below Figure 1, the following services are allocated adjacent to the 4.9 GHz US band:

- Fixed and Mobile in the 4.4-4.94 GHz frequency range.

- RAS in the 4.99-5.00 GHz frequency range.

- Aeronautical mobile/mobile-satellite/radionavigation in the 5.00-5.091 GHz frequency range, covered by CFR 47 part 87.

- Aeronautical mobile/mobile-satellite/radionavigation in the 5.091-5.15 GHz frequency range, covered by CFR 47 parts 25 and 87.

Figure 1: Extract of FCC Table of Frequency Allocations

And, as also mentioned in the FCC tables of frequency allocations, the regulatory requirements for the 4.9 GHz US band are specified in FCC CFR 47 part 90.

Those technical conditions shall ensure coexistence with the adjacent services, RAN4 relying on Regulators’ decision for the coexistence with adjacent services (RAN4 is focusing on coexistence with adjacent 3GPP systems, not with other type of services).

Observation: The regulatory requirements for the 4.9 GHz US band are specified in CFR 47 part 90.

It could be noted that, for certain frequency ranges (e.g. below 4.9 GHz, US Navy spectrum usage in some locations, or protection of Radio Astronomy site), additional coordination might be needed, but this is out of scope of 3GPP RAN4 when setting RF requirements for a band.

### CFR 47 part 90

In §90.210, the emission Masks L (for low power transmitted, lower than 20dBm, Figure 2) and M (for high power transmitted, greater than 20dBm, Figure 3) have been specified for the 4.9 GHz band, noting they apply for frequencies above 150% of the authorized bandwidth.

Figure 2: Mask L						Figure 3: Mask M

The power limits are specified in §90.1215. They depend on the channel bandwidth for low power devices and shall not exceed 21dBm/MHz (peak) for high power devices, assuming a maximum antenna gain of 9 dBi.

Those requirements should be the starting point when specifying both BS and UE RF requirements for this new 4.9GHz US band, monitoring any further update.

Also, §90.1209 specifies the policies governing the use of the 4940-4990 MHz band, but this is usually out of RAN4 scope.

# Conclusion

In this contribution, we studied the adjacent spectrum aspects of the new 4.9 GHz US band and made the following observation:

Observation: The regulatory requirements for the 4.9 GHz US band are specified in CFR 47 part 90.

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu