3GPP TSG-RAN WG4 Meeting #116	R4-2510519

# Bengaluru, India, August 25th – 29th, 2025

Source: 	China Telecom

Title: 	Views on uplink CA_n5-n8 with non-concurrent n5 DL and n8 UL

Agenda Item:	6.11.2

Document for:	Approval

Introduction

In the last RAN4#115 meeting, the WF [1] agreed the feasible solution for CA_n5-n8 with non-concurrent n5 DL and n8 UL. This contribution provides our views on left issues on RRM impact and capability reporting.

<Way forward>:

For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19:

Support case 1 and case 2 via RRC reconfiguration.

- Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated.

- Case 2: 1UL/2DL configuration

Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19.

Whether new capability is needed or not would be further discussed.

Discussion

Regarding RRM measurement, when the n5 DL remains active but no resources are allocated, the UE must still continuously measure the carrier quality (RSRP/RSRQ). If the network completely refrains from allocating CSI-RS resources for this carrier, the UE's measurement results may drift. Nevertheless, existing protocols already allow the BS to configure measurement gaps to coordinate measurement behavior, which should mitigate this risk. E.g., as specified in 38.133 clause 9.5.1, When configured by the network, the UE shall be able to perform L1-RSRP measurements of configured CSI-RS, SSB or CSI-RS and SSB resources for L1-RSRP. The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-RSRP measurements within the active BWP. One of the measure period for L1-RSRP measurements could be the measurement period of TL1-RSRP_Measurement_Period which is defined as follows. It could be found under non-DRX mode, the period is max(TReport, ceil(M*P)*TSSB) which is larger than or equal to TSSB. Thus, the allowed time for RRC reconfiguration to remove n8 UL and schedule n5 DL is large enough to be feasible.

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise, K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Obervation 1: Existing RRM specification could support uplink CA_n5-n8 with non-concurrent n5 DL and n8 UL via RRC reconfiguration.

Regarding capability reporting: Case 1 requires the BS to avoid allocating n5 DL resources to specific UE, meanwhile, case 2 requires the BS remove UL configuration of n8 UL to the specific UE via RRC. Although BS know the capability support when UE reporting UL CA_n5-n8, it is still beneficial to explicitly report this capability in case some UEs have other implementation for supporting UL CA_n5-n8 in future.

Proposal 1: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Conclusion

In this contribution, we give our observation and proposal on left issues on RRM impact and capability reporting for CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Obervation 1: Existing RRM specification could support uplink CA_n5-n8 with non-concurrent n5 DL and n8 UL via RRC reconfiguration.

Proposal 1: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

- References

- R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei