3GPP TSG-RAN WG4 Meeting # 116		                                             R4-2510547

Bengaluru, India, 25 – 29 August 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	7.14.3.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during the last meeting. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

In the last meeting, the companies agreed to introduce Solution 1 and Solution 3. For these two solutions, there are still some parts that need to be discussed. There are two points we would like to provide our views which are whether solution 1 is possible to extend to the FR1 and FFS of solution 3.

Solution 1:

At the last meeting, the companies discussed several issues related to Solution 1. The companies had different views on the details of Solution 1 and how to expand the solution to FR1. We will provide our views in the following parts.

RAN 2 LS discussion

In the #114 meeting, RAN4 sent LS to RAN2 on the signalling design for solution 1. Now, the RAN2 discussion on the network indication of a specific SCC per-band for measurement, the second layer network indication has two alternatives that need RAN4 to do selection and determination. The content shown following is captured from the RAN2 LS (R4-2509014).

About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting.

In terms of the information that RAN2 delivered to RAN4, the discussion of this should focus on the feasibility of Alternative 2 mentioned in RAN2 LS.

Proposal 1: RAN4 to discuss the feasibility of Alternative 2 mentioned in RAN2 LS.

If no new RRC signalling that introduced in Solution 1, the logic of one CC per band selection is complete and reasonable when there are multiple CCs configured in one band, when the first network indication is enabled. The second network-based SCC selection in Solution 1 is aimed at letting Network determine the specific SCC per band for measurement when neither PCC nor PSCC is in the same band. In our view, the new RRC signalling does not offer a significant gain in configuring delay compared with the RRC reconfiguration.

On the other hand, it is a network implementation approach where the network is able to configure one MO per band at a time, or the network reconfigures one specific SCC in one band after configuring multiple SCCs in this band through RRC reconfiguration. Therefore, with no new RRC signalling, Solution 1 can work well. Therefore, we propose Alternative 2 from the RAN2 LS as the preferred solution for implementing Solution 1 SCC selection..

Proposal 2: Alternative 2 from the RAN2 LS is able to be approved as the preferred solution for implementing Solution 1 SCC selection.

Clarify the network indication in solution 1

Companies had conflicting views on the second network indication in the last meeting, that shown as follows:

Issue 1-1-2: Clarify the network indication solution in solution 1 - Option 1 (OPPO): RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update. - Option 2 (Apple, CATT): network indication to enable/disable this CSSF enhancement of solution 1 can be semi-static while network indication of a specific SCC per-band for measurement can be dynamic. - (CATT, Apple)Network indication of a specific SCC per-band for measurement is dynamic indication based on MAC CE.

As mentioned above, the second network, indicating SCC selection, it is possible to be designed as a dynamic indication, but the conclusion from RAN2 LS precluded the dynamic signalling approach. In terms of RAN2 LS, the second network discussion will follow the RAN2 LS recommended approach.

Observation 1: The dynamic network indication has been precluded in RAN2. RAN4 discussion will focus on the feasibility of the RAN2 LS recommended approach.

Revision on Solution 1

The discussion of the last meeting has been captured below from the last meeting’s WF[1].

Issue 1-2-1: revision on solution 1 - Proposal: For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

In our view, we are open to continuing the discussion on the description of Solution 1. This may relate to the network indication format. For example, if the network indication can reconfigure the MOs, the network can configure the SSB-based report on the intended MO. Even when the network configures CSI-RS on multiple MOs without any SSB-based reports, the network will still select one MO for the UE to measure and report on. Therefore, this revised Solution 1 should be updated after the SCC selection network indication discussion is finished.

If the discussion on RAN2 LS has a consensus on alternative 2, RAN4 is need to revise the details of Solution 1 as:

- If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are configured for measurement in the same band and UE is allowed to determine the CC in the band to be measured based on followings,

  - Network indication of enable/disable this CSSF enhancement is per-UE.

  - Follow the following principle for serving and neighboring cell measurement

    - Measure PCC if PCC in band

    - Measure PSCC if PSCC in band

    - Measure SCC on which UE is configured to report SSB based measurements, if

      - there is only one SCC on which UE is configured to report SSB based measurements on same FR2 band and no PCC/PSCC in band

  - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation.

- Otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

Note: Following the WID, the above agreement applies for the measurement without gap.

Observation 2: If the discussion on RAN2 LS has a consensus on 2 alternatives, RAN4 is need to revise the details of Solution 1, whether or not to update the conclusion.

Extend solution 1 to FR1

The following content has been extracted from the last meeting’s WF[1].

Issue 1-1-3: Expand solution 1 to FR1 case [Wayforward]: Further discuss in next meeting: If extend for FR1: Aspect 1: co-located or non-collocated - Option 1: only limit to serving cell measurement (E///) - Option 2: cover both serving cell and neighboring cell measurements - In clause 7.6.4 of TS 38.133, it clearly sys only co-located deployment is applied for intra-band contiguous CA. - Companies are encouraged to check the spec in this meeting. Aspect 2: Network configuration on enable/disable the feature - Option 1: the configuration is the same for different FRs - Option 2: the configuration can be different for different FRs (Nokia, ZTE, China Telecom, Apple, HW, E///) - Under the condition that the UE capability is per FR [or per band]. - Option 2 will be adopted if the solution 1 will be extended to FR1.

Solution 1 enables the network to allow UE to measure a single serving CC per band, even when multiple serving CCs’ MOs are configured. This solution is based on the assumption that FR2 CCs experience minimal interference due to the use of directional beams for signal transmission and reception. However, in FR1, signal transmission and reception typically rely on omnidirectional antennas, which are more susceptible to interference. This susceptibility, combined with the fact that intra-band cells are not completely co-located, means that cells within the same band may experience varying levels of interference. Consequently, a single CC cannot accurately represent the measurement results for the entire band in FR1. In our view, the network configuration for enable/disable is supposed to be different for different FRs to ensure that this enhancement can work well.

Proposal 3: The network configuration for enable/disable can be separate for FR1 and FR2.

Solution 3:

In the last meeting, the 3-searcher solution CR has been endorsed; some remaining issues need to be discussed.

Issue of inter-frequency and inter-RAT measurement in SA mode

In SA mode, there are some MOs with no measurement gap, inter-frequency and inter-RAT. These two types of MO are introduced for gap enhancement in Rel-18. The conclusion in Rel-18 shows that the MOs without gap and NR MOs cannot able to be measured in parallel. In the current 3-searcher solution, for certain circumstances, the MO without gap and NR MO will be treated simultaneously, but the Rel-19 UE does not have this capability. Here is the conclusion in Rel-18 measurement gap discussion:

Issue 2-3-1: searcher limitation - Agreements - For Case b-2, performing inter-RAT measurement and NR measurements in parallel without searcher limitation is NOT supported in Rel-18. - FFS whether to apply the same limitation to case b-1.

This agreement indicates that the inter-RAT without measurement gap MO is following the 2-searcher processing logic and can not do this without the searcher limitation. This means the UE can only process one PCC and one inter-RAT MO simultaneously in Rel-18. So the 3-searcher solution also needs to follow this limitation. So this issue also happened in inter-frequency without gap measurement. In our view, the inter-frequency and inter-RAT without gap MO need to be treated in the same way. Therefore, in the EN-DC with FR1 only CA and FR1 only CA scenarios, the UE is only able to process one MO of inter-frequency or inter-RAT at a time. Solution 3 should be updated as follows: all SCC MOs can share two searchers, and without gap MOs need to share one searcher with part of SCC MOs.

Proposal 4: In the EN-DC with FR1 only CA and FR1 only CA scenarios, the UE is only able to process one MO of inter-frequency or inter-RAT at a time. Solution 3 should be updated as follows for the above cases: all SCC MOs are able to share two searchers, and without gap MOs need to share one searcher with part of SCC MOs.

Conclusions

Proposal 1: RAN4 to discuss the feasibility of Alternative 2 mentioned in RAN2 LS.

Proposal 2: Alternative 2 from the RAN2 LS is able to be approved as the preferred solution for implementing Solution 1 SCC selection.

Observation 1: The dynamic network indication has been precluded in RAN2. RAN4 discussion will focus on the feasibility of the RAN2 LS recommended approach.

Observation 2: If the discussion on RAN2 LS has a consensus on 2 alternatives, RAN4 is need to revise the details of Solution 1, whether or not to update the conclusion.

Proposal 3: The network configuration for enable/disable can be separate for FR1 and FR2.

Proposal 4: In the EN-DC with FR1 only CA and FR1 only CA scenarios, the UE is only able to process one MO of inter-frequency or inter-RAT at a time. Solution 3 should be updated as follows for the above cases: all SCC MOs are able to share two searchers, and without gap MOs need to share one searcher with part of SCC MOs.

References

- R4-2508264 WF on RRM requirements for NR_RRM_Ph5_Part1