3GPP TSG-RAN WG4 Meeting # 116	 R4-2510552

Bengaluru, India, 25 - 29 August 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM core requirements for Rel-19 ATG

Agenda Item:	7.8.5

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-cell, we provide our analysis on the remaining ATG core part issues that impact to support of CA in a case-by-case manner in the following part.

Sharing factor between L1 measurement and RRM measurement

In the last meeting, companies proposed to discuss the SCell addition/activation mechanism.

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration - Proposals - Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration - Proposal 1-1: - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g., - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement - Proposal 1-2: - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured], - Klayer1_measurement=1, - - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or; - - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or; - Proposal 2: - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement. - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells. - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin - Proposal 3: - reuse the legacy specification, i.e. - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

When a collision occurs during the L1 and L3 measurements in the inter-frequency measurements without measurement gap, the Klayer1_measurement scaling factor is used to deal with the collision. The remaining issue on this factor for Rel-19 ATG is whether 1 symbol tolerance for L3 measurement is sufficient for ATG with 300km ISD and approximately 1200 km per hour speed. The conclusion from our side is that 1 symbol before and after the L3 measurement ossician is adequate. Here we would like to provide our analysis.

Firstly, the reason for introducing this 1 symbol tolerance is when UE is at the edge of the serving cell, UE needs to do the measurement for the neighbour cell. Due to the propagation delay difference between the delay from the serving cell to UE and the delay from the neighbour cell to UE (Figure 1 shows that the L1 and L2 are assuming links for serving cell and neighbour cell when UE at the cell edge to do the neighbour cell measurement, the link L1 and L2 have a propagation delay difference), the L3 measurement occasion will have a shift which means UE can not accurately determine the best time to measure neighboring. To ensure that this time shift does not result in partial overlapping of the L1 and L3 measurement ossicians, a 1-symbol duration is needed before and after the L3 measurement ossician. And the 1-symbol duration is sufficient to allow the UE to complete the neighbour cell measurement.

Figure 1. Propagation delay difference

After that, we would like to provide the analysis on the ATG scenario. There is one critical point we want to make here: 1-symol tolerance is associated with the propagation delay difference instead of the ISD.

Observation 1: 1-symol tolerance design in Klayer1_measurement=1 is associated with the propagation delay difference instead of the ISD.

The 1-symbol tolerance allows the UE to do the neighbour cell L3 measurement in a location that is within 4.5 km from the edge of the cell, as shown in Figure 2. Assuming the ATG with a 1200 km per hour speed begins with neighbour cell measurement at the edge of both the serving cell and the neighbour cell. There are 13.5s for ATG to do the neighbour measurement. Obviously, this is sufficient to finish the neighbour cell measurement for ATG.

Figure 2. Propagation delay difference of the ATG scenario

Therefore, the legacy requirement for Klayer1_measurement = 1 is able to be reused for Rel-19 ATG support antennaArrayType-r18 in FR1.

Proposal 1: The legacy requirement for Klayer1_measurement = 1 is able to be reused for Rel-19 ATG supports antennaArrayType-r18 and does not support [case 4 capability] in FR1.

For ATG UE supports antennaArrayType-r18 and supports common Rx beam, the requirement of Klayer1_measurement = 1 needs an update. The updated version shows as follows:

------------------------------------------------------------Text proposal-------------------------------------------------------------------

Klayer1_measurement=1,

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

------------------------------------------------------------Text proposal-------------------------------------------------------------------

Proposal 2: The legacy requirement for Klayer1_measurement = 1 for Rel-19 ATG supports antennaArrayType-r18 and [case 4 capability] in FR1 is needed to be update as:

------------------------------------------------------------Text proposal-------------------------------------------------------------------

Klayer1_measurement=1,

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

------------------------------------------------------------Text proposal-------------------------------------------------------------------

Conclusion

Observation 1: 1-symol tolerance design in Klayer1_measurement=1 is associated with the propagation delay difference instead of the ISD.

Proposal 1: The legacy requirement for Klayer1_measurement = 1 is able to be reused for Rel-19 ATG supports antennaArrayType-r18 and does not support [case 4 capability] in FR1.

Proposal 2: The legacy requirement for Klayer1_measurement = 1 for Rel-19 ATG supports antennaArrayType-r18 and [case 4 capability] in FR1 is needed to be update as:

------------------------------------------------------------Text proposal------------------------------------------------------------------

Klayer1_measurement=1,

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on serving frequency of PCC or SCC that [UE support the case 4 capability] outside measurement gap and fully-overlapped by intra-frequency SMTC occasions of PCC or SCC are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement;

------------------------------------------------------------Text proposal------------------------------------------------------------------

References

[1] R4-2508263 WF on RRM requirements for NR_ATG_enh