3GPP TSG-RAN WG4 Meeting # 116	R4-2510573

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.25.3.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on CSI-RS based L1 measurement on neighbor cells.

Discussion

Measurement procedure

As per RAN4#114bis agreement, the baseline of CSI-RS based L1 measurement on neighbour cell is 3-step.

- Step 1 (gNB TX wide SSB beam, UE RX wide SSB beam): UE performs SSB based L3 measurement and acquires the SSB index information. UE reports the L3 SSB measurement results with SSB index to network.

- Step 2 (gNB TX wide SSB beam, UE RX fine SSB beam): UE performs SSB based L1-RSRP measurement on the candidate cell to refine the SSB based RX beam. Step 2 is what we specified for R18 LTM where UE sweeps RX beam based on SSB.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE performs measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping. Step 3 is similar as R15 CSI-RS based L1-RSRP measurement, the difference is step3 is on candidate cell and R15 CSI-RS based L1-RSRP measurement is on serving cell.

As a compromise, one optional UE capability is agreed to introduced if UE is able to skip step 2 for CSI-RS L1 candidate measurement in FR2. However whether step 2 can be skipped for serving cell is still under discussion. The case is that the candidate cell which is configured for LTM CSI-RS measurement is a serving cell, e.g., SCell or PSCell.

- Issue 3-2: whether step 2 is skipped for serving cell? Candidate options: - Option A: RAN4 requirements regarding ‘skipping step 2’ do not apply for the case CSI-RS based L1 measurement for LTM candidate cell is configured in legacy CSI-RS based L1 measurement in serving cell in FR2. (QC, HW) - Option A1: Requirement regarding ‘skipping step 2’ is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to ‘off’, is one of the FR2 serving cells. (QC, OPPO) - Option A2: The CSI-RS based L1-RSRP measurement requirement is not applicable if an FR2 candidate cell, from which CSI-RS is configured with repetition set to 'off', is one of the FR2 serving cells (QC). - Option B: keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM. (MTK, Apple, E///)

If step 2 is removed, the CSI-RS based L1 measurement procedure is: CSI-RS resources within a resource set are periodically transmitted with different downlink spatial domain transmission filter, and UE sweeps Rx beam with finer CSI-RS level beam. The schematic diagram is shown in Figure 1.

Figure 1. CSI-RS L1 measurement on neighbour cell with skipping step2

From UE implementation perspective, UE behaviour for LTM step-2 skipped CSI-RS L1 measurement as a candidate cell and legacy CSI-RS L1 measurement as an serving cell are various. To reduce implementation complexity, UE would perform one kind of behaviour for the same cell. In our understanding, the above four options in WF are similar, which are aiming to ensure the step 2 skipping requirements are not applicable for the candidate cell which is serving cell. To simply the discussion, we are fine to use the description of option B.

Proposal 1: Keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

- Issue 3-3: Sharing factor for resource collision between serving and neighbour cell Candidate options: - For FR2 CSI-RS L1 measurement requirement, - if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain,. - Option 1: sharing factor shall be introduced. (HW, QC, E///) - Option 2: introduce measurement restriction as legacy. (MTK, vivo)

Regarding the case that the CSI-RS resources for L1 measurement on serving cell and the CSI-RS resources for L1 measurement on neighbour cell are colliding in time domain, there are two different directions to define requirements.

- One scheme is to introduce a scaling factor. This scheme has already been used in SSB-based L1-RSRP measurement with different PCI, where scaling factor P1/P2 is introduced to additionally consider the overlapping between SSB for L1 measurement on serving cell and SSB for L1 measurement on the cell with different PCI. The advantage of the scheme is that there are specific measurement latency requirements when CSI-RS resources are collided. The drawback is that it is quite complicated to derive the scaling factor.

- The other scheme is using “measurement restriction”. In FR2 when the CSI-RS for L1-RSRP measurement on serving cell is overlapped with another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement on neighbor cell, when the two CSI-RS are not QCL-ed, UE is required to measure one of but not both CSI-RS resources. The advantage of the scheme is its simplicity. The drawback is that there are no specific requirements for CSI-RS L1 measurement delay.

Technically, we are open to both above schemes. As this is the last meeting for core requirements, it seems the measurement restriction way is proper at current stage.

Proposal 2: Due to limited time for complete core requirements, it is fine to define measurement restriction when CSI-RS resources for L1 measurement on serving cell and neighbour cell are colliding in time domain.

Restriction on CSI-RS resource configuration

In RAN4#114 meeting, it is agreed to have at most two separate windows for CSI-RS resources for L1 measurement on candidate cells.

- In FR1, when CSI-RS based L1 measurement does not cause scheduling restriction, no need to consider restriction on measurement timing configuration. - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction: - All CSI-RS resources on [one intra-frequency layer] are configured within up to two separate windows where each window is up to [5 ms]

In RAN4#115 meeting, two further proposals were provided.

- Issue 3-6: Measurement restriction Candidate options: - Option 1: Apple, ZTE, Ericsson - In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5ms. - Option 2: vivo - When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms.

Both option 1 and option 2 are only talking about the spacing of windows in time domain. Allowing one window to emerge per 20ms can provide the network with CSI-RS configuration flexibility. However from UE side, UE needs to monitor more CSI-RS resource positions. In addition, the locations of the two windows are not discussed so far.

Recalling R16 CSI-RS based L3 measurement, the period of all CSI-RS resources of intra-MO are the same. The candidate periodicity value is {4ms, 5ms, 10ms, 20ms, 40ms}. When the periodicity of the configured CSI-RS resource is 20 ms or 40 ms, up to two separate windows can be configured. The locations of the two windows are specified as below:

- The starting point of the first window is the slot boundary of the serving cell, where the corresponding slot contains the configured L3 CSI-RS resource of the serving cell in the servingCellMO with the smallest offset, and - The starting point of the second window if configured is determined by an offset of half of the CSI-RS periodicity in slots with regards to the starting point of the first window

For R19 CSI-RS based L1-RSRP measurement on neighbor cell, the periodicity and offset of CSI-RS resources from different cells are different. Moreover the candidate periodicity value can be up to 640ms. Therefore the legacy CSI-RS resource window(s) definition for L3 measurement can not be directly reused.

First, to accommodate legacy UE processing capability specified for CSI-RS based L3 measurement, the principle that each window is separated by 20ms shall be followed. Secondly, as the periodicity of CSI-RS resources from different cells is different, it can be specified that there are at most two separate windows within every P where P is min{CSI-RS periodicity of cells on one intra-frequency layer} when P is larger than 20 ms, otherwise one window is needed.

The starting point of the first window can still follow the slot boundary of serving cell where the corresponding slot contains the configured L3 CSI-RS resource of the serving cell as legacy. However one small modification is to be pointed out. For example, when the period of CSI-RS resources in serving cell is 80ms and offset is 50ms, and P is 40ms, then the starting point of the first window is 10ms rather than the absolute offset of serving cell (50ms). More general, the starting point of the first window is supposed to be [(the slot boundary of serving cell) mod P], where the corresponding slot contains the configured L1 CSI-RS resource of the serving cell.

The starting point of the second window if configured is determined by an offset of half of P in slots with regards to the starting point of the first window.

Proposal 3: There are at most two separate windows within every P where P is min{CSI-RS periodicity of cells on one intra-frequency layer} when the periodicity of the configured CSI-RS resources is larger than 20 ms.

- the starting point of the first window is [( the slot boundary of serving cell) mod P], where the corresponding slot contains the configured L1 CSI-RS resource of the serving cell.

- The starting point of the second window if configured is determined by an offset of half of P in slots with regards to the starting point of the first window.

Early CSI acquisition on candidate cell

- Issue 3-9: early CSI acquisition Candidate options: - Option 1: Define timeline for baseline early CSI acquisition. (vivo, Nokia, E///) - Option 2: Define scheduling/measurement restriction requirements for optional early CSI acquisition. (QC) - Option 3: not define RAN4 requirements for early CSI acquisition. (MTK, HW, Apple)

Early CSI acquisition is supported in RAN1.

Agreement in RAN1#120 meeting For target cell CSI acquisition, - A UE is provided with RRC configurations for periodic CSI-RS resource(s) and CSI report(s) for one or more candidate cell - For a candidate cell, - down-select from the following alternatives: - Alt 1: A single CSI report configuration is configured - Alt 2: Multiple CSI report configurations can be configured - down-select from the following alternatives: - Alt X: A single CSI-RS resource for CMR is associated with a CSI report configuration - Alt Y: Multiple CSI-RS resources for CMR can be associated with a CSI report configuration - FFS: Semi-persistent CSI-RS resource - After the RRC configuration and before the reception of CSC, the UE may measure CSI based on the configured CSI-RS resource(s), which is subject to UE capability - FFS: whether or how to select a subset of CSI-RS resources to measure - FFS: when the UE may start measuring the configured CSI-RS resources - UE determines the CSI report configuration based on the CSC - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration - The latest available measured CSI on target cell resource(s) is conveyed at least by a single report, and the report is sent to the target cell - Option 1: to use UCI - Option 2: to use MAC CE Note: with this agreement, the working assumption made in RAN1#119 is automatically confirmed.

Working Assumption in RAN1#119 meeting As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE. - The report is sent directly to target cell. - Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE. - Other than UE capability, there is no additional spec impact compared to the baseline (only one triggering mechanism will be specified).

As per RAN1 agreement, the baseline CSI reporting is to be performed after LTM CSC MAC CE. At this moment, the connection of serving cell is broken and the SW processing unit/resource for serving cell is released. When UE receives LTM CSC MAC CE, and as long as the T/F tracking of target cell is acquired, CSI measurement can be performed. As per the latest RAN1 agreement, if valid CSI is not available, code point of lowest CSI index is reported in PUSCH. The PUSCH is separately defined for RACH-less LTM and RACH based LTM procedure. It means that whether early CSI acquisition is completed before the first UL transmission has no impact on the LTM cell switch delay.

Proposal 4: Baseline early CSI acquisition would not impact LTM cell switch delay.

Then the next issue is whether to define requirements of baseline early CSI acquisition on a candidate cell. We understand the duration includes CSI resource acquisition, UE processing for CSI, and waiting for available UL resource for reporting. However it is similar as legacy CSI measurement procedure except the starting point which is the instance UE receives CSC. In legacy, no RRM requirements are defined for CSI measurement. Following the legacy way, there is no RRM requirement for CQI acquisition on candidate cells.

Proposal 5: RAN4 not to define requirements for baseline early CSI acquisition on candidate cells.

The optional capability is that CSI measurement can start before reception of LTM CSC MAC CE. To some extent, the behaviour of CSI acquisition on neighbour cell is alike that of CSI-RS based L1-RSRP measurement on candidate cell. However, there are also some differences, e.g., CSI measurement can be semi-persistent and triggered by MAC CE, whether or how to select a subset of CSI-RS resources to measure. Moreover if there is no requirement for baseline early CSI acquisition, it is not proper to specify requirements of optional CSI acquisition. What’s more, considering limited time slots for completing the core requirement of the WI, we suggest not to define requirements for optional early CSI acquisition on candidate cells either.

Proposal 6: RAN4 not to define requirements for optional early CSI acquisition on candidate cells.

Conclusion

The contribution provides analysis on CSI-RS based L1 measurement. The following proposals are proposed:

Proposal 1: Keep the current measurement procedure unchanged for serving cell when it is in the candidate cell for LTM.

Proposal 2: Due to limited time for complete core requirements, it is fine to define measurement restriction when CSI-RS resources for L1 measurement on serving cell and neighbour cell are colliding in time domain.

Proposal 3: There are at most two separate windows within every P where P is min{CSI-RS periodicity of cells on one intra-frequency layer} when the periodicity of the configured CSI-RS resources is larger than 20 ms.

- the starting point of the first window is [( the slot boundary of serving cell) mod P], where the corresponding slot contains the configured L1 CSI-RS resource of the serving cell.

- The starting point of the second window if configured is determined by an offset of half of P in slots with regards to the starting point of the first window.

Proposal 4: Baseline early CSI acquisition would not impact LTM cell switch delay.

Proposal 5: RAN4 not to define requirements for baseline early CSI acquisition on candidate cells.

Proposal 6: RAN4 not to define requirements for optional early CSI acquisition on candidate cells.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2504965, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple