3GPP TSG-RAN WG4 Meeting # 116	R4-2510579

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on impacts for measurement event prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.19.3.2

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

In RAN4 measurement prediction was carried out since RAN4#112bis meeting, and measurement event prediction was discussed since RAN4#114bis meeting [2]. This contribution focuses on measurement event prediction for AI/ML mobility.

Issue 1-1-1: Measurement event prediction use case Agreement: - RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

Discussion

In RAN2, there are two methods to predict measurement event, i.e., indirect and direct measurement event prediction. The general functional framework for indirect and direct event prediction are illustrated in below figures (duplicated from TR38.744).

In indirect event prediction, measurement prediction AI/ML model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing. In direct event prediction, measurement event is predicted directly by an AI/ML model, no measurement prediction model is involved.

Figure 4.3-1: Indirect measurement event prediction [TR38.744]

Figure 4.3-2: Direct measurement event prediction [TR38.744]

In RAN4# 114bis meeting, the potential impact on RRM core requirements were discussed [3]. According to RAN2 discussion, the handover procedure based on event prediction for temporal domain prediction is optimized with three options.

As per the latest description in TR38.744 [4], option1, 2 and option 3 are with regard to how to decide on the time point to transmit handover command. We understand option1/2/3 can reduce handover failure number. From handover delay requirements perspective, in legacy the starting point of handover delay is from the instance UE received the handover command. In AI event prediction model, although the time point of HO preparation and the time instance of handover command indication are different from legacy, the UE subsequent behaviour (e.g. AGC, fine time tracking, RACH) after receiving HO command still follows legacy way in option 1, 2 or 3.

Cited from TR38.744 Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier. If measurement event is predicted based on intra-frequency temporal domain case B, there is option 3 w.r.t. how to decide on the time point to transmit handover command: Figure 5.5.1-3: Handover model option 3 Option 3 is illustrated in Figure 5.5.1-3. Once a predicted measurement event e.g. A3 event is received network can transmit handover command immediately after handover preparation is completed. UE will report predicted measurement event at the time instance it is to be triggered.

Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay.

In RAN4#114bis meeting, the event prediction accuracy was discussed.

Issue 3-1-2: Performance metric for event prediction use case Agreement: - Potential RRM requirement impact for event prediction use case - for indirect case: - RAN4 to study the requirement for the predicted event triggered reporting - RAN4 to study the requirement for [delay and accuracy] - RAN4 to discuss the metric for the event prediction - Additionally, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], - Candidate options: - Option 1: RAN4 will define requirement for the predicted RSRPs - Option 2: RAN4 will not define requirement for the predicted RSRPs

In RAN2 evaluation, performance metric F1 score is introduced [TR38.744]:

F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3)

In the following, we provide analyses on indirect event prediction and direct event prediction respectively.

Indirect event prediction

For indirect event prediction, the counter n1, n2 and n3 in the formula are defined as following [TR38.744]:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]

where ETD is the timing difference between the instance ground-truth event occurs and the instance predicted event occurs.

Figure 5.3.1-1: illustration of ETD [TR38.744]

From defining requirements perspective, there are some options.

Option 1: Define accuracy for indirect event prediction

Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

Option 2: Not define accuracy requirement for indirect event prediction

In indirect event prediction, measurement prediction model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing without further involvement of an AI/ML model. In a sense, as long as the measurement prediction accuracy is able to be satisfied, the event prediction accuracy can be guaranteed as well. In our understanding, if the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the measurement prediction accuracy. This intermediate predicted RSRP is what we are discussing in measurement prediction topic.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

- Option 1: Define accuracy for indirect event prediction

- Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

- Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

- Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

Direct event prediction

In direct event prediction, the input of the model with direct prediction is the same as indirect prediction, and measurement event is predicted directly by an AI/ML model, and no measurement prediction AI model is involved. The output is a likelihood of an event prediction.

For direct event prediction, the counter n1, n2 and n3 in the formula are defined as following [TR38.744]:

- Counter n3 (true event prediction): it increases by 1 when a ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

- Counter n1 (false event detection): it increases by 1 when no ground-truth event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold

- Counter n2 (missed event detection): it increases by 1 when a ground-truth event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold

For direct event prediction method, a measurement event could be predicted within an occurrence window starting from current time instance i.e. t0 and future time instance t1 with a probability X% [TR38.744].

Figure 5.3.1-2: occurrence window of direct prediction method [TR38.744]

We think the accuracy of direct event prediction can be defined in below options:

Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value.

Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within a time duration [t0, t1].

Proposal 3: RAN4 to discuss the following options for defining accuracy for direct event prediction:

- Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value.

- Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

Predicted event triggered reporting delay

In legacy, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface, which is 2 x TTIDCCH. The legacy measurement reporting delay is defined at the measurement reference point where the actual channel meets event.

In legacy, the event triggered measurement reporting delay is defined as cell identification duration (T identify), where UE really realizes that the channel changes and the event is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_with_index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

In R19 indirect event predication, at current time (i.e. t0) measurement event is predicted at some point of time in future (i.e. t1), and UE will report the event prediction @t0 to network. The legacy measurement reporting delay can be applied for the predicated event reporting, however it is meaningless as the requirement is not able to represent UE event prediction capability/performance. In addition, the legacy event triggered measurement reporting is not applicable for event predication delay anymore, since the event prediction delay has nothing with Tidentify_intra.

Regarding indirect event prediction, we think the event prediction delay can be specified. The event prediction delay is defined as the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event. The event prediction delay can represent the UE capability of event prediction. The requirements of event prediction delay and event prediction accuracy can guarantee event prediction performance.

Proposal 4: RAN4 to study the requirement for event prediction delay, where event prediction delay is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event.

Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

- Option 1: Define accuracy for indirect event prediction

- Option 1-1: the accuracy of event prediction is defined as ETD, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event, under the condition that precision and recall or F1 score consistently is above a value.

- Option 1-2: the accuracy of event prediction is defined as precision and recall or F1 score under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

- Option 2: Not define accuracy requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied.

Proposal 3: RAN4 to discuss the following options for defining accuracy for direct event prediction:

- Option 1: the accuracy of event prediction is defined as event prediction probability X% within an occurrence window [t0, t1], under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within an occurrence window [t0, t1] with X% probability, under the condition that precision and recall or F1 score is consistently above a value.

- Option 2: another accuracy definition is defined as precision and recall or F1 score under the conditions of event prediction probability X% within an occurrence window [t0, t1]. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently is above a value (e.g., 0.9), under the condition of X% event prediction probability within an occurrence window [t0, t1].

Proposal 4: RAN4 to study the requirement for event prediction delay, where event prediction delay is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event.

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2502594, WF on AI/ML Mobility, Nokia

- R4-2504901, WF on AI/ML Mobility, Nokia

- TR38.744