3GPP TSG-RAN WG4 Meeting #116	R4-2510591

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor was further discussed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on remaining issues.

# Discussion

In last RAN4 meeting, RAN4 discussed whether to introduce the conditions that allows UE to quit L3 measurement delay reduction by optimizing RX BSF, and the agreements are summarized as follows:

### Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

As it is agreed to introduced the quit conditions, it is necessary to also introduce the conditions that UE shall re-enter the fast beam sweeping L3 measurement mode; otherwise, it will become a one-shot fast measurement. The recommended WF is a reasonable approach to require UE to re-enter fast beam sweeping L3 measurement after another timer expires.

Proposal 1: RAN4 to define the conditions that UE shall re-enter the Multi-Rx mode after UE quitting as:

After UE quits the Multi-Rx based L3 measurement, UE shall re-activate Multi-Rx based L3 measurement when the FBS triggering conditions is met (i.e. serving Cell quality is worse than the threshold) AND the time span since UE quits last Multi-Rx based L3 measurement is longer than T2.

Based on the analysis above, the following question is how to define the value of T1 and T2. From our understanding, there is no needs to introduce two dedicated timers in RAN2 spec for this purpose. They be predefined in RAN4 spec as constant values. For T1, the motivation is to guarantee that UE has performed L3 measurement long enough. The value of T1 and T2 are proposed as follows. For T1, it means UE has at least perform fast measurement for 5 times.

- T1: 5*Tidentify

- T2: 5 seconds

Proposal 2: The value of T1 and T2 are predefined in RAN4 spec as: T1 is equal to 5*Tidentify; T2 is equal to 5 seconds.

Another remaining issue is about the UE feature. The status in last meeting is summarized as follows:

Agreement: For FBS capability design: - UE capability is per-band and optional - TDD only - FR2-1 only - FFS: in the same FG, whether to define separate UE capability for (intra-frequency) and (intra-frequency + inter-frequency)

As explained in last meeting, supporting these features requires additional UE capability for both RF and baseband resources. And the capacity of RF/baseband resource varies for different band combinations. The applicable scenario is single FR2-1 PCell with one carrier in FR2-1 for L3 measurement. There are following cases to be considered:

- Case 1: Intra-band infra-frequency: PCell and target carrier for L3 measurement are in the same band and intra-frequency.

- Case 2: Intra-band inter-frequency: PCell and target carrier for L3 measurement are in the same band and inter-frequency.

- Case 3: Inter-band: PCell and target carrier for L3 measurement are in different bands and inter-frequency.

From our understanding, case 1 is the most typical case as it is more likely that UE is configured with one carrier for L3 measurement for one band. Case 2 is less realistic than case 1 in the real deployment. As for case 3, it is also not the typical cases that gNB trigger the fast measurement on one band based on the quality of another band as the coverage of these two bands could be different. And from UE implementation perspective, enabling the feature for inter-frequency measurement will require more baseband resources.

The comments received from last meeting are mainly for following aspects:

- Whether to further differentiate inter-frequency with gap and inter-frequency without gap: Indeed, for inter-frequency with MG, the measurement for inter-frequency carrier and intra-frequency carrier are not simultaneously. But from scenario perspective, no matter whether it is with gap or without gap, it is not worth to implement it since it is not typical case in the realistic deployment but just a possible case.

- Whether to differentiate intra-band and inter-band: For inter-band L3 measurement: it has to be inter-frequency measurement. Since it is already agreed that the feature is a per band UE capability, which is under a certain BC if UE supports inter-band. If UE support this feature for both intra-frequency and inter-frequency, it means UE support both intra-f and inter-f on this target band no matter intra-band or inter-band. Similarly, if UE supports only intra-frequency, it means UE supports only intra-frequency no matter intra-band or inter-band.

Based on the analysis above, the proposed the UE feature list is as attached, which is quite similar as the stable version at the end of last meeting.

Proposal 3: RAN4 to introduce UE feature for FR2-1 L3 measurement delay reduction based on reduced beam sweeping factor as the table in Annex.

# Conclusions

Proposal 1: RAN4 to define the conditions that UE shall re-enter the Multi-Rx mode after UE quitting as:

After UE quits the Multi-Rx based L3 measurement, UE shall re-activate Multi-Rx based L3 measurement when the FBS triggering conditions is met (i.e. serving Cell quality is worse than the threshold) AND the time span since UE quits last Multi-Rx based L3 measurement is longer than T2.

Proposal 2: The value of T1 and T2 are predefined in RAN4 spec as: T1 is equal to 5*Tidentify; T2 is equal to 5 seconds.

Proposal 3: RAN4 to introduce UE feature for FR2-1 L3 measurement delay reduction based on reduced beam sweeping factor as the table in Annex.

# References

[1] R4-2504900 WF on RRM requirements for NR_RRM_Ph5_Part2

[2] R4-2504962 LS on Rx BSF optimization for NR RRM Ph5

[3] R4-2415656 Topic summary for [112bis][209] NR_RRM_Ph5_Part1

[4] R4-2414026 WF for NR_RRM_Ph5_Part1

# Annex

# NR_RRM_Ph5

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-x | FR2-1 L3 measurement delay reduction based on reduced beam sweeping factor | 1. The reduced beam sweeping factor that UE supports for FR2-1 L3 measurement delay reduction on a carrier-to-be-measured on a specific band. : |  | Yes |  | UE does not support FR2-1 L3 measurement delay reduction based on reduced beam sweeping factor | Per Band | TDD only | FR2-1 only |  | Candidate values: {2, 4, 6} for FR2-1. Note 1: It is only supported for power class 3. Note 2: UE supporting this FG uses one additional bit to indicate whether UE can apply this FG for FR2-1 L3 SSB-based inter-frequency measurement with or without MG. Note 3: the feature is applicable provided that single FR2-1 serving cell and single carrier are configured. | Optional with capability signaling |