3GPP TSG-RAN WG4 Meeting #116	R4-2510600

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.20.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2]. RAN4 discussed the RRM impacts with agreements and issues captured in [3]. In this contribution, we further provide our views on the remaining issues on UE initiated BM.

# Discussion

Based on the discussion in last meeting, the agreements reached are summarized as follows [3]:

#### Issue 1-1: How to define measurement period in Event triggered measurement reporting delay for the basic feature (M instance = 1, Event-2)? Agreement: - RAN4 define different separate for measurement period for the UE is configured or not for “timeRestrictionForChannelMeasurement” - To check with RAN1 whether the parameter can be configured if RAN1 identify any issue #### Issue 1-2: Components of Event triggered measurement reporting delay? Agreement: - If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams. - Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air. - UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0. #### Issue 1-4: CSI-RS based L1-RSRP for inter-cell BM? Agreement: - Reuse the same requirements structure as that in Rel-17 clause 9.5. No need to add sharing factor for serving cell CSI-RS and NSC CSI-RS. From the configuration of CSI-RS perspective, there is only one configuration. If this CSI-RS is QCLed to the SSB of serving cell or SSB of additionPCI cell, they use the same set of requirements. #### Issue 1-5: Whether to specify new unified TCI switching delay requirements? Agreement: - The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management - Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion. #### Issue 1-7: Requirements for event-1 and event-7? Agreement: - For Event-1 and Even-7, reuse the same design as the requirements defined Event-2

One of the remaining issues is whether to define window-based requirements. RAN4 hasn’t have detailed discussion for this in previous meetings. One reason is that RAN1 is still discussing the detailed design. Considerable progress has been achieved in previous RAN1 meeting, and the relevant agreements are shown as follows:

Agreement RAN1#120bis Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Agreement RAN1#121 On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding the evaluation periodicity for determining event instance, the periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). Above applies for all events.

From our understanding, there is not much RAN4 efforts on considering this. From UE measurement behaviour/requirements, there is no difference compared with no-time window based. The only additional part is how to determine number of event instance. However, the impact on RAN4 is how to define the evaluation behaviour/requirements, which is also common for non-time window based.

Besides, we think time window-based triggering is also important for this feature, different from L3 event triggering with filtering, L1-RSRP could be impacted by instance channel fading, which may lead to wrong event triggering, and it may mislead NW following action (e.g. wrong TCI state switching).

Observation 1: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 1: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering.

Based on RAN1 agreements in last meeting, UE will start the timer once an event-2 instance is obtained. From RRM requirements perspective, when the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter.

However, the sense of introducing window-based event triggering is that UE shall only trigger the event when the evaluation is reliable enough. For instance, if gNB configure the counter as 4, UE shall only trigger the event when 4 even instance has been identified, which means the channel quality change is not due to the instance channel fading, and gNB is confident to trigger TCI state switching accordingly.

Observation 2: The motivation of introducing window-based UEIBR is to have reliable event report to mitigate the impact of instant channel fading (may happens for non-window-based case).

Proposal 2: The requirements shall be defined to verify both following purpose:

- The event is trigger timely after number of event instances have been evaluated.

- The event is not wrongly/too early when the number of event instances have not been evaluated.

Proposal 3: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event no later than TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, and no earlier than TL1-RSRP_event triggered_measurement_Period * (C-1) + Tfirst UL channel, where C is the configured counter.

In other words, if a UE just ignore the configure the counter and window, the UE just triggers the report as long as the event criteria is met for once (same as non-window-based case), it can still meet the requirements and such behaviour cannot be prevented.

For time windowed based case, another issue may need to be discussed is how to count one event instance based on the measurement. Considering that new beam and current beam have same periodicity, similar as no-time-window based case, there is always “pair” of new results from both new beam and current beam. Considering following cases as shown in the figure. For the left part, if there is instance fading happens at point A for current beam, it will only create on event instance, UE will not trigger event to NW, However, if UE only perform measurement on new beam and using history results of current beam, one bad result due to instance change fading will lead to multiple event instance and triggers wrong event to NW work. Another example if as shown in the right part, different comparation will leads to different results of number counting.

Fig.1 Instance counting using different results

Observation 3: Different behaviour on choosing the results for evaluation may results different evaluation results (e.g. different number of event instance counting), and it is not clear for gNB to configure the number of conter.

Proposal 4: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Another issue is whether to consider the case the TCI state is changed during L1 measurement period. The status are summarize as follows:

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? <Way forward > - Option 1: (Nokia, Ericsson, MTK) - No need to capture this scenario in RAN4 requirements - Option 2: (Xiaomi, Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: (Apple) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

From our understanding, it is rather a corner case. The UEIBR is to indicate the changes on channel conditions to help gNB to trigger TCI state switching. If gNB trigger the TCI states, the evaluation behaviour (e.g. whether to restart or not) can be left to UE implementation since the report is not that essential at that time. The only impacts is that the TCI state switching may change the determination of current beam and new beam. If companies really want to avoid such case, it can be capture as the side conditions that the current beam and new beam(s) does not change through the measurement/evaluation/reporting period.

Proposal 5: No need to consider the scenario that TCI state is changed during the L1 measurement time.

Another issue is about whether to consider event triggering and reporting criteria.

#### Issue 1-8: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)? <Way forward > - Option 1: (Samsung, Apple, Huawei, Ericsson) - For event trigger L1 reporting, do not define such capability in parallel per category like L3. - Option 2: (Nokia, MTK) - Define capabilities for support of UE-initiated/event-driven beam reporting on top of existing L3 events in section 9.1.4. For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

From our understanding, the existing requirements for L3 is implicitly restrict the number of measID could be configured to UE. The MO configuration and measID configuration is quiet flexible from gNB perspective. However, for UEIBR, how many events that UE is required to evaluate needs to considering following two aspects: 1) number of events (event 1/2/7); 2) number of new beams. Based on latest RAN1 feature list, the supported event and supported number of new beams can be explicitly indicated by UE capabilities. Thus, there is no needs to further define such capabilities in RAN4 which may further confuse gNB configurations.

Observation 4: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event.

Proposal 6: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria.

Based on proposal 6, consequently there is no needs to have common design among different WI.

# Conclusions

Observation 1: Time window-based Event triggering is important for this feature, which may help mitigate the wrong event triggering cause by instant channel fading.

Proposal 1: RAN4 define requirements for both Time window-based Event triggering and no-time-window-based event triggering

Observation 2: The motivation of introducing window-based UEIBR is to have reliable event report to mitigate the impact of instant channel fading (may happens for non-window-based case).

Proposal 2: The requirements shall be defined to verify both following purpose:

- The event is trigger timely after number of event instances have been evaluated.

- The event is not wrongly/too early when the number of event instances have not been evaluated.

Proposal 3: When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event no later than TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, and no earlier than TL1-RSRP_event triggered_measurement_Period * (C-1) + Tfirst UL channel, where C is the configured counter.

Observation 3: Different behaviour on choosing the results for evaluation may results different evaluation results (e.g. different number of event instance counting), and it is not clear for gNB to configure the number of conter.

Proposal 4: For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

Proposal 5: No need to consider the scenario that TCI state is changed during the L1 measurement time.

Observation 4: Based on current RAN1 feature list, UE can explicitly indicate the supported event (event 1/2/7) and the supported number of new beams for each event.

Proposal 6: Do not define RAN4 capabilities on support of Event Triggering and Reporting Criteria.

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2508271 WF on RRM requirements for NR_MIMO_Ph5_Part1