3GPP TSG-RAN WG4 Meeting #116	R4-2510649

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	7.7.7

Document for:	Discussion

Introduction

RRM requirements for NTN in Ku band are discussed in RAN4#115, and the outcomes are captured in [1]. Based on [1] the following issues need to be further discussed.

- Satellite switch requirements

- Inter-satellite cell reselection requirements

- RLM requirements

In this paper, we will provide our views on RRM requirements for NTN in Ku band.

Discussion

Satellite switch requirements

Issue 1-3: satellite switch requirements Agreement: - Introduce RRM requirement for satellite switching with re-sync for Ku band with VSAT UE for FR1 and FR2 numerologies - Define requirement for hard satellite switch, - Tinterrupt = Tsat_beam + Tsearch + Tprocessing + T∆ + Tmargin ms - Further discuss the details on the definition of each component. - FFS whether to define requirements for soft satellite switch - FFS only switching delay requirement or both switching delay requirement and scheduling restriction requirement - Option for the delay requirement: - Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing ms

In RAN4#115, it is agreed to define RRM requirements for satellite switch for Ku band for hard switch, and it is FFS for soft switch.

We support to define RRM requirements also for soft switch. In our view, the benefit of soft switch over hard switch as observed in FR1-NTN also holds for FR2-NTN. The option provided in the WF for delay requirement is also reasonable.

One additional aspect RAN4 needs to discuss for soft switch is the scheduling restriction. For FR1-NTN, scheduling restriction applies when UE does not support mixed SCS for SSB and data, or when UE does not support simultaneous measurement of another satellite with different Doppler while receiving data in serving satellite. When VSAT is considered, scheduling restriction will be caused by Rx beam switching. This is similar to FR2 TN where UE may steer the Rx beam to another direction than the serving cell.

In FR1-NTN, scheduling restriction is defined on symbol level if deriveSSB-IndexFromCell is enabled, and SMTC window level otherwise, and this is same as FR2 TN. For FR2-NTN, one additional factor Rx beam switching time needs to be considered. In [2], it is mentioned that the Rx beam switching time is typically several 100us, and can be assumed to be less than 1ms. This is much larger than that assumed for FR2 TN which is less than 200ns. If 1ms Rx beam switching time is assumed, the length of scheduling restriction needs to be extended, e.g. to cover 1ms before and after the SMTC window.

Finally, we understand soft switch would only apply to UE with electronic steered beam. For mechanical steered beam, the Rx beam switching time can be on second level. With such a large scheduling restriction, there is no point in using soft switch.

Proposal 1: RAN4 to define requirements for soft satellite switch.

- The delay Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

- Scheduling restriction is assumed around SSB burst of target satellite, and the exact length is FFS depending on the Rx beam switching time

- The requirements apply only for UE with electronic steered beam

Inter-satellite cell reselection requirements

Issue 1-5: inter-satellite cell reselection requirements (section 4.2C). Candidate options: - Option 1: RAN4 to create inter-satellite cell reselection requirements for VSAT UEs with electronically steered beams in Rel-19 (Nokia) - Option 2: Do not define inter-satellite cell reselection requirements in this WI. (HW, ZTE, E///) - Option 2a: RAN4 to consider inter-satellite measurement related discussion for Ku-band NTN in a future release (e.g., Rel-20). (LGE)

We do not support option 1 in Issue 1-5.

- In R18 RAN4 did not define inter-sat cell reselection requirements for VSAT working in Ka band. It is not clear why requirements are needed for VSAT working in Ku bands. Technically, we do see some significance of the requirements for VSAT, but it is not proper to define them in this WI as it is not a specific issue for Ku bands.

- Defining the requirements requires non-trivial efforts. RAN4 needs to discuss the UE behavior for cell search and measurement of a different satellite, e.g. whether it is beam sweeping or beam steering, and the related delay requirements. Considering there are only 2 meetings left for the core part, we suggest not to extend the WI scope too much.

Proposal 2: Do not define inter-satellite cell reselection requirements in this WI.

RLM requirements

Issue 1-8: RLM (section 8.1C) Candidate options: - Option 1: LMax is equal to 8 for FR1-NTN numerolgoy, and 64 for FR2-NTN numerology. - Option 2: LMax is equal to 8 for both FR1-NTN numerology and FR2-NTN numerology.

We support option 2 for Issue 1-8.

In our understanding, the main consideration factor in LMax is the SCS. With 120kHz SCS, it is possible to transmit up to 64 SSBs per SSB burst for FR2-NTN, and RAN1 has not made any exception for Ku band.

Proposal 3: LMax is equal to 8 for FR1-NTN numerology, and 64 for FR2-NTN numerology.

Conclusions

In this paper we provided our views on RRM requirements for NTN in Ku band.

Proposal 1: RAN4 to define requirements for soft satellite switch.

- The delay Tsoft_switch = max(t-service - t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing  ms

- Scheduling restriction is assumed around SSB burst of target satellite, and the exact length is FFS depending on the Rx beam switching time

- The requirements apply only for UE with electronic steered beam

Proposal 2: Do not define inter-satellite cell reselection requirements in this WI.

Proposal 3: LMax is equal to 8 for FR1-NTN numerology, and 64 for FR2-NTN numerology.

Reference

- R4-2508262, WF on RRM requirements for NR_NTN_Ku_bands, Apple