3GPP TSG-RAN WG4 Meeting #116	R4-2510652

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on RRM impacts for SBFD operation

Source: 	Huawei, HiSilicon

Agenda item:	7.21.4.2

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed for RRM impacts for SBFD operation.

- CSI-RS based L1 measurement - puncturing in frequency domain

- RACH requirements

- Lmax when using NR-U approach for L1 CSI-RS measurement

In this paper, we will provide our views on RRM impacts for SBFD operation.

Discussion

CSI-RS based L1 measurement

- Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreement - Define relaxed accuracy requirements based on 24 RB for Case 3 following RAN4#114-bis agreement, based on the simulation assumption in Annex. - Further discuss the upper bound of the measurement period due to processing time and its applicability.

For impact of frequency domain puncturing on CSI-RS L1 measurement, RAN4 agreed to use same number of samples as in existing requirements to define measurement period, and further discuss whether and how to add a lower bound due to scaled processing time as supported in RAN1 [2].

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

In our view, it is reasonable to add a lower bound on the measurement interval.

For a single CSI-RS resource, UE is not supposed to take measurement on an occasion if it has not finished processing of a previous occasion of the same resource, so measurement interval should not be smaller than the processing time. Based on analysis in [2], for UE indicating that CSI processing time is scaled by 2, the processing time would be around 8ms.

On the other hand, in existing delay requirements for L1-RSRP and L1-SINR, UE may be required to measure every 4 slots, since both TReport and TCSI-RS can be configured as 4 slot which is smaller than 8ms.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320 ms | ceil(M*P)*TDRX |

It is noted that the CSI processing time is only applicable for L1-RSRP/L1-SINR which follow CSI framework. RLM/BFD/CBD is not impacted by the RAN1 agreement.

In RAN4#115, some companies [3] suggested to define applicability of the lower bound in relation to the applied accuracy. We share similar view as [3] and our view is that the scaled processing time means UE is measuring 2 DL subbands, so if UE indicates it needs scaled processing time thus has lower bound applied, it should meet exiting accuracy based on 48 RB in Case 3. UE not needing scaled processing time would not have the lower bound in measurement period, but is allowed a relaxed accuracy based on 24 RB.

Proposal 1: If a UE indicates that CSI processing time is scaled by 2, its delay requirements for L1-RSRP and L1-SINR is updated the by adding a lower bound of 8ms to the measurement interval, and the UE should meet existing accuracy requirements based on 48 RB.

RACH requirements

- Issue 2-1-4: Requirements for RACH requirements

We do not see the need for RAN4 to update RACH requirements.

In RAN4#115, some companies suggested to update RACH requirements to reflect the RO type change (from legacy RO to additional RO and vice versa) at PRACH transmission re-attempt. In our view this is a mechanism introduced by RAN2 and has already been clearly captured in RAN2 spec.

In RAN2 MAC CR [4], in 5.1.2 it is specified that the RO selection is based on the selected RO type, and RO type selection is defined in 5.1.1 (initial selection) and 5.1.4 (switch based on attempt number). Current RAN4 requirements are referring to 5.1.2, which means the RO type is already considered.

###### 6.2.2.2.1.2 Correct behaviour when receiving Random Access Response The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. ###### 6.2.2.2.1.3 Correct behaviour when not receiving Random Access Response The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

We do not prefer to duplicate too much in RAN2 spec in RAN4 requirements especially when it is only about UE behaviour without performance impacts.

Proposal 2: RAN4 not to update RACH requirements for SBFD.

Lmax when using NR-U approach

- Issue 2-1-5 (new): Lmax when using NR-U approach for L1 CSI-RS measurement

RAN4 agreed to use NR-U approach to extend L1 CSI-RS measurement period to account for the dropped CSI-RS occasion due to collision with dynamic UL scheduling and invalid symbol type. In RAN4#115, it is agreed to further discuss whether and how to define Lmax in NR-U approach.

In RLM requirements for NR-U, UE behavior related to Lmax is

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB,CCA period becomes better than the threshold Qin_SSB,CCA within TEvaluate_in_SSB,CCA evaluation period. During the in-sync evaluation procedure, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers when Lin exceeds Lin,max, where Lin and Lin,max are defined in table 8.1A.2.2-1.

In our understanding, the intention of specifying this UE behavior is to avoid UE making IS indication based on filtering RS samples from too long time ago. We do not have strong view whether to apply same UE behavior in SBFD. On one hand, the same intention also holds in SBFD. On the other hand, the case where L exceeds Lmax is unlikely to happen in real world but checking Lmax would complicate UE implementation. We slightly prefer to apply same UE behavior in SBFD if there is no strong concern from UE vendors.

It is noted that in NR-U, RLM OOS and BFD are handled differently from RLM IS because it is difficult for UE to determine RS availability at the operating point of RLM OOS and BFD. For SBFD, the resource availability is depending on NW configuration and scheduling, so RLM OOS and BFD should be handled in same way as RLM IS.

In CBD requirements for NR-U, UE behavior related to Lmax is

Table 8.5A.5.2-1: Evaluation period TEvaluate_CBD_SSB_CCA for FR1 Configuration; TEvaluate_CBD_SSB_CCA (ms). non-DRX, DRX cycle ≤ 320 ms; Max(25, Ceil((3 + LCBD) P) TSSB). DRX cycle > 320 ms; Ceil((3 + LCBD) P) TDRX. NOTE 1: TSSB is the periodicity of SSB in the set q1. TDRX is the DRX cycle length. NOTE 2: When DRX is not configured, LCBD is the number of CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. When DRX is configured, LCBD is the number of DRX cycles in which at least one of the CBD-RS SSB occasions not available at the UE during TEvaluate_CBD_SSB_CCA where LCBD ≤ LCBD,max. The UE, which is configured with DRX, is not required to determine the availability of SSB occasions more frequent than Once per Max(25 ms, P * TSSB) if DRX cycle ≤ 320 ms, Once per P * TDRX if DRX cycle > 320 ms. NOTE 3: LCBD,max=7 for Max(TDRX, TSSB) ≤ 40 assuming TDRX=0 for non-DRX, LCBD,max=5 for 40 < Max(TDRX, TSSB) ≤ 320, LCBD,max=3 for TDRX > 320. NOTE 4 If LCBD>LCBD,max, the UE shall assume no new candidate beams are found for this evaluation period..

In our understanding, the intention of specifying this UE behavior is similar to RLM IS, i.e. to avoid UE selecting candidate beam based on filtering RS samples from too long time ago. Similarly, we slightly prefer to apply same UE behavior in SBFD if there is no strong concern from UE vendors.

In L1-RSRP requirements for NR-U, UE behavior related to Lmax is

UE shall report RSRP_0 (Not valid) if L1>L1max, where L1 and L1max are defined in table 9.5A.4.1-1.

In our understanding, the intention of specifying this UE behavior is to inform NW about the unavailability of the RS occasion at UE side. In general, when RS occasion is unavailable at UE side, UE should report the history results, and if there are too many unavailable occasions during the measurement period, UE would report RSRP_0 following NR-U requirements. We do not see same UE behavior needed in SBFD because in SBFD both NW and UE have clear and aligned understanding about the availability of the RS occasions, and there is no need for UE to report a special value to inform NW.

Proposal 3: When re-using NR-U approach for L1 CSI-RS measurement requirements

- Re-use the same UE behavior and Lmax values defined for RLM IS in NR-U for RLM OOS, RLM IS and BFD in SBFD

- Re-use the same UE behavior and Lmax values defined for CBD in NR-U for CBD in SBFD

- Do not define Lmax and related UE behavior for L1-RSRP and L1-SINR in SBFD

Conclusions

In this paper we provided our views on RRM impacts for SBFD operation.

Proposal 1: If a UE indicates that CSI processing time is scaled by 2, its delay requirements for L1-RSRP and L1-SINR is updated the by adding a lower bound of 8ms to the measurement interval, and the UE should meet existing accuracy requirements based on 48 RB.

Proposal 2: RAN4 not to update RACH requirements for SBFD.

Proposal 3: When re-using NR-U approach for L1 CSI-RS measurement requirements

- Re-use the same UE behavior and Lmax values defined for RLM IS in NR-U for RLM OOS, RLM IS and BFD in SBFD

- Re-use the same UE behavior and Lmax values defined for CBD in NR-U for CBD in SBFD

- Do not define Lmax and related UE behavior for L1-RSRP and L1-SINR in SBFD

Reference

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504480, Discussion on SBFD Operation, MediaTek inc.

- R4-2506210, RRM requirements for SBFD operation, Nokia, Nokia Shanghai Bell

- R2-2504444, MAC running CR for Evolution of NR duplex operation: Sub-band full duplex (SBFD), Samsung