3GPP TSG-RAN WG4 Meeting #116	R4-2510662

Bengaluru, India, 25 – 29 August, 2025

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.27.5.2

Document for:	Approval

Introduction

RRM requirements for other requirements in NTN are discussed in RAN4#115, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues for DL coverage enhancement.

- SSB periodicity extension

- SMTC enhancement

- RACH scheduling restriction

In this paper, we will provide our views on RRM requirements for DL coverage enhancement in NTN.

Discussion

SSB periodicity extension

In RAN4#115 following is agreed on RRC Re-establishment requirements.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) No discussion in this meeting: - Due to the potential enhancement of the SMTC configuration in Issue #0, companies' views on this matter appeared to be diverse. ## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms) Agreement: online - In RRC re-establishment requirements, for the case SSB Ês/Iot < -8dB and unknown NR cell, - Remove the [] around the value 22 in the following two bullets. - Tidentify_intra_NR: MAX (800 ms, [22] x TSMTC) for intra-frequency carrier - Tidentify_inter_NR,i: MAX (800 ms, [22] x TSMTC,i) for inter-frequency carrier - Remove Note 1 in Table 6.2C.1.2.1-1 and Table 6.2C.1.2.1-2 for the applicable scenarios, and the value of TSMTC and TSMTC,i is FFS

The remaining issue is the definition of TSMTC and TSMTC,i for the case when serving cell SSB Ês/Iot is < -8dB. In our view, with Note 1 removed, the value of TSMTC and TSMTC,i should be

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier, and

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier

In current requirements, the assumption is that when serving cell SSB Ês/Iot is < -8dB, serving cell timing may be unreliable, so the SMTC offset based on serving cell timing is not used for searching target cell. On the other hand, the periodicity of the SMTC provided by the NW is reliable and can still be used by UE to perform blind search. When SMTC is not provided for the carrier, based on agreement for Issue 1-2-A, the STMC periodicity is assumed to be 160ms.

On issue discussed in RAN4#115 is the potential impact of SMTC enhancement, e.g. what the requirement should be when 2 SMTC periodicities are configured for a carrier. Some companies [2] suggested to use the maximum periodicity. In our view this proposal is reasonable because UE is doing blind search when serving cell SSB Ês/Iot is < -8dB, and it will be very complicated for UE to search different cells with different periodicities because the SSB offset of the target cell is unknown to the UE.

On the applicability of 160ms as default SSB periodicity for initial cell selection, RAN1 agreed to introduce FG 65-1, so the updated requirements for RRC Re-establishment should apply only for UE supporting FG 65-1. On the other hand, RAN1/2 did not make any agreement on whether UE should assume 160ms SSB periodicity for initial cell reselection on all carriers, so RAN4 can further wait for their conclusions.

Proposal 1: For RRC Re-establishment requirements, for the case when serving cell SSB Ês/Iot is < -8dB, TSMTC and TSMTC,i are

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier

  - if 2 SMTC periodicities are configured for a carrier, the maximum periodicity is used

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier.

Another issue in defining RRC Re-establishment requirements for SSB periodicity extension is how to handle the case where SMTC is not configured. RAN4 has agreed that if no SMTC is configured 160ms is assumed as SMTC periodicity. It is noted that when no SMTC is configured, UE can only do blind search. However, in current requirements e.g. Table 6.2C.1.2.1-2, blind search is only assumed for Serving cell SSB Ês/Iot < -8dB.

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. |  |  |  |

This means following legacy requirements, when SMTC is not configured for a carrier, UE needs to meet the highlighted requirements, i.e. K_satellite * MAX (800 ms, 13 x 20 ms). This is not reasonable as 1) 13 samples are not sufficient for blind search and 2) K_satellite cannot be derived without SMTC. Therefore, we suggest to clarify the requirements such that blind search is assumed when Serving cell SSB Ês/Iot < -8dB or when SMTC is not configured for the carrier.

It is noted that this clarification is also needed for legacy requirements in R17. Also, it is only needed for inter-frequency carrier because the case when SMTC is not configured for intra-frequency carrier is not considered in legacy requirements nor in the agreement for Issue 1-2-A.

Proposal 2: Clarify that when SMTC is not configured for a carrier, the blind search requirements (defined for Serving cell SSB Ês/Iot < -8dB) should apply.

SMTC enhancement

In RAN4#115, RAN4 confirmed that the following 2 SMTC related enhancements are feasible.

- E1: support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs

- E2: support configuring two different SMTC periodicities (with different offsets) in one frequency layer for idle, inactive and connected mode neighbor cell measurements

For E1, there are following FFS bullets.

- FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment

We do not see any RRM impact from E1. The impact of SMTC selection already exists in 38.133 since R17, and even RAN2 introduced additional mechanisms to assist UE SMTC selection e.g. location-based SMTC selection, RAN2 also agreed that “In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode”, which is aligned with existing RAN4 requirements.

We also do not see the need to limit the number of SMTCs to <=4 in moving cell scenario. The need of more SMTC comes from DL beam hopping, which is same for fixed cell and moving cell scenarios. Such a limit will impact the mobility performance of moving cell scenario together with DL beam hopping. On the other hand, the number of satellite ephemeris is not increased compared to R17 with maximum of 8 ephemeris for neighbor cell configurable to the UE.

Proposal 3: RAN4 to confirm that there is no RRM requirement impact to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, and there is no need to limit the number of SMTCs for earth moving scenario.

For E2, RAN4 has not discussed the RRM impacts although there are proposals from some companies. In our view, the key is on the measurement requirements especially the scaling factor Kmulti_SMTC. In current requirements, Kmulti_SMTC is defined separately for the cases when SMTCs do not overlap with each other and when SMTCs partially overlap with each other.

Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, and if SMTCs do not overlap with each other, - Kmulti_SMTC=1, if GEO satellites are measured on the carrier; - Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier; if SMTCs partially overlap with each other, - Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier; - Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier; where - NLEO,i is the number of LEO satellites to be measured within i-th SMTC, - NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC, - NSMTC,overlap is the number of SMTCs that partially overlap with each other.

It is however not very clear when SMTCs are considered to be partially overlapping with each other. In Figure 1 we show examples of 2 cases where UE is configured with smtc1 and smtc2 in Case 1, and smtc1 and smtc4 in Case 2. It is noted that both cases are supported in R17 spec, and in our view both cases are considered as “SMTCs that partially overlap with each other”.

Figure 1: Example of existing cases where “SMTCs that partially overlap with each other”

RAN4 has clear definition for SMTC “colliding”.  In clause 9.1C.9.3, the definition of collision between occasions of multiple SMTCs is specified, and in clause 4.2C.2.3, it is specified how to determine whether 2 SMTCs are overlapping based on collision of their occasions.

Excerpt from clause 9.1C.9.3 Collision between occasions of multiple SMTCs may occur as specified in this clause. When UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to 4 ms. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. Excerpt from clause 4.2C.2.3 Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

In TN requirements, Case 1 does not require any scaling, but in NTN there is an additional consideration factor about different satellites, i.e. SMTC#1 and SMTC#2 can be associated with different satellites that UE cannot measure at a single occasion.

With above interpretation of existing requirements, we will discuss the new requirements for E2 based on the example in Figure 2, where UE is configured with 3 SMTCs with different periodicities and offsets.

Figure 2: Example of new cases where “SMTCs that partially overlap with each other”

In our view, Kmulti_SMTC should be defined SMTC specific as different SMTC may see different collision. SMTC#1 is colliding with 2 other SMTCs, but SMTC#2 or #3 is only colliding with 1 other SMTC. It is unreasonable to define NSMTC,overlap as 3 for every SMTC.

In addition, for one single SMTC the collision may be different at different occasions within a period. The 1st and 2nd occasions of SMTC#1 have collision while the 3rd and 4th occasions of SMTC#1 do not. It is means the calculated Kmulti_SMTC for SMTC#1 may be different at different occasions of SMTC#1 (note Kmulti_SMTC may also be different for the 1st and 2nd occasions as number of satellites to be measured in SMTC#2 and SMTC#3 may be different). To define the minimum requirements, we suggest to use the max Kmulti_SMTC among all occasions as the SMTC specific scaling factor.

Proposal 4: To support configuring two different SMTC periodicities with different offsets, RAN4 to update the definition of scaling factor Kmulti_SMTC for measurement requirements:

For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Besides the measurement requirements, [2] made a thorough analysis of potential RRM impacts to support E2. Besides, RRC re-establishment and measurement requirements related to Kmulti_SMTC as discussed above, we share same view as most of the analysis in [2] except the following ones:

- There is no need to further relax M1 factor for serving cell measurement in IDLE mode. Even number of configured SMTCs can be >4, UE is not measuring >4 SMTCs at any time based on RAN2 agreement.

- There is no need to use maximum SMTC periodicity for HO requirements. For HO procedure there is a specific target cell, and target cell is always associated with one SMTC in MO configuration, so it is clear to UE which SMTC periodicity it should use for the target cell.

- The scaling factor Klayer1_measurement in measurement requirements should not be calculated based on the SMTC of the cell being identified. In L1 requirements, scaling factor P is calculated based on the union of the SMTCs. If L1 resource is fully overlapping with the union of SMTCs, occasions of each SMTC would be shared with L1 measurement.

Proposal 5: To support configuring two different SMTC periodicities with different offsets,

- Do not further relax M1 factor for serving cell measurement in IDLE mode.

- Use the periodicity of SMTC associated with target cell for defining HO delay and interruption.

- Calculate scaling factor Klayer1_measurement based on the union of the SMTCs.

RACH scheduling restriction

In RAN4#114-bis, the issue of RACH scheduling restriction was shortly discussed based on [3] but without agreement. In our view, the issue is valid and needs to be addressed by both RAN1 and RAN4.

- Although the objective is for DL coverage enhancement, the issue of insufficient satellite beams that can be simultaneously active is valid for both Tx and Rx. Therefore, beam hopping may also apply for UL.

- Offset of RACH occasion (RO) cannot be configured, but it is always in radio frame 0 or 1 according to Table 6.3.3.2-2 of 38211.

  - If 160ms RO periodicity is used, then only PRACH from UEs in a limited subset of satellite footprints can be received by the NW because in radio frame 0 or 1 the Rx beam is active only for a subset of satellite footprints.

  - If <160ms RO periodicity is used, then for a specific satellite footprint, not all ROs will be valid because the Rx beam is active only in a subset of ROs for this satellite footprint, and it is possible that UE selects an RO that will not be received by the NW as raised up in [3].

For above reasons, we see it meaningful to apply at least a time offset for valid RO determination so that UE in each satellite footprint will transmit PRACH on ROs that corresponds to active Rx beam for its satellite footprint. The detailed solution, including how the offset are determined and whether the offset also applies for Tx and Rx of other RACH related signals and channels, can be up to RAN1. Once RAN1 decides on the solution, RAN4 can define scheduling restriction requirements accordingly.

Proposal 6: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping.

A draft LS is provided in the Annex.

Conclusions

In this paper we provided our views on RRM requirements for DL coverage enhancement in NTN.

Proposal 1: For RRC Re-establishment requirements, for the case when serving cell SSB Ês/Iot is < -8dB, TSMTC and TSMTC,i are

- the actually configured SMTC periodicity (up to 160ms) if SMTC is provided for the carrier

  - if 2 SMTC periodicities are configured for a carrier, the maximum periodicity is used

- the default SMTC periodicity of 160ms if SMTC is not provided for the carrier.

Proposal 2: Clarify that when SMTC is not configured for a carrier, the blind search requirements (defined for Serving cell SSB Ês/Iot < -8dB) should apply.

Proposal 3: RAN4 to confirm that there is no RRM requirement impact to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs, and there is no need to limit the number of SMTCs for earth moving scenario.

Proposal 4: To support configuring two different SMTC periodicities with different offsets, RAN4 to update the definition of scaling factor Kmulti_SMTC for measurement requirements:

For each SMTC#i, Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i.

Proposal 5: To support configuring two different SMTC periodicities with different offsets,

- Do not further relax M1 factor for serving cell measurement in IDLE mode.

- Use the periodicity of SMTC associated with target cell for defining HO delay and interruption.

- Calculate scaling factor Klayer1_measurement based on the union of the SMTCs.

Proposal 6: Send LS to inform RAN1 about the need for time offset for PRACH transmission considering UL beam hopping.

Reference

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)

- R4-2506433, Discussion on the RRM requirement for NTN phase3 DL CE, CMCC

- R4-2503536, On R19 other NTN RRM requirements, Apple

Annex: draft LS on scheduling restriction for RACH related signals and channels

3GPP TSG-RAN WG4 Meeting #115 R4-250xxxx Malta, MT, 19 – 23 May, 2025 Title: LS on scheduling restriction for RACH related signals and channels Response to: Release: Release 19 Work Item: NR_NTN_Ph3-Core Source: RAN4 To: RAN1 Cc: Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 is defining requirements for DL coverage enhancement, and it is RAN4 understanding that the satellite beam hopping may also apply in UL. RAN4 is discussing scheduling restriction requirements for PRACH and other Tx and Rx of other RACH related signals and channels, and it is identified that at least a time offset for PRACH transmission is needed considering UL beam hopping. RAN4 kindly asks RAN1 to decide the details of the time offset for PRACH transmission considering UL beam hopping, and whether the time offset applies to other Tx and Rx of other RACH related signals and channels. 2. Actions: To RAN1: RAN4 kindly asks RAN1 to decide the details of the time offset for PRACH transmission considering UL beam hopping, and whether the time offset applies to other Tx and Rx of other RACH related signals and channels. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #116 August 25 – August 29, 2025 India (TBC), IN RAN WG4 Meeting #116-bis October 13 – October 17, 2025 Prague, CZ