3GPP TSG-RAN WG4 Meeting #116	R4-2510686

# Bangalore, India, 25 Aug - 29 Aug, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of R19 XR

Agenda Item:	7.26.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, around the RRM aspects of XR scenario, some good progress was achieved in [1] and we list as below:

Topic 2 Measurements with gaps Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms - Issue 2-2: NCSG <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with ncsgInd-r17. - Issue 2-3: Preconfigured measurement gaps <Agreement> - MG cancelling DCI doesn’t apply to MGs configured via GapConfig-r17 with preConfigInd-r17. - Issue 2-4: Rel-18 concurrent gaps <Agreement> - MG cancelling DCI doesn’t apply to Rel-18 concurrent gaps. Topic 3 XR skipping requirements impact - Issue 3-6: Impact on different frequency layers <Agreement>: - Not pursue Proposal 1 and 2 in Rel-19. Issue 3-7: UL XR impact <Agreement>: - This issue is closed. Topic 4 UE assistance information Issue 4-1: Additional aspects for UAI <Agreement>: - The UE may provide the UAI related information, for the UAI in the agreement captured above, per MG configuration. - per-FR gap if gap is configured with gapConfig with per-FR gaps - per gap configuration if gap is configured with gapConfig-r17 Topic 5 Additional signalling aspects Issue 5-1: Semi-static schemes <Agreement>: - From RAN4 perspective, no consensus to support semi-static solutions in Rel-19. Issue 5-2: Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec. Issue 5-3: Partial measurement skipping <Agreement>: - This issue is closed. Topic 6 CR handling Issue 6-1: Naming convention <Agreement>: - Use “Measurement gap occasion cancelation” to refer to measurement gaps that were skipped or cancelled by DCI. - Use Lcancel for number of skipping occasions - Indication of which formula/table to use - For the UE supporting [FFS feature name] provided that the configuration and conditions in clause 9.1.y.4 are met (new) - Use RAN2 pseudo code approach for new clause 9.1.y Topic 7 UE capabilities Issue 7-1: UE capabilities for XR – general proposals <Agreement>: - Define optional UE feature for support of UAI with per UE capability, [and no FR1/FR2 differentiation].

- In this contribution, we provide our further views on the suspending issues for XR in Rel-19.

- Discussion

2.1 Applicable MG types

Type 2 gaps

In last meeting, it has been approved that the Type 2 gaps i.e. R18 concurrent MGs are applicable to the MG canceling. While a small issue is still suspending, it was highlighted as below:

<Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

We believe the FFS issue raised concern on the case when the distance between the two non colliding concurrent gaps are just larger than 4ms, then it is complicated for NW to indicate the skipping of the 2nd gap. We show this case in figure 1 as below. Our view is that no matter to skip the 1st gap(MG1 in Figure 1) or the 2nd gap(MG2 in Figure 2), NW could accomplish via adjusting the DCI transmission instance. As shown in Figure 1, if NW expects to cancel MG1, then NW could send DCI1, the time offset of 5ms between the DCI and the canceled MG meets in this case. If NW expects to cancel MG2, then NW just send DCI2 since the time offset of 5ms meets between DCI2 and MG2, furthermore, DCI2 would not impact MG1 since the time offset does not meet between DCI2 and MG1. So we can not see any challenge to cancel any gap when non colliding concurrent gaps exists.

- Figure 1. An example of allowing gap skipping for concurrent MGs

Proposal 1: For the gap canceling in the case of two non colliding concurrent gaps are separated by just larger than 4ms, NW could cancel any gap via different DCI transmission instance, the FFS part could be addressed.

2.2 Exact requirements

- Formulas for measurement delay extension without DRX

- The principle for the measurement delay calculation has been approved in last meeting, i.e. the NR-U approach is applied for measurement delay extension. The exact formulas of extension should be determined. This issue was discussed in last meeting and the following options were captured in [1] as below:

- RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter

- Between these options, Option 2 is the NR-U approach, and actually Option 1 is revised based on Option 2 to control the extension in case it is too long. So we are fine with Option 1.

Proposal 2: RAN4 to define the measurement delay extension for inter-frequency measurement with gap without DRX as: Max(200 ms CSSFinter, 8×Max(MGRP, SMTC period)×CSSFinter + Nskip× Max(MGRP , SMTC period)).

- -  where Nskip is the number of  skipped MG occasions in the measurement period.

- Formulas for measurement delay extension with DRX

- Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

- Along with the extension formula for non DRX, the formula for the extension with DRX is preferred by us to adopt the revised option compared with NR-U approach, so Option 1 is preferred by us.

Proposal 3: RAN4 to define the measurement delay extension for inter-frequency measurement with gap with DRX as:

- If DRX cycle ≤ 320 ms, Max(200 ms CSSFinter, Ceil(8×1.5)×Max(MGRP, SMTC period, DRX cycle)×CSSFinter + Nskip× Max(MGRP, SMTC period, DRX cycle))

- Otherwise, 8  DRX cycle  CSSFinter + Nskip  DRX cycle

- where Nskip is the number of skipped MG occasions in the measurement period.

- Conditions for measurement skipping

Around this issue, the following options were raised in last meeting:

- Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

For Option 3, the concern behind is whether UE could utilize multiple non-successive measurement samples to derive the measurement result when the interval between them are very large. To our understanding, how often does UE measure one MO, it is up to UE implementation in legacy, but considering UE derives the measurement result through multiple measurement samples, if the non-slipped gaps used to derive one measurement result distribute too dispersively, it seems no benefit to utilize the measurement on the gap occasion long time ago provided successive gap skipping happens. So Option 3 is necessary. Regarding the exact maximum separation between two non-skipped gaps, one simple solution is to define the maximum separation as which guarantee the two non-skipped gaps are covered by a single measurement period, as shown in Figure 2.

For Option 2, it seems to achieve similar goal as Option 3, i.e. to avoid too much successive gap occasions being skipped. But if the gap skipping is not successive during the measurement period, it is hard to decide the upper bound number referred by Option 2.

Option 1 means not any additional optimization, it is also fine to us.

Proposal 4: If some optimization is considered regarding the condition of measurement skipping, Option 3 is preferred by us, i.e. define a maximum separation between two non-skipped gaps for measurement.

- The maximum separation could be defined as: the maximum separation guarantees the two non-successive non-skipped gaps are covered by a single measurement period.

- The skipping impact in frequency domain

Furthermore, the gap is configured as per-UE type or per-FR type. When receiving the skipping indication through scheduling DCI, the impacted frequency range is decided by the type of the skipped gap. If it is a per-UE gap, then both FRs are impacted by the gap skipping. If it is a per-FR gap, then only the same FR is impacted by the gap skipping. All these have been achieved as common understanding in last RAN4 meeting as below, but whether and how to capture in RAN4 spec, which is still diverse between companies.

<Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

Some companies raised that RAN1 CR has already captured these, in latest RAN1#121 meeting, RAN1 approved a TP to capture all the content relevant to gap skipping as below:

## 10.6 Indication for cancelation of RRM measurement gaps An XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133] as described below. A UE can be indicated by a value ‘1’ of an XYZ measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception to cancel an earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. The earliest RRM measurement gap occasion is at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. If a UE indicates “ValueOption 1” for FG 64-1-ABC, the UE ignores a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format. If a UE indicates “ValueOption 2” for FG 64-1-ABC, a value ‘0’ of the XYZ measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the earliest associated RRM measurement gap occasion after the PDCCH reception where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the DCI format is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the XYZmeasurement gap cancellation field and are associated with a same earliest associated RRM measurement gap occasion where the UE would perform RRM measurements on cells in a same frequency band as the one or more scheduled cells associated with the more than one DCI formats, the UE applies the indication provided by the XYZmeasurement gap cancellation field of a last DCI format, from the more than one DCI formats, that is in a PDCCH reception that ends at least 3 msec or 5 msec, as determined by a reported UE capability, before the start of that earliest RRM measurement gap occasion.

After careful check on this RAN1 TP, we have the following observation:

Observation 1: RAN1 TP interpret the impact frequency range as “on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]”, but what is the “applicable range of the measurement gap occasion”, no further explanation and 38.133 is referred.

RAN1 focuses on the which serving cell could transmit traffic at the skipped gap occasion, instead of which frequency layer’s measurement is impacted.

Based on current RAN1 TP status, it is necessary to capture the exact applicable range of the measurement gap occasion in RAN4 spec. Furthermore, based on previous RAN4 agreements as below, actually not only the measurement with gap is impacted by the gap skipping, the measurement without gap and relevant scheduling restriction are also impacted.

<Agreement> in 114 meeting - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

Proposal 5: When UE receives gap skipping indication in DCI,

- For per-UE gap, the gap skipping is valid for the measurements on any frequency layer during the canceled gap occasion;

- For per-FR gap, the gap skipping is valid for the measurements on any frequency layer of the same FR during the canceled gap occasion.

Regarding how to capture, we have the following proposal:

Proposal 6: The impact of gap skipping could be captured in L3 measurement with gap, L3 measurement without gap, L1 measurement with gap, L1 measurement without gap chapters, including the impact on the scheduling restriction.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

Proposal 1: For the gap canceling in the case of two non colliding concurrent gaps are separated by just larger than 4ms, NW could cancel any gap via different DCI transmission instance, the FFS part could be addressed.

Proposal 2: RAN4 to define the measurement delay extension for inter-frequency measurement with gap without DRX as: Max(200 ms CSSFinter, 8×Max(MGRP, SMTC period)×CSSFinter + Nskip× Max(MGRP , SMTC period)).

Proposal 3: RAN4 to define the measurement delay extension for inter-frequency measurement with gap with DRX as:

- If DRX cycle ≤ 320 ms, Max(200 ms CSSFinter, Ceil(8×1.5)×Max(MGRP, SMTC period, DRX cycle)×CSSFinter + Nskip× Max(MGRP, SMTC period, DRX cycle))

- Otherwise, 8  DRX cycle  CSSFinter + Nskip  DRX cycle

- where Nskip is the number of skipped MG occasions in the measurement period.

Proposal 4: If some optimization is considered regarding the condition of measurement skipping, Option 3 is preferred by us, i.e. define a maximum separation between two non-skipped gaps for measurement.

- The maximum separation could be defined as: the maximum separation guarantees the two non-successive non-skipped gaps are covered by a single measurement period.

Observation 1: RAN1 TP interpret the impact frequency range as “on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]”, but what is the “applicable range of the measurement gap occasion”, no further explanation and 38.133 is referred.

RAN1 focuses on the which serving cell could transmit traffic at the skipped gap occasion, instead of which frequency layer’s measurement is impacted.

Proposal 5: When UE receives gap skipping indication in DCI,

- For per-UE gap, the gap skipping is valid for the measurements on any frequency layer during the canceled gap occasion;

- For per-FR gap, the gap skipping is valid for the measurements on any frequency layer of the same FR during the canceled gap occasion.

Proposal 6: The impact of gap skipping could be captured in L3 measurement with gap, L3 measurement without gap, L1 measurement with gap, L1 measurement without gap chapters, including the impact on the scheduling restriction.

- Reference

- R4-2508285, WF on RRM requirements for XR_Ph3, Nokia