3GPP TSG-RAN WG4 Meeting #116	R4-2510695

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on LBCA requirements

Agenda item:	7.3.4

Document for:	Discussion

Introduction

In this contribution we discuss the remaining RRM requirements aspects of LBCA switching.

Discussion

General

In the last RAN4 meeting, RRM requirement applicability was discussed under issue 1-1-3 [1] without a proper conclusion:

Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching [Wayforward]: FFS: options in topic summary R4-2505556.

Some agreements were made on individual issues regarding measurement availability, but for completeness, we will repeat our general proposal here. The conclusion on this issue can be reflected to the other individual open issues.

Measurement requirements for serving cells, FDD PCell or SDL SCell, heavily depend on the switching pattern design and the SSB availability on the activated serving cell based on the pattern. Based on the type of the SSB availability, there can be three possible scenarios, which are explained below:

- Scenario 1: All {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for the FDD PCell and/or SCell based on the LB-CA switching pattern.

- Scenario 2: A fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available for the FDD PCell and/or SCell based on the LB-CA switching pattern.

- Scenario 3: No {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for the FDD PCell and/or SCell based on the LB-CA switching pattern.

Figure 1, Figure 2 and Figure 3 illustrates Scenario 1, Scenario 2, and Scenario 3 respectively.

In Figure 1 we provide one example of a switching pattern that allows for the UE to be active on all the SSB slots for both the PCell and SCell at different SSB instances. Since, all the SSBs for both the PCell and SCell are available for measurement, there is no need for delay extension for either of the serving cells.

Figure 1 Example of Scenario 1

The LB-CA switching pattern can lead to a scenario where all the {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for measurement in the serving cells.

In Figure 2 we provide one example of a switching pattern that allows for the UE to be active on SSB slots for both the PCell and SCell at different SSB instances. Since, a fraction of SSBs (in this case half) are not available for either the PCell or SCell, the delay is scaled in accordance to the fraction of available occasions in both the serving cells.

Figure 2 Example of Scenario 2

The LB-CA switching pattern can lead to a scenario where only a fraction of {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} being available for measurement in the serving cells.

In Figure 3 we provide one example of a switching pattern that doesn’t allow for the UE to be active on any of the SSB slots for SCell at different SSB instances, whereas the UE is active on all the SSB slots for PCell. Since, none of the SSBs for both SCell are available for measurement, the corresponding requirements do not apply for the SCell.

Figure 3 Example of Scenario 3

The LB-CA switching pattern can lead to a scenario where none the {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for measurement in the serving cells.

For gap-less based L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

  - If all {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed

  - If a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

  - If no {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Interruption requirements

In the last RAN4 meeting, interruption requirements when more than two carriers are considered were discussed without a conclusion under issue 1-2-2:

Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered [Wayforward]: FFS: options in topic summary R4-2505556.

In RAN4 #114, RF agreed that there may be a third carrier combined with 2 low band carriers, which not to be used with a switching pattern.

Issue 1: Scope clarification - Specify the switching period(s) based on the scenario with the switching between two low bands in Rel-19 WI - It is not precluded that such two low bands are further combined with the additional band - The combination of those two low bands with other band can be further discussed in the future release

Considering the RF agreement, it is not precluded that additional carriers can be combined with the LBCA-sw, although the scenario to be specified in the ongoing Rel-19 work item only considers two band. In this scenario, we have 3 band carriers, 2 low band carriers which operate with carrier switching, and another band that does not need carrier switching. One of the first contributions proposing such combination can be found at [2]. If this is allowed by RF and UE capabilities, we need do discuss from RRM perspective whether the 3rd carrier may experience interruptions or not.

RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

RAN4 to discuss the need for interruption requirements in the following scenario

  - UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Regarding the scenario above, allowing for interruptions could be very detrimental for performance of the 3rd band. In L3 measurements, RAN4 has specified interruption requirements for measurements without gaps, and those interruptions can be controlled by the network by configuring the SMTC periodicity, e.g. 40, 80, 160 ms. Therefore, the interruptions for L3 measurements can be kept at low levels. On the other hand, in the case of LBCA-sw the situation is different. If the UE architecture is such that it causes interruptions on the 3rd carrier every time it switches between FDD and SDL carrier, the interruption overhead would be very high for the 3rd carrier, and that would be much more often than the SMTC periodicity.

The RAN1 design of the carrier switching pattern is flexible to allow flexible configuration, but some practical considerations have to be taken into account when configuring the LBCA-sw pattern. For example, the UE can only be configured to stay in the SDL for a limited number of consecutive slots, otherwise there will not be opportunity for the UE to acknowledge PDSCH or send other UL signals. Therefore, it can be assumed that it needs much more frequent switching than what is used for L3 measurements.

Figure 1 shows one example of LBCA-sw configuration, and the impact of 1 slot interruptions on the 3rd band. As it can be noticed, in this configuration, if 1 slot is considered to be interrupted at every FDD/SDL transition, there are 14 slots interrupted from 40 slots in the pattern. That results in 35% interruption ratio.

If we take the proposal in R4-2506739 [3], the resulting interruption is of 2 slots for every FDD/SDL transition if the 3rd band also uses 15 kHz SCS. In that case the interruption ratio would increase to 70% in the scenario of Figure 1.

Proposals on the interruption length on a 3rd band without LBCA-switching could result in 30% to 70% interruption ratio.

Figure 1 Example of LBCA+sw causing interruption on 3rd band without LBCA switching

As pointed out in our analysis, this issue is very important because the interruption ratio could potentially be huge. If the requirements are unclear in this case, the operation with a 3rd band would be not feasible. At the same time, no requirements could mean that networks are not comfortable in configuring such combinations. As a result, even if a UE doesn’t need interruption, it would not be able to benefit from the operation with a 3rd band.

Since this WID is planned conclude the work in RAN4 #116, we believe that the best approach is to consider only the cases without interruptions on the 3rd carrier as part of this WID, or to decide to define the requirements once 3 or more bands are defined.

If band combos with 3 or more bands are included where 2 bands use LBCA-sw, RRM will discuss the interruption requirements.

Rel-19 LBCA-sw RRM requirements do not apply if more than 2 cells are active simultaneously.

Impact on SCell requirements

L3 measurements on activated SDL SCell

In the last RAN4 meeting, the following was agreed/open for activated SDL measurements [1]:

Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design.

Considering the FFS on issue 1-3-2, it is FFS the details of the measurement delay requirements for activated SCell L3 measurement. Taking the agreement from 1-3-2, and considering existing formulas for concurrent gap measurement delay, we can simply extend the delay considering the number of total SMTC occasions and the number of available SMTC occasions on a time window.

Apply the conclusion from Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching.

  - If case b in issue 1-1-3 is agreed, new Kl factor can be used for requirement design.

    - Kl=Ntotal/Navailable, where

    - Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W,

    - Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W

    - The window W is of duration max(SMTC period, 40 ms)

L3 measurements on deactivated SCell

In the last RAN4 meeting, the following was agreed/open for deactivated SDL measurements [1]:

Issue 1-3-4: Requirement design of deactivated SDL SCell measurement and corresponding interruption Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

As for interruption due to deactivated SCell measurement, we already have an agreement to follow legacy requirements. Therefore, we propose that we just clarify the agreement, i.e. which exact legacy case to be considered. In the requirements of 8.2.2.2.3, there is a distinction for the interruption length when the target band is contiguous intra-band or not. In the case of inter-band the interruption length is reduced to the requirement of Table 8.2.2.2.2-1, whereas for intra-band, and depending on UE capabilities, the interruption is extended to include the SMTC duration as in Table 8.2.2.2.3-1. We understand that the LBCA-sw case is similar to the intra-band case where the UE is also causing an interruption during the SMTC duration. Therefore, we can follow the legacy requirements of Table 8.2.2.2.3-1 and state that the missed ACK/NACK should not exceed 0.5% if measCycleSCell is 640 ms or larger

Legacy interruption length requirement for intra-band deactivated SCC measurement can be reused.

  - Interruption requirements in table 8.2.2.2.3-1 apply for deactivated SDL measurement

  - Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell is 640 ms or larger

SCell activation delay

Issue 1-3-8: interruption or scheduling restriction during SDL SCell activation/deactivation [Wayforward]: FFS: options in topic summary R4-2505556.

Since RAN4 agreed that the switching pattern will not apply during SCell activation, it has been discussed whether to handle the SCell measurements related to SCell activation through interruptions or scheduling restrictions. To make progress, RAN4 can try to agree to define scheduling restrictions.

During the SDL SCell activation, upon receiving a command from the network to activate the SCell, the UE will receive the necessary reference signals (RS) on the to be activated SCell for enabling the SCell activation. Scheduling restrictions are then needed on the OFDM symbols in the PCell overlapping and adjacent to the reference signals to be measured on the SDL SCell.

In Figure 5 we provide one example of the steps involved during the activation of an SDL SCell with scheduling restrictions. In this example the UE receives a SCell activation command, and whenever there are reference signals to be measured from the SCell, the network applies scheduling restrictions to allow the UE to monitor signals on the SCell. After the activation time completes, the UE sends a CSI report and the SCell is active. At this stage the LBCA switching pattern starts.

Figure 5 Switched carrier behaviour during measurement of SCell RS during SCell activation time using scheduling restrictions

One point that was raised about the scheduling restrictions, is whether they should also include switching time and TA before and after the reference signals. In our view, this extension of the symbols for scheduling restrictions can be done in a simple way by reusing the parameters for the switching gap defined by RAN1.

  - RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

  - The scheduling restriction applies for the symbols of PCell overlapping with the reference signals of SCell to be measured for SCell activation or the associated switching delays, where:

    - UE is not expected to receive PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot or transmit PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the PCell that overlap, even partially, with the set of symbols of LBCA-SwitchingGap-Duration-PCelltoSCell + SSB/SMTC duration/CSI-RS + LBCA-SwitchingGap-SCelltoPCell.

Impact on PCell

In the last RAN4 meeting, the following was agreed/open [1]

PCellPCellPCell Issue 1-4-2: others RRM requirement of FDD PSCell in LB CA via switching [Wayforward]: FFS: options in topic summary R4-2505556.

Regarding the other PCell RRM requirements, RAN4 can reuse the conclusions from the SCell requirements. In our view the simplest approach is to take into consideration the ratio of available SSB/CSI-RS occasions when defining L1/L3 measurement delay extension.

Follow SCell conclusions when defining measurement requirements on PCell due to LBCA switching pattern.

Conclusion

The following Observations and Proposals were made:

Observation 1: The LB-CA switching pattern can lead to a scenario where all the {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for measurement in the serving cells.

Observation 2: The LB-CA switching pattern can lead to a scenario where only a fraction of {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} being available for measurement in the serving cells.

Observation 3: The LB-CA switching pattern can lead to a scenario where none the {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available for measurement in the serving cells.

Proposal 1: For gap-less based L3 measurement and L1 measurement of a serving cell which can be either FDD PCell or activated SDL SCell:

a.	If all {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed

b.	If a fraction of {the SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is scaled in accordance to the fraction of available occasions.

c.	If no {SSB occasions in SMTC window or CSI-RS for L3 measurement} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Observation 4: RF has reached agreement that a scenario with 2 low bands operating with an additional band is not precluded.

Observation 5: RAN4 to discuss the need for interruption requirements in the following scenario

a.	UE configured with 2 low band carriers operating with carrier switching, and a 3rd band carrier operating without switching

Observation 6: Proposals on the interruption length on a 3rd band without LBCA-switching could result in 30% to 70% interruption ratio.

Proposal 2: If band combos with 3 or more bands are included where 2 bands use LBCA-sw, RRM will discuss the interruption requirements.

Proposal 3: Rel-19 LBCA-sw RRM requirements do not apply if more than 2 cells are active simultaneously.

Proposal 4: Apply the conclusion from Issue 1-1-3: RRM requirement applicability for R19 LB CA via switching.

a.	If case b in issue 1-1-3 is agreed, new Kl factor can be used for requirement design.

i.	Kl=Ntotal/Navailable, where

ii.	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W,

iii.	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W

iv.	The window W is of duration max(SMTC period, 40 ms)

Proposal 5: Legacy interruption length requirement for intra-band deactivated SCC measurement can be reused.

a.	Interruption requirements in table 8.2.2.2.3-1 apply for deactivated SDL measurement

b.	Interruptions on PCell or activated SCell(s) due to measurements when an SCell is deactivated are allowed with up to 0.5 % probability of missed ACK/NACK when the configured measCycleSCell is 640 ms or larger

Proposal 6:

a.	RAN4 to define the scheduling restriction caused by the measurements during SDL SCell activation procedure.

b.	The scheduling restriction applies for the symbols of PCell overlapping with the reference signals of SCell to be measured for SCell activation or the associated switching delays, where:

i.	UE is not expected to receive PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot or transmit PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the PCell that overlap, even partially, with the set of symbols of LBCA-SwitchingGap-Duration-PCelltoSCell + SSB/SMTC duration/CSI-RS + LBCA-SwitchingGap-SCelltoPCell.

Proposal 7: Follow SCell conclusions when defining measurement requirements on PCell due to LBCA switching pattern.

- References

- R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2507060, Discussion on low-band CA switching time mask and fallback applicapability, MediaTek Inc.

- R4-2506739, Discussion on low band carrier aggregation via switching, Huawei, HiSilicon

- R1-2505061,  Introduction of low NR band carrier aggregation via switching for NR, Samsung