3GPP TSG-RAN WG4 Meeting #116	R4-2510706

Bengaluru, India, 25th – 29th August 2025

Agenda item:	7.19.3.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on requirements for RRM measurement prediction on AIML mobility

Document for:	Discussion

- Introduction

The SI was revised at RAN#105[1] and WF was approved at RAN4#115 [2]. In this contribution, we further discuss the remaining requirements for RRM measurement prediction on AIML mobility.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

Issue 2-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy Issue 2-5: Impacts on beam/cell level measurements Agreement: Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2. Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase. Issue 2-7: Inter-frequency prediction scenario Agreement: RAN4 to study whether and how correlation coefficient can be considered in simulation assumption for RAN4 requirements.

Returning to the definition of Relative accuracy of predicted L3-RSRP, this is for confirming the accuracy of AIML predictions in terms of difference measurement between two RSRP values. The key aspect is whether the difference between two RSRP value is accurate or not. From this viewpoint, even if there is a certain time difference between predicted L3-RSRP of cell 1 reporting time and L3-RSRP of cell 2 reporting time, it is no matter for Relative RSRP evaluation as illustrated in figure 1.

Figure 1 Relative RSRPs in two different cases of reported RSRP timing.  (Case 1) Cell 1 predicted RSRP and Cell predicted/measured RSRP at the same timing are used for report. (Case 2) The predicted RSRP of Cell 1 is measured earlier than that of Cell 2.

Observation 1: Relative RSRP evaluation doesn’t depend on time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

Proposal 1: Relative accuracy of predicted L3-RSRP for intra-frequency L3-RSRP can be specified without considering time difference between the two reported RSRPs.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: Relative RSRP evaluation doesn’t depend on time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy.

Proposal 1: Relative accuracy of predicted L3-RSRP for intra-frequency L3-RSRP can be specified without considering time difference between the two reported RSRPs.

References

[1] 3GPP, RP-242393, “Revised SID on AIML for mobility in NR”, OPPO.

[2] 3GPP, R4-2508268, “WF on AI/ML Mobility”, Nokia.