3GPP TSG-RAN WG4 Meeting #116	R4-2510707

Bengaluru, India, 25th – 29th August 2025

Agenda item:	7.19.3.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on requirements for measurement event prediction on AIML mobility

Document for:	Discussion

- Introduction

The SI was revised at RAN#105[1] and WF was approved at RAN4#115 [2]. In this contribution, we further discuss the remaining requirements for RRM measurement event prediction on AIML mobility.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

RAN4 requirements for measurement event prediction Issue 3-1: Performance metric for event prediction use case Agreement: In case of indirect measurement event prediction use case, if the report includes the predicted RSRPs [corresponding to the predicted event occurrence time], RAN4 will define requirement for the absolute and/or relative accuracy of predicted RSRP.

From RAN2#130 discussion, the report contents are summarized as follows.

Agreements Configuration and report of inference input to network sided model - For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used. For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero. FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction. - To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model. Data collection For the collection of data for the training of a network-sided model for RRM measurement prediction: - UE can be configured to log, at a certain logging periodicity, one or more of the following: - L3 cell level measurements, - L3 beam level measurements, - L1-filtered beam level measurements (if sub-case 1 and 3 is supported) - Cell ID (FFS CGI of serving cell. If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback) - Time info (if as agreed by AI/ML and/or if needed) - The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase. Required enhancements are FFS. - The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity. - The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case. - The availability indication can be triggered due to: - full buffer being reached (if configured), (FFS if buffer is per use case) - buffer threshold being reached (if configured), or (FFS if buffer is per use case) - low power (if configured) - Upon sending the availability indication, UE indicates: - Data is available - Reason for the triggering of the availability indication (full buffer. Threshold) - Low power indication - The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling) - The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO). - The UE releases the collected data upon transitioning to IDLE/INACTIVE - For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial. It can be up to WI phase if this is done depending on whether a simple solution can be found For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

There was no report content related to indirect measurement event predictions in the agreement on the reports from RAN2#130. Thus, RAN4 don’t need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction.

Observation 1: The report doesn’t include the predicted RSRPs in case of indirect measurement event prediction use case.

Proposal 1: No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: The report doesn’t include the predicted RSRPs in case of indirect measurement event prediction use case.

Proposal 1: No need to specify additional requirements on relative accuracy of predicted RSRP for measurement event prediction.

References

[1] 3GPP, RP-242393, “Revised SID on AIML for mobility in NR”, OPPO.

[2] 3GPP, R4-2508268, “WF on AI/ML Mobility”, Nokia.