3GPP TSG-RAN WG4 Meeting #116	R4-2510708

Bengaluru, India, 25th – 29th August 2025

Agenda item:	7.22.4

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM requirements for A-IoT

Document for:	Discussion

- Introduction

From the last meeting RAN4#114bis, we started to discuss RRM requirements on A-IoT device 1 based on the WID [1]. Some timing related issues are still open, we contribute the general view.

- Discussion

According to the latest WF [2], the open issues of RRM core requirements are described as follows.

Sub-topic 1-1: A-IoT random access procedure Issue 1-1: A-IoT random access procedure Agreement: - RAN4 to include following aspects for RRM requirements for A-IoT random access. The details of UE behaviour will be updated based on RAN2 spec. - CBRA procedure - the correct behaviour when transmitting A-IoT MSG1(Random ID transmission) - the correct behaviour when receiving A-IoT MSG2 - the correct behaviour when not receiving A-IoT MSG2 - CFRA procedure - the correct behaviour when transmitting First D2R message Sub-topic 1-2: timing Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any. Issue 1-2-2: Device receive timing FFS: - Option 1 (ZTE): RAN4 shall consider accuracy requirements on TD2R_min if the range of it depends on RAN1.

In terms of RACH procedure, we need to specify RRM requirements for CBRA procedure and CFRA procedure. Firstly, we propose on the case as follows,

- CBRA procedure

- the correct behaviour when transmitting A-IoT MSG1(Random ID transmission)

- the correct behaviour when receiving A-IoT MSG2

- CFRA procedure

- the correct behaviour when transmitting First D2R message

From RAN 1 and RAN2 discussion to date, the procedure of R2D and D2R is illustrated as below.

Figure 1 R2D and D2R procedure.

In terms of RACH procedure, the synchronization between the reader and device is triggered each time the reader transmits R2D and the device receives it. The timing offset can be acquired from R2D message and continuous transmission of D2R after receiving R2D is not supported. Since there is no timing acquisition deference between the Msg1 and other D2R messages, we don’t need to specify additional RRM specification on A-IoT random access procedure.

Observation 1: There is no deference of timing offset acquiring process between the Msg1 and D2R on the case as follows.

- CBRA procedure

- the correct behaviour when transmitting A-IoT MSG1(Random ID transmission)

- the correct behaviour when receiving A-IoT MSG2

- CFRA procedure

- the correct behaviour when transmitting First D2R message

In the remaining case as below, it depends on RAN2 progress.

- CBRA procedure

- the correct behaviour when not receiving A-IoT MSG2

RAN2 agreements in RAN2#130 are summarised as follows.

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it. Agreements on NACK reception: - After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access. FFS how to specify. Agreements on RN16/AS ID maintenance: - Confirm a device is not expected to maintain both AS ID and RN16. After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader. This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission. How to capture device behavior is FFS

The candidates of CBRA procedure have been narrowed down to Option B and Option C. The trigger of transmitting Msg3 is receiving Msg2, and the devices don’t transmit signalling when not receiving MSG2. Thus, no need to specify the RRM requirements on this case.

Observation 2: A-IoT devices don’t transmit signalling and no need to specify delay requirements on the case as follows.

- CBRA procedure

- the correct behaviour when not receiving A-IoT MSG2

From the above, we don’t need to specify the additional requirements on A-IoT RACH procedure.

Proposal 1: No need to specify additional RRM specification on A-IoT random access procedure in terms of timing requirement.

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: There is no deference of timing offset acquiring process between the Msg1 and D2R on the case as follows.

- CBRA procedure

- the correct behaviour when transmitting A-IoT MSG1(Random ID transmission)

- the correct behaviour when receiving A-IoT MSG2

- CFRA procedure

- the correct behaviour when transmitting First D2R message

Observation 2: A-IoT devices don’t transmit signalling and no need to specify delay requirements on the case as follows.

- CBRA procedure

- the correct behaviour when not receiving A-IoT MSG2

Proposal 1: No need to specify additional RRM specification on A-IoT random access procedure in terms of timing requirement.

References

[1] 3GPP, RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.

[2] 3GPP, R4-2508275, “WF on RRM requirements for Ambient_IoT_Solutions”, CMCC.