3GPP TSG-RAN WG4 Meeting #116	R4-2510709

Bengaluru, India, 25th – 29th August 2025

Agenda item:	7.26.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM requirements for XR

Document for:	Discussion

- Introduction

The WI was revised at RAN#107[1] and WF was approved at RAN4#115 [2]. In this contribution, we further discuss the remaining RRM requirements for XR.

- Discussion

According to the latest WF [2], the open issues of RRM requirements are described as follows.

Topic 3 XR skipping requirements impact Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) × CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

During the previous meeting, several companies expressed concerns that Option 1 and Option 3 are not feasible from an implementation perspective. However, Option 2 applies the CSSFinter factor to the entire delay formula, including the Nskip term, which can result in excessively large measurement delays.

Therefore, we propose a modified version, Option 2a, with the following formula. This version introduces a cap on the maximum delay to prevent excessive scaling due to CSSFinter, ensuring the delay remains within acceptable limits:

Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip.

Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- Capped Delay = max(Original Delay, Delay Cap)

where Original Delay = max(200 ms, (8 + Nskip) × max(MGRP, SMTC period)) × CSSFinter

and Delay Cap is set to a reasonable upper bound (e.g., 1500 ms)

On the other hand, item measurement delay extension with DRX was not discussed during the previous meeting. Therefore, we would like to continue proposing our view from the last session. If the requirement is not feasible in perspective implementation, we also propose the capped delay in this case.

Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- DRX cycle ≤ 320 ms

  - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

- DRX cycle > 320 ms

  - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

- Conclusion

In this contribution, we propose RRM requirements for this WI as follows.

Observation 1: For measurement delay extension without DRX, the controversial point is whether the skipping factor should affect CSSFinter or not and to keep same number of measurement samples with skipping consideration, CSSFinter should be out of the effect of Nskip.

Proposal 1: Without DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- Capped Delay = max(Original Delay, Delay Cap)

where Original Delay = max(200 ms, (8 + Nskip) × max(MGRP, SMTC period)) × CSSFinter

and Delay Cap is set to a reasonable upper bound (e.g., 500 ms)

Proposal 2: With DRX case, RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

- DRX cycle ≤ 320 ms

  - Max(200 ms  CSSFinter, Ceil(8 1.5)  Max(MGRP, SMTC period, DRX cycle)  CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) )

- DRX cycle > 320 ms

  - 8  DRX cycle  CSSFinter +  Nskip  DRX cycle

References

[2] 3GPP, RP-240107, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia.

[2] 3GPP, R4-25082825, “WF on RRM requirements for XR_Ph3”, Nokia.