3GPP TSG-RAN WG4 Meeting #116	R4-2510713

# Bengaluru, India, August 13th – 17th 2025

Source:	Ericsson

Title:	Simulation results for Rel-19 SCM for demodulation

Agenda item:	7.12.3

Document for:	Information

# Introduction

In RAN4#115 meeting, comparison and alignment simulation assumptions are agreed [2], and the comparison metrics on channel properties and performance are also agreed [1]. This meeting would be the last meeting to make conclusions based on simulation results and analysis. In this contribution, we provide simulation results for those cases missed in previous meetings and also channel property results for CDL based models.

# Simulation

CDL model name explanation for alignment and comparison (See Annex A)

CDLC A4_12clusters: profile is derived from CDLC A2_20clusters with cluster truncation to 12 clusters. The spatial filter is based on AAV (1,1).

CDLC A2_20clusters: profile is derived from CDLC in TR38.901 including 20 clusters after cluster combination and without cluster truncation.

PMI hypothesis for PMI statistic histogram

Precoding matrix index i=0, 1, 2, …, 15,  where i1,1=modi, 8,i1,2=0 and i2=floor(i/8).

| PMID | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| i1,1 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| i1,2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| i2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Note: The histogram is taken from the high normalized throughput range, i.e., >=70%. The PMI with highest probability in histogram will be used for “fixed PMI” simulations.

## 2.1	SU PDSCH

### 2.1.1	Simulation parameters

Table 2.1.1-1: Common simulation assumptions for PDSCH

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| TDD Slot Configuration Pattern |  |  | 7D1S2U (PDSCH is not scheduled in S slot for Rank8) |
| Channel Bandwidth/SCS |  |  | 40MHz/30kHz |
| Antenna configuration |  |  | Rank4: 4T4R Rank8: 8T8R |
| MCS |  |  | 13 (64 QAM table) |
| Codebook configuration for PDSCH and DMRS |  | CodebookType | Specific to each channel model |
|  |  | Codebook configuration | 4T4R: (N1,N2,O1,O2) = (2,1,4,1) 8T8R: (N1,N2,O1,O2) = (4,1,4,1) |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | Resource allocation type |  | Type 0 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 2 for rank 8 1 for rank 4 |
| Number of HARQ Processes |  |  | 8 |
| Maximum HARQ transmissions |  |  | 4 |
| UE receiver type |  |  | MMSE-IRC |
| Test metric |  |  | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |

Table 2.1.1-2: Simulation assumptions for CDL channel

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | Rank4 | Rank8 |
| FR / Carrier frequency |  | FR1,3.5GHz |  |
| UE speed and movement direction |  | 3km/h, (65°,90°) |  |
| AAV assumptions |  | (M,N,P,Ms,Ns) = (1,2,2,1,1) | (M,N,P,Ms,Ns) = (1,4,2,1,1) |
| Codebook for DMRS and PDSCH |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination |
| Channel Geometry | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |  |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |  |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |
| Test metric | - Rank 4: SNR(dB)@ 30% and 70% of maximum throughput - Rank 8: SNR(dB)@30% and 70% of max throughput for each codeword |  |  |

### 2.1.2	Simulation results

| Figure 2.1.2-1 PMI preference and fixed PMI results for 8 layers PDSCH |  |
| --- | --- |

The simulation results are summarized in the following tables.

Table 2.1.2-1 Simulation results summary for SU-PDSCH with rank 8

| Channel model | PMI configuration | UE velocity / Doppler | CBW/SCS [MHz/kHz] | Per CW SNR [dB] |  |  |  | Total SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 30% |  | 70% |  | 30% | 70% |
|  |  |  |  | CW0 | CW1 | CW0 | CW1 |  |  |
| CDLC A4_12clusters | random | 3 km/h | 40 / 30 | 5.55 | 15.04 | 14.61 | 25.61 | 8.62 | 21.86 |
|  | random | 30 km/h | 40 / 30 | 4.24 | 15.65 | 15.32 | 28.54 | 8.02 | 24.74 |
|  | fixed_4_0_0 | 3 km/h | 40 / 30 | 5.43 | 17.88 | 9.42 | 20.06 | 6.94 | 18.84 |
|  | fixed_2_0_0 | 3 km/h | 40 / 30 | 4.18 | 24.87 | 10.37 | 27.01 | 9.21 | 25.34 |

Table 2.1.2-2 Simulation results summary for SU-PDSCH with rank 4

| Channel model | PMI configuration | UE velocity / Doppler | CBW/SCS [MHz/kHz] | Total SNR [dB] |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 30% | 70% |
| CDLC A4_12clusters | random | 3 km/h | 40 / 30 | 5.88 | 16.56 |
|  | random | 30 km/h | 40 / 30 | 5.83 | 17.17 |
|  | fixed_4_0_0 | 3 km/h | 40 / 30 | 6.08 | 15.12 |
|  | fixed_2_0_0 | 3 km/h | 40 / 30 | 6.50 | 16.70 |

## 2.2	SU PMI

### 2.2.1	Simulation parameters

Table 2.2.1-1: Common simulation assumptions for PMI

| Parameter |  |  | Unit | 8TX |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |
| Subcarrier spacing |  |  | kHz | 30 |
| Duplex Mode |  |  |  | TDD |
| TDD DL-UL configurations |  |  |  | 7D1S2U S=6D+4G+4U |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 8 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8, (4,6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8, (5) |
|  | CSI-RS interval and offset |  | slot | 10, 1 |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Not configured for eType II Wideband for Type I |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 11111111111111 |
| Codebook configuration | Codebook Type |  |  | typeII-r16 typeI-SP |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 7 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |
| Test metric |  |  |  | SNR@90% of maximum throughput with follow Type I PMI SNR@90% of maximum throughput with follow eType II PMI |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |

Other parameters for CDL channel please refer to Table 2.1.1-2.

### 2.2.2	Simulation results

(a)  TDLC300 with XP med corr rank 4                                         (b) TDLC300 with non-XP med corr rank 4

(c)  TDLC300 with XP med corr rank 2                                         (d) TDLC300 with non-XP med corr rank 2

(e)  CDLC A4 with rank 4                                                                          (f) CDLC A4 with rank 2

Figure 2.2.2-1	PMI results with 8Tx

(a)  CDLC A4 with rank 4 and 0.5λ AE separation                       (b) CDLC A4 with rank 2 and 0.5λ AE separation

(c)  CDLC A4 with rank 4 and 4λ AE separation                               (d) CDLC A4 with rank 2 and 4λ AE separation

(e)  CDLC A4 with rank 4 (0.5λ vs 4λ)                                               (f) CDLC A4 with rank 2 (0.5λ vs 4λ)

Figure 2.2.2-2	PMI results with 32Tx

All simulation results are summarized in following tables.

Table 2.2.2-1 Simulation results summary for PMI with 8Tx

| Channel model | AAV config. | AE separation (hor, ver) [lamda] | UE velocity / Doppler | PMI | Rank | CB config. | SNR [dB] |  | Gamma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 70% | 90% |  |
| CDLC A4_12clusters | AAV (1,1) | (0.5, 0.5) | 3 km/h | F | 4 | WB Type-I | 9.65 | 12.49 | 1.77 |
|  |  |  |  | F | 2 | WB Type-I | 0.2 | 2.64 | 2.43 |
|  |  |  |  | F | 4 | SB eType-II | 8.38 | 12.33 | 1.79 |
|  |  |  |  | F | 2 | SB eType-II | -0.75 | 1.53 | 2.9 |
| TDLC300 | XP medium correlation |  | 10 Hz | F | 4 | WB Type-I | 13 | 14.42 | 1.92 |
|  |  |  |  | F | 2 | WB Type-I | 0.76 | 1.91 | 3.1 |
|  |  |  |  | F | 4 | SB eType-II | 12.66 | 14.60 | 1.92 |
|  |  |  |  | F | 2 | SB eType-II | 0.49 | 1.68 | 3.16 |
|  | Non-XP medium correlation |  | 10 Hz | F | 4 | WB Type-I | - | - | - |
|  |  |  |  | F | 2 | WB Type-I | 11.37 | 13.58 | 2.21 |
|  |  |  |  | F | 4 | SB eType-II | - | - | - |
|  |  |  |  | F | 2 | SB eType-II | 10.79 | 12.72 | 2.30 |

Table 2.2.2-2 Simulation results summary for PMI with 32Tx

| Channel model | AAV config. | AE separation (hor, ver) [lamda] | UE velocity / Doppler | PMI | Rank | CB config. | SNR [dB] |  | Gamma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 70% | 90% |  |
| CDLC A4_12clusters | AAV (1,1) | (0.5, 0.5) | 3 km/h | F | 4 | WB Type-I | 4.22 | 6.92 | 4.63 |
|  |  |  |  | F | 2 | WB Type-I | -4.18 | -1.9 | 12.40 |
|  |  |  |  | F | 4 | SB eType-II | 1.59 | 4.87 | 7.59 |
|  |  |  |  | F | 2 | SB eType-II | -6.46 | -4.36 | 36.73 |
|  |  | (0.5, 4) | 3 km/h | F | 4 | WB Type-I | 4.58 | 7.03 | 3.48 |
|  |  |  |  | F | 2 | WB Type-I | -3.34 | -1.84 | 10.26 |
|  |  |  |  | F | 4 | SB eType-II | 0.72 | 2.41 | 12.83 |
|  |  |  |  | F | 2 | SB eType-II | -5.64 | -4.73 | 49.01 |

## 2.3	Channel properties analysis

Way Forward:

Utilise channel properties to compare across channel model candidates (Enhanced TDL and CDL) in each comparison case utilising the following properties, qualitative assessment discussions to be brought to RAN4#116 based on the following parameters:

- Power Angular/Phase Distribution in Tx and Rx Directions (including comments on stability and diversity)

- Time Coherence as channel correlation between symbols

- Frequency Coherence measured as channel correlation between subcarriers

Contributions will be assessed either offline between meetings or during RAN4#116 to subjectively decide whether channel models fit expectations of a spatial channel.

### 2.3.1	Time coherence

The general time correlation function R(Δt)  is given by the normalized inner product between channel vectors separated by a time lag Δt,  averaged over all valid time indices as

RΔt =E{h(t)h*(t + Δt)}E{h(t)h*(t )}E{h(t + Δt)h*(t + Δt)}

where h(t) is the vectorized channel at time t, and Δt=Δ TCSIRS with TCSIRS=5 ms is the transmission periodicity of the CSI-RS signal and Δ∈{0,1,2,…,1000}. The expectation E[⋅] is approximated by a sample average across all antenna pairs and frequency realizations.

The coherence time Tc is defined as the minimum time lag (in seconds) at which the average magnitude of the time correlation function drops below a specified threshold γ.

Tc=minΔt  RΔt|≤γ}.

In our simulations, we use γ=0.5.

(a) CDLC A4 with UE speed 3km/h, TC = 0.28s            (b) CDLC A4 with UE speed 30km/h, TC = 0.03s

(c) CDLC A2 with UE speed 3km/h, TC = 0.28s           (d) CDLC A2 with UE speed 30km/h, TC = 0.03s

(e) CDLC 901 with UE speed 3km/h, TC = 0.34s           (f) CDLC 901 with UE speed 30km/h, TC = 0.03s

(g) TDLC300 with UE speed 3km/h, TC = 0.2s           (h) TDLC300 with UE speed 30km/h, TC = 0.02s

Figure 2.3.1 Time coherence comparison

Table 2.3.1-1 Time coherence comparison results with threshold 0.5

| Time coherence (s) | CDLC A4 (12 clusters) | CDLC A2 (20 clusters) | CDLC 901 (24 clusters) | TDLC300 |
| --- | --- | --- | --- | --- |
| UE speed 3km/h | 0.28 | 0.28 | 0.34 | 0.2 |
| UE speed 30km/h | 0.03 | 0.03 | 0.03 | 0.02 |

### 2.3.2	Frequency coherence

To evaluate the frequency selectivity of the wireless channel, we compute the frequency correlation function (FCF) across subcarriers and extract the corresponding coherence bandwidth. The normalized frequency correlation function is calculated as

Rf(Δf)=E[hf⋅hf+Δf*]Ehf2⋅Ehf+Δf2

where h(f) is the channel frequency response at subband f. The expectation E[⋅] is approximated by a sample average across all antenna pairs and time realizations, and Δf is the frequency lag in PRBs. The coherence bandwidth fc is then defined as the frequency lag corresponding to Δf  at which the average magnitude of the FCF falls below a certain threshold γ.

fc=minΔf  |RfΔf|≤γ}.

In our simulations, we use γ=0.3.

(a) CDLC A4 with UE speed 3km/h, FC BW = 8 PRBs            (b) CDLC A4 with UE speed 30km/h, FC BW = 7 PRBs

(c) CDLC A2 with UE speed 3km/h, FC BW = 8 PRBs            (d) CDLC A2 with UE speed 30km/h, FC BW = 7 PRBs

(e) CDLC 901 with UE speed 3km/h, FC BW = 8 PRBs            (f) CDLC 901 with UE speed 30km/h, FC BW = 7 PRBs

(g) TDLC300 with UE speed 3km/h, FC BW = 7 PRBs            (h) TDLC300 with UE speed 30km/h, FC BW = 7 PRBs

Figure 2.3.2-1 Frequency coherence comparison

All results are summarized in the table below.

Table 2.3.2-1 Frequency coherence comparison results with threshold 0.3

| Frequency coherence BW (PRB) | CDLC A4 (12 clusters) | CDLC A2 (20 clusters) | CDLC 901 (24 clusters) | TDLC300 |
| --- | --- | --- | --- | --- |
| UE speed 3km/h | 8 | 8 | 8 | 7 |
| UE speed 30km/h | 7 | 7 | 7 | 7 |

### 2.3.3 	Power angular distribution

(a) CDLC A4 with AAV (1,4,2)                                                          (b) CDLC A4 with AAV (8, 8, 4)

Figure 2.3.3-1 PAD for CDLC A4 with different AAV

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2508776, Way Forward for [115][321] FS_NR_demod_SCM, Nokia

[2]	R4-2508622, Simulation assumptions for SCM, Huawei

# Annex A

Table A-1 CDL-C A2_20clusters

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 76 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 80 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 232 | 0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 240 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 290 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 300 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 340 | -14.1 | -73 | 43.1 | 92.3 | 50.8 |
| 9 | 448 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 10 | 476 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 11 | 792 | -11.7 | -60.2 | 79 | 91.7 | 51.7 |
| 12 | 988 | -16.2 | 64.6 | 26.3 | 105.2 | 121 |
| 13 | 1552 | -16.9 | 62.2 | -22.6 | 94 | 53.9 |
| 14 | 1678 | -16.9 | 55 | -1.6 | 91.8 | 61.6 |
| 15 | 2002 | -18.8 | -76.3 | -19.8 | 90.7 | 37.5 |
| 16 | 2044 | -20.1 | 57.4 | 4.5 | 108.2 | 53.1 |
| 17 | 2300 | -19 | 58.1 | 0.2 | 91.7 | 35.8 |
| 18 | 2420 | -18.7 | 67.3 | -4.9 | 91.6 | 52.9 |
| 19 | 2568 | -24.6 | 76.2 | -28.3 | 106.4 | 46.4 |
| 20 | 3154 | -25.8 | -91.4 | 29.3 | 109.5 | 44.7 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |

Table A-3 CDL-C A4_12clusters on AAV (1,1)

| Cluster # | Cluster delay (ns) | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -7.4 | -38.2 | -96.9 | 96.8 | 98.1 |
| 2 | 132 | -1.2 | -21.8 | 118.9 | 98.5 | 71.1 |
| 3 | 138 | -5.5 | -34.2 | -124.4 | 100.9 | 67.6 |
| 4 | 402 | 0.0 | -5.9 | 171.2 | 99.2 | 76.6 |
| 5 | 416 | -10.4 | 44.2 | 51.9 | 106.4 | 62.8 |
| 6 | 502 | -10.1 | -50.6 | 63.4 | 94.5 | 56.6 |
| 7 | 518 | -13.7 | 49.1 | -42.1 | 107.5 | 69.8 |
| 8 | 776 | -8.1 | -44.2 | 65.1 | 104.6 | 103.4 |
| 9 | 826 | -9.8 | -50.4 | -63.4 | 105.2 | 50.1 |
| 10 | 1372 | -11.7 | -60.2 | 79.0 | 91.7 | 51.7 |
| 11 | 1712 | -16.2 | 64.6 | 26.3 | 105.2 | 121.0 |
| 12 | 2906 | -16.9 | 55.0 | -1.6 | 91.8 | 61.6 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter |  | cASD in [°] | cASA in [°] | cZSD in [°] | cZSA in [°] | XPR in [dB] |
| Value |  | 1.4 | 15.6 | 3.6 | 12.2 | 7 |