3GPP TSG-RAN WG4 Meeting # 115	    R4-2510762

Bengaluru, IN, 25 – 29 August 2025

Agenda Item:	7.19.2

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

# Introduction

This contribution discusses general aspects related to the AI/ML aided mobility. The discussions presented in this paper addresses aspects related to modeling of error in measurements used for RRM prediction, and requirements for data collection for network sided model.

# Discussion

## Error Modelling

To predict the RRM measurement, the UE needs to perform measurements for model input. The accuracy of predicted measurements depends on the accuracy of the measurements provided as an input to the AI/ML model. This has been acknowledged by RAN4 in previous meetings where the following agreements were made.

Agreement from RAN4#114 [1]

- For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

WF from RAN4#114bis [2]

- Companies are encouraged to bring proposals on how to capture factors to be considered for error models to consider the impact of measurement errors on prediction accuracy.

The measurements performed by the UE are typically impacted by RF impairments and the baseband error. To evaluate the impact of error in measurements used as input to the AI/ML model on accuracy of the predicted measurement both factors need to be considered in the simulations. A truncated Gaussian distribution with zero mean and variance corresponding to the RF calibration error (±2.5dB for FR1 and ±4dB for FR2) for RSRP measurement can be considered to statistically characterize RF impairment error in RAN4 simulations. This approach is also aligned with the approach taken to evaluate the performance of AI/ML aided beam management [3]. The baseband error on the other hand refers to the estimation error. For example, error made by the UE when estimating the RSRP of the SSB signal received from a cell/beam. Baseband error depends on the signal strength, reference signal bandwidth, noise, interference, propagation condition etc. that are typically part of the RAN4 simulation assumptions. It is therefore our understanding and proposal that statistical modelling of baseband error is not required for RAN4 evaluations.

Proposal 1: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

- RF error in dB for FR1 is characterized by Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

- RF error in dB for FR2 is characterized by Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

## Data collection for network sided model

For AI/ML aided mobility, AI/ML model can also be deployed at the network side. The network in this scenario may configure UE to report the measurements for training and or inference purposes. As the performance of the AI/ML model at the network side relies on the accuracy of measurement reported by the UE, RAN4 should discuss to identify relevant requirements for the data collection procedure for the network sided model. Provided no new measurements are identified by RAN2, RAN4 should further discuss whether the existing requirements are applicable for data collection for network sided model. If new measurement is defined by RAN2, corresponding requirements for the new measurement shall be specified by RAN4.

Proposal 2: Provided no new measurements are identified by RAN2, RAN4 should further discuss whether the existing requirements are applicable for data collection for network sided model.

Proposal 3: If new measurement is defined by RAN2 for data collection for network sided model, requirements for the new measurement is specified by RAN4.

# Summary

In this contribution discussions on aspects related to modeling of error in input measurements used for RRM prediction, and requirements for data collection for network sided model are discussed. The proposals below summarize the discussion presented in this paper.

Proposal 1: Statistical model is used to only model RF error in the evaluations for impact of input measurement error on accuracy of predicted RRM measurements.

- RF error in dB for FR1 is characterized by Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

- RF error in dB for FR2 is characterized by Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 2: Provided no new measurements are identified by RAN2, RAN4 should further discuss whether the existing requirements are applicable for data collection for network sided model.

Proposal 3: If new measurement is defined by RAN2 for data collection for network sided model, requirements for the new measurement is specified by RAN4.

# References

R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

R4-2504901, WF on AI/ML Mobility, Moderator (Nokia).

R4-2508081, Updated simulation assumptions for AI/ML BM, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, HiSilicon, MediaTek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT.