3GPP TSG-RAN4 Meeting #116	R4-2510772

Bengaluru, India,  250825 - 250829

Source:	Ericsson

Title:	RRM core requirements for Ambient-IoT

Agenda item:	7.22.4

Document for:	Approval

# 1	Introduction

In RAN4#115 the following WF was approved [1]:

… ## 1.2 Sub-topic 1-2: timing Issue 1-2-1: Device transmit timing Agreement: - RAN4 define D2R transmit timing requirements. - Define D2R transmit timing error requirements as follows: - For MSG1 X=1, - The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1. - For MSG1 X=2, follow the same principle as starting point. - Note 1: The timing error requirements for other D2R could be updated accordingly based on further RAN1 agreements. - Note 2: The end of the R2D transmission and last edge could be updated based on further RAN1 agreements if any. Issue 1-2-2: Device receive timing FFS: - Option 1 (ZTE): RAN4 shall consider accuracy requirements on TD2R_min if the range of it depends on RAN1. …

In this contribution we further discuss proposals in relation to issues 1-2-1 and 1-2-2.

# 2	Discussion

## 2.1	Device transmit timing

For MSG 1, X=1 we have:

Figure 1: Messages for X=1 case.

The issue of MSG1 X=1 was analyzed in [2]:

Figure 2: Time error for D2R transmission (Fig 2 in [2])

An the conclusion was, as stated in WF [1]:

- Define D2R transmit timing error requirements as follows:

- For MSG1 X=1,

- The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

For MSG1 X=2 we have:

Figure 3: Messages for X = 2 case.

The agreement from WF [1] that for MSG1 X=2, we shall follow the same principle as starting point is fine. That is the best we can do. The definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1, is still valid, as before, but for the next MSG1.

Observation 1: The definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1, is still valid, as before, but for the second MSG1.

However, as indicated in Figure 3 above, if SFO is the Sample Frequency Offset of 10%, then the end of the first MSG1 transmission, can be SFO⋅TMSG1 late and the start of the second MSG1 can be SFO⋅TMSG1 early. This means that we need to guarantee MSG1 to be scheduled in such a way that they have 2⋅SFO⋅TMSG1 separation.

Observation 2: we need to guarantee MSG1 to be scheduled in such a way that they have 
2⋅SFO⋅TMSG1 separation between nominal end of first MSG1 and nominal start of second MSG1 transmission.

Observation 3: The definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1, is still valid, as before, but for the second MSG1. We can discuss exact syntax if overloading of meaning is not the preferred way.

From figure 1 we can also deduce that we can reuse the same proposals also for MSG3 transmissions from the device. This means that we have definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG3 as well.

This can be summarized as

For MSG1 X = 1 or X = 2

The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Proposal 1:

For MSG1 X = 1 or X = 2

The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

## 2.2	Device receive timing

FFS: - Option 1 (ZTE): RAN4 shall consider accuracy requirements on TD2R_min if the range of it depends on RAN1.

Recall the definition of  TD2R_min in TS38.769 in the below.

A-IoT processing time aspects are studied in terms of the following timing relationships: TR2D_min: Minimum time between a R2D transmission and the corresponding D2R transmission following it. TD2R_min: Minimum time between a D2R transmission and the corresponding R2D transmission following it. TD2R_max: Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device. See clause 6.3 for message descriptions.

TD2R_min is defined to establish the minimum time duration between a device's data transmission and its readiness to receive.

This parameter primarily defines the switching timing—that is, the minimum time window required for the Ambient IoT device to transition from transmission mode to reception mode. Additionally, it aids network-side scheduling by indicating when the reader should expect a response from the device.

During the last RAN1 meeting, it was agreed that RAN4 shall define the value(s) of TD2R_min.

Agreement It is up to RAN4 to define the value(s) for TD2R_min. - Note: RAN1 expects that the value(s) for TD2R_min will be defined in RAN4 specifications

A smaller TD2R_min enables the network to schedule R2D transmission more quickly. However, a larger TD2R_min may simplify UE implementation. Therefore, RAN4 should define a value (or range) of TD2R_min that balances network efficiency with device complexity.

Proposal 2: RAN4 to define the requirement of TD2R_min.

The device’s readiness to receive can be treated as the range  (TD2R_min -TD2R_offset , TD2R_min +TD2R_offset ) with considering accuracy TD2R_offset,

Among them, TD2R_min -TD2R_offset  actually is harmless, but TD2R_min +TD2R_offset is more critical. Too long TD2R_offset , e.g., too later readiness may lead to difficulty of NW scheduling, e.g., handling multiple Ambient-IoT devices. Apparently, it’s expected that the accuracy of TD2R_min  is specified.

Another concern to the accuracy of TD2R_min  is CW. Since CW is provided to the device during D2R transmission, it shall be shut down after D2R transmission, and at least before R2D transmission. If TD2R_min is inaccurate to some level, an ongoing CW may cause interference between R2D and D2R.

Proposal 3: RAN4 to define accuracy requirements for TD2R_min, which may take Te in Toffset as baseline.

Proposal 4: TD2R_min shall be considered valid only when CW is turned OFF.

# 3 	Summary

Observation 1: The definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1, is still valid, as before, but for the second MSG1.

Observation 2: we need to guarantee MSG1 to be scheduled in such a way that they have 
2⋅SFO⋅TMSG1 separation between nominal end of first MSG1 and nominal start of second MSG1 transmission.

Observation 3: The definition of TSFO, the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1, is still valid, as before, but for the second MSG1. We can discuss exact syntax if overloading of meaning is not the preferred way.

Proposal 1:

For MSG1 X = 1 or X = 2.

The reference point for the device transmission timing is Toffset1 after the end of corresponding R2D transmission as define in TS 38.219. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last edge of the corresponding R2D transmission to the starting time of MSG1.

Proposal 2: RAN4 to define the requirement of TD2R_min.

Proposal 3: RAN4 to define accuracy requirements for TD2R_min, which may take Te in Toffset as baseline.

Proposal 4: TD2R_min shall be considered valid only when CW is turned OFF.

# References

[1] 		R4-2508275, WF on RRM requirements for Ambient_IoT_Solutions, CMCC

[2]		R4-2507120, Discussion on RRM impacts for R19 A-IoT device, Huawei, HiSilicon