3GPP TSG-RAN4 Meeting #116	R4-2510775

Bengaluru, India,  250825 - 250829

Source:	Ericsson

Title:	Draft LS reply regarding CB-msg3-EDT

Agenda item:	7.28.4

Document for:	Approval

# 1	Introduction

RAN2 has forwarded an LS to RAN4 (and RAN1) [1]. In this contribution we analyze the question and propose a draft LS response.

# 2	Discussion

The LS from RAN2 reads as follows [1]:

# 1. Overall Description: During RAN2#130 meeting, RAN2 discussed CB-msg3-EDT and achieved the following agreements regarding HARQ feedback for MSG4: - Agreements regarding HARQ feedback for MSG4 (i.e., CB-msg3 response): - The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern. - Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK. - For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3. - Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. - Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation. Besides, RAN2 also made the following agreements regarding TAC in MSG4: - Agreements regarding TAC in MSG4 (i.e., CB-msg3 response): - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation). RAN2 would like to check following: - whether there is any concern from RAN1 on the agreements regarding HARQ feedback for MSG4 - whether there is any concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4 # 2. Actions: # To RAN1 and RAN4: ACTION: RAN2 respectfully asks RAN1 and RAN4 to provide feedback if any and take the above information into account in the specification work if necessary. # 3. Date of Next RAN2 Meetings: RAN2#131 25th August – 29th August 2025 Bangalore, India RAN2#131bis 13th October – 17th October 2025 Prague, Czech

As the remaining timing error is less than 97*TS (TS 36.133 sections 7.20A and 7.24A) and it can be in negative or positive direction, to bring the error to an error less or equal to Rel-18 NTN in connected mode, it is sufficient to be able to signal TA commands in the range [-96:16:96] *Ts (or [-112:16:128] *Ts using 4 bits or reusing the 6 bit TAC in connected mode [-496:16:512]*Ts).

Please find a draft LS in the appendix.

# 3 	Summary

Proposal 1: Agree draft LS text, as per appendix.

# References

[1] 		R2-2504776, LS on CB-msg3-EDT, HUAWEI

=== Annex ===

# Title:	Reply LS on CB-msg3-EDT

# Response to:	-

# Release:	Rel-19

# Work Item:	IoT_NTN_Ph3-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:	-

Contact Person:

Name:	Magnus Larsson

E-mail Address:	magnus dot k dot larsson at Ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

# 1. Overall Description:

During RAN4#116 meeting, RAN4 the discussed CB-msg3-EDT LS from RAN2 [1] 

RAN2 made the following agreements regarding TAC in MSG4:

- Agreements regarding TAC in MSG4 (i.e., CB-msg3 response): - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

RAN2 asks RAN4 whether there is any concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4.

As the remaining timing error is less than 97*Ts (TS 36.133 sections 7.20A and 7.24A) and it can be in negative or positive direction, to bring the error to an error less or equal to Rel-18 NTN in connected mode, it is sufficient to be able to signal TA commands in the range [-96:16:96] *Ts (or [-112:16:128] *Ts using 4 bits or reusing the 6 bit TAC in connected mode [-496:16:512]*Ts).

Based on the analysis above RAN4 has no concern with regards to the agreements regarding TAC in MSG4 (i.e., CB-msg3 response).

# 2. Actions:

# To RAN2 and RAN1:

ACTION: RAN4 respectfully asks RAN2 and RAN1 to take above information into account in the specification work if necessary.

# 3. Date of Next RAN4 Meetings:

RAN4#116bis	13 October 2025 – 17 October 2025				Prague, Czech Republic

RAN4#117	17 November 2025 – 21 November 2025		Dallas, USA