# 3GPP TSG RAN WG4 Meeting #116	 																	R4-2510785

# Bengaluru, India, 25 Aug. – 29 Aug. 2025

Agenda Item:	7.25.4

Source: 	Ericsson

Title:	Initial discussion on Rel-19 mobility RRM performance

Document for:	Discussion

# Introduction

After RAN plenary in September 2024, the following objective has been agreed in the WID [1] for Mobility work item.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1]

  - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

NOTE: 	RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

- Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we will address several aspects regarding the test case design for mobility RRM requirement.

# Discussion

## Event triggered L1 measurement reporting

Our understanding of the use case of the RAN2 introduced events is as follows:

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

These RAN2-agreed events are designed to request candidate cell Level 1 measurement reports to enable early DL/UL synchronization on candidate cells or trigger an LTM cell switch. We recommend testing at least one event intended for cell switching and one for early TCI state activation, specifically Event LTM3 and Event LTM4.

- At least Event LTM3 and Event LTM4 be included in the RRM testcase.

RS Considerations for Testing: Both SSB-based and CSI-RS-based event-triggered reporting delays, as defined by RAN2, should be tested.

Measurement accuracy:

No filtering is agreed upon, and hence we expect the measurement accuracy to be the same as the legacy. We do not see a need for testing measurement accuracy or introducing new accuracy requirements.

Test cases:

In the context of LTM Release 18, the following test cases have been agreed upon for SSB-based L1-RSRP measurement. To optimize discussion time, we propose leveraging established test case coverage. Moreover, to minimize the number of tests required, we suggest configuring both SSB and CSI-RS for event-triggered reporting within the same test case wherever possible. This approach aims to consolidate testing efforts while ensuring comprehensive evaluation.

- RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP.

List of test cases agreed in Rel-18:

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

## CSI-RS based L1 measurement

Accuracy Requirements:

Given that the SNR side conditions for the neighbor cell CSI-RS-based L1-RSRP are the same as those for the serving cell CSI-RS-based L1-RSRP, we propose reusing the measurement accuracy criteria established for the serving cell CSI-RS-based L1-RSRP.

- We suggest that RAN4 agree to adopt the same measurement accuracy for the neighbor cell CSI-RS-based L1-RSRP as is used for the serving cell L1-RSRP accuracy.

Test Cases:

For CSI-RS-based L1-RSRP, we have agreed not to specify requirements involving a measurement gap and RTD > CP. Therefore, we propose the following list of test cases based on the Rel-18 LTM test cases:

- We recommend that RAN4 agree on the following list of test cases for NW-scheduled reporting of CSI-RS-based L1-RSRP.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

## Intra-CU conditional LTM

Scenarios

When Conditional LTM is introduced in Rel-19, RAN4 agreed the conditional LTM can be triggered based both on L1 and L3 measurement. The L1 measurement can be based on SSB or CSI-RS.

- RAN4 to define the test case to verify both SSB based L1 measurement , CSI-RS based L1 measurement triggered CLTM and SSB based L3 measurement triggered LTM.

At RAN4 # 112 bis meeting,  the applicable scenario for CLTM  was agreed to consider NR SA scenario defined in Rel-18 LTM

- PCell switch to a neighboring LTM candidate cell

- FR1 cell to FR1 cell

- FR1 cell to FR2 cell

- FR2 cell to FR2 cell

- FR2 cell to FR1 cell

- PCell switch to an LTM candidate cell that is a serving SCell in MCG

- FR1 cell to FR1 cell

- FR2 cell to FR2 cell

The conditional LTM  handover from FR1 Pcell to FR1 neighbouring LTM candidate or a candidate that is a serving Scell will be different on the test setup, but the delay requirement will be the same. From simplification test case point of view, the two test scenarios can be merged into one.

- The applicable test scenario for Rel-19 CLTM can be designed for below 4 sub-scenarios:

- FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

- FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

- Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

- FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

Test methodology and configuration

At RAN4 # 114 meeting, the CLTM delay requirement was agreed to take the conditional handover as baseline due to the similarity between the CHO and CLTM. The CHO and CLTM are both based on the UE evaluating preconfigured handover/LTM conditions to trigger the event and UE executing the condition for target cell switch.

However, the UE procedure to evaluate the candidate and execute on the target cell for CLTM is different from CHO.

For example, the UE candidate evaluation for CLTM can be based on either L1 or L3 measurement while the UE only evaluates CHO candidate based on L3 measurement. The LTM test case however was designed based on both L3 reporting for candidate and L1-RSRP for the target cell.

- RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure.

In CLTM, the UE will receive a pre-configured execution condition for the candidate cell. When the event being triggered for certain candidate cell, the UE will execute on this candidate. Agreement on RAN4#114,  the ending point of the CLTM will be based on whether it is RACH based or RACH-less CLTM. This is the same as the LTM procedure that both the RACH based and RACH-less cell switch are supported based on whether UE has valid TA value.  For CHO, the RACH-less handover is not supported. This is another difference between the CLTM and CHO.

The conditional handover test timeline have only T1 and T2, T1 is the time from UE received the RRC reconfiguration message until the event being fulfilled while the T2 is the handover interruption time. But the LTM test timeline has 4 time periods. The T4 represents the interruption time. The difference can be seen from figure 1.

Figure 1: test timeline between CHO and LTM

- RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure.

# Summary

The following have been proposed in the current contribution:

- At least Event LTM3 and Event LTM4 be included in the RRM testcase.

- RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP.

- Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- We suggest that RAN4 agree to adopt the same measurement accuracy for the neighbor cell CSI-RS-based L1-RSRP as is used for the serving cell L1-RSRP accuracy.

- We recommend that RAN4 agree on the following list of test cases for NW-scheduled reporting of CSI-RS-based L1-RSRP.

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

- RAN4 to define the test case to verify both SSB based L1 measurement , CSI-RS based L1 measurement triggered CLTM and SSB based L3 measurement triggered LTM.

- The applicable test scenario for Rel-19 CLTM can be designed for below 4 sub-scenarios:

- FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

- FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

- Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

- FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

- RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure.

- RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.