3GPP TSG-RAN WG4 Meeting # 116	R4-2510797

# Bengalore, India,  25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CSI-RS based radio link monitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update the L1in, L1outand L1in,max and L1out,maxvalue for CSI-RS based RLM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add L1in,max and L1out,max value Add X which is the maximum consecutive invalid meausremnet occasion Add UE behavior when L1in> L1in, max or X ≥ M Details can be seen from R4-2510792 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI-RS based RLM evaluation during SBFD operation is not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change 1>

#### 8.1.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS ms period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS ms evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-2 for FR2 with scaling factor N=1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-3 for FR1 (deactivated PSCell).

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-4 for FR2 (deactivated PSCell) with scaling factor N=1.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For the value of L1Out and L1In,

1>	If the UE does not support SBFD,

2>	 L1Out = 0;

2>  L1In = 0;

1>	Else if the UE supports SBFD,

2>	 If DRX is not configured,

3> L1Out is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3> L1In is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS, when DRX is not configured

2>	 If DRX is configured,

3>	 L1Out is the number of DRX cycles in which at least one occasion of the CSI-RS resource for RLM is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3>	 L1In is the number of DRX cycles in which at least one occasion of the CSI-RS resource for RLM is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS;

If the UE supports SBFD, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

When L1in> L1in, max, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers.

For the value of  X,

1>	If the UE does not support SBFD,

2>	 X= 0;

1>	Else if the UE supports SBFD,

2>	 If DRX is not configured,

3> X is the number of consecutive occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS  or TEvaluate_In_CSI-RS;

2>	 If DRX is configured,

3>	 X is the number of consecutive DRX cycles in which at least one occasion of the CSI-RS resource for RLM is overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS or TEvaluate_In_CSI-RS;

If the UE supports SBFD, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

When X ≥ M, layer 1 of the UE shall not send any in-sync indication for the cell to the higher layers.

For a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrentMeasGap-r17 and musim-GapPreference-r17, and when concurrent measurement gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are are configured,

-	an RLM-RS resource occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clauses 9.1.8 and 9.1.10,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion within the window W, and

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any non-dropped GAP occasion, nor non-dropped MUSIM gap occasion, nor any SMTC occasion within the window W, and

-	an RLM-RS resource occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

-	TL1 is periodicity of the target RLM-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s), concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs] are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is not overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the RLM-RS resource is not overlapped with GAP and RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the RLM-RS resource outside GAP is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

If the UE is configured with Pre-MG only, an RLM-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured , an RLM-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

NOTE:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

When the configured aperiodic MUSIM gap is overlapping with RLM-RS resource occasion, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if RLM-RS resource occasions are fully overlapped with MUSIM gap(s) or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based RLM.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of Mout and Min used in table 8.1.3.2-1, table 8.1.3.2-2, table 8.1.3.2-3 and table 8.1.3.2-4 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

If the UE supports SBFD, the requirements in this clause apply provided that the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((Mout +L1Out)×P)×TCSI-RS) | Max(100, Ceil((Min + L1In)×P) × TCSI-RS) |
| DRX ≤ 320 ms | Max(200, Ceil(1.5×(Mout+ L1Out)×P)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×(Min+ L1In)×P)× Max(TDRX, TCSI-RS)) |
| DRX > 320 ms | Ceil((Mout+ L1Out)×P) × TDRX | Ceil((Min+ L1In)×P) × TDRX |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE2: When DRX is not configured, Lin is the number of RLM-RS CSI-RS occasions which are not available at the UE during TEvaluate_in_CSI-RS, where L1in ≤ L1in, max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS CSI-RS occasion is not available at the UE during TEvaluate_in_CSI-RS, where L1in ≤ L1in, max. NOTE3: When DRX is not configured, Lout is the number of RLM-RS CSI-RS occasions which are not available at the UE during TEvaluate_out_CSI-RS, where L1out ≤ L1out, max. When DRX is configured, Lout is the number of DRX cycles in which at least one RLM-RS CSI-RS occasion is not available at the UE during TEvaluate_out_CSI-RS, where L1out ≤ L1out,max. NOTE4: L1in,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX L1in,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms L1in,max=5 for TDRX > 320ms NOTE5: L1out,max=20 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX L1out,max=15 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms L1out,max=10 for TDRX > 320ms |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((Mout+ L1Out)×P×N)×TCSI-RS) | Max(100, Ceil((Min+ L1In)×P×N) × TCSI-RS) |
| DRX ≤ 320 ms | Max(200, Ceil(1.5×(Mout+ L1Out)×P×N)× Max(TDRX, TCSI-RS)) | Max(100, Ceil(1.5×(Min+ L1In)×P×N)× Max(TDRX, TCSI-RS)) |
| DRX > 320 ms | Ceil((Mout+ L1Out)×P×N) × TDRX | Ceil((Min+ L1In)×P×N) × TDRX |
| NOTE1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE2: When DRX is not configured, Lin is the number of RLM-RS CSI-RS occasions which are not available at the UE during TEvaluate_in_CSI-RS, where L1in ≤ L1in, max. When DRX is configured, Lin is the number of DRX cycles in which at least one RLM-RS CSI-RS occasion is not available at the UE during TEvaluate_in_CSI-RS, where L1in ≤ L1in, max. NOTE3: When DRX is not configured, Lout is the number of RLM-RS CSI-RS occasions which are not available at the UE during TEvaluate_out_CSI-RS, where L1out ≤ L1out, max. When DRX is configured, Lout is the number of DRX cycles in which at least one RLM-RS CSI-RS occasion is not available at the UE during TEvaluate_out_CSI-RS, where L1out ≤ L1out,max. NOTE4: L1in,max=10 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX L1in,max=8 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms L1in,max=5 for TDRX > 320ms NOTE5: L1out,max=20 for Max(TDRX,TCSI-RS) ≤ 40ms where TDRX=0 for non-DRX L1out,max=15 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms L1out,max=10 for TDRX > 320ms |  |  |

Table 8.1.3.2-3: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P) × measCyclePSCell | Ceil(Min×P) × measCyclePSCell |
| DRX ≤ 320 ms | Ceil(1.5 ×Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(1.5 ×Min×P) × Max(TDRX, measCyclePSCell) |
| DRX > 320 ms | Ceil(Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(Min×P) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

Table 8.1.3.2-4: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P×N) × measCyclePSCell | Ceil(Min×P×N) × measCyclePSCell |
| DRX ≤ 320 ms | Ceil(1.5 × Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(1.5 × Min×P×N) × Max(TDRX, measCyclePSCell) |
| DRX > 320 ms | Ceil(Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(Min×P×N) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

### <End of Change 1>