3GPP TSG-RAN WG4 Meeting #116	   R4-2510802

# Bangalore, India, 25th – 29th , August , 2025

Agenda Item:	7.25.3.3

Source: 	Ericsson

Title:	Discussion on general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September 2024, the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

For RAN4#115 meeting, many open issues are listed in the WF[2]. In this contribution, we will address several aspects which are related to RRM requirements for conditional LTM.

# Discussion

## Conditional LTM delay

Issue 1-2-2: Tmeasure < Agreement > In C-LTM delay, Tmeasure is measurement time. It is defined from the end of TEvent_DU until UE starts TCLTM-RRC-processing to a target cell. For SSB based L1-RSRP measurement: - Tmeasure is SSB based L1-RSRP measurement period in R18 LTM plus SSB index acquisition time if needed. - Measurement latency for both serving and candidate cells need to be considered, and follow the definition of measurement delay in the event-triggered L1 reporting delay discussed under the corresponding agenda item. For SSB based L3-RSRP measurement: - Reuse Tmeasure in CHO. For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS. For CSI-RS based L3-RSRP measurement: - RAN4 does not define CLTM requirements for the case where CSI-RS based L3-RSRP measurements are used in the execution condition evaluation.

Firstly we would like to discuss whether it is necessary for UE to acquire SFN information for a known target cell for the CSI-RS based L1-RSRP measurement. The TCSI-RS_SFN_intra was the legacy CSI-RS based L3-RSRP measurement component and the SFN typically will be needed when UE is configured with mobility CSI-RS and associated with the SSB. For UE to derive the mobility CSI-RS, the UE needs to acquire the SFN and detect the PSS/SSS accordingly. This is different from CSI-RS based L1-RSRP measurement for example CSI-RS resource configured for L1-RSRP typically is the NZP-CSI-RS resource set instead of the mobility CSI-RS.

- Observation 1:  SFN acquisition time TCSI-RS_SFN_intra is specified for L3-RSRP when UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB  which specified in TS 38.133 9.10.2.5.

A known cell typically represents that UE have detect this cell, also being able to maintain coarse synchronization of the cell, regardless of the side condition details varies. When UE needs to acquire SFN typically means the UE doesn’t have the frame number for coarse synchronization. We find the terms ‘acquire SFN information’ technically contradictory with the term ‘known target cell’.

- Proposal 1: Remove the TCSI-RS_SFN_intra from the Tmeasure  for CSI-RS based L1-RSRP measurement.

To notice that Rel-19 mobility work item have parallel work on going to specify what is the side condition for a known cell based on endorsed draft CR R4-2508396. We think it is more reasonable to follow the same definition of known cell then elaborate more details for Tmeasure specified for CLTM.

Details on wording change can be seen from our draft CR R4-2510803

- Proposal 2: Further discuss the Tmeasure  definition for CSI-RS based L1-RSRP which is reused from the known target cell side condition specified in the objective CSI-RS based L1 measurement.

# Summary

The following have been observed in the current contribution:

	Observation 1:  SFN acquisition time TCSI-RS_SFN_intra is specified for L3-RSRP when UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB  which specified in TS 38.133 9.10.2.5.

The following have been proposed in the current contribution:

	Proposal 1: Remove the TCSI-RS_SFN_intra from the Tmeasure  for CSI-RS based L1-RSRP measurement.

	Proposal 2: Further discuss the Tmeasure  definition for CSI-RS based L1-RSRP which is reused from the known target cell side condition specified in the objective CSI-RS based L1 measurement.

# References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.

- R4-2518283 “WF on RRM requirements for NR_Mob_Ph4_Part2”, China Telecom, May, 2025