3GPP TSG-RAN WG4 Meeting #116																		R4-2510804

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.2

Source: 	OPPO

Title: 		Discussion on general aspects on AI/ML for NR air interface

Document for:	Approval

- Discussion

Delay Requirements for inference/monitoring reporting:

According to our understanding, the delay requirement for inference/monitoring reporting should be considered. Since the UE need to conduct both the measurement and AIML based processing before reporting, RAN4 should account for these two factors when defining the reporting delay requirement. In RAN1, according to the latest agreement(RAN1#121), for UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively. Here, d and d’ are needed for AIML based processing delay and detailed values of d and d’ will be further discussed in UE feature.

Besides, for AI/ML based predictive use cases (e.g., time-domain CSI prediction, time-domain beam prediction), the reporting delay requirements might just ensure that the predicted information is available by the target time(e.g., time to use predicted CSI, time to use predicted beam). For example, if a UE uses input data (e.g., reference signals) at time T1 to predict target information (e.g., channel state or beam information) that is to be used at time T2, the delay for reporting such information can be relaxed. The UE does not need to complete reporting immediately, it only needs to meet the T2 deadline.

Proposal 1: Delay requirements for inference/monitoring reporting are necessary. Both measurement delay and AI/ML based processing delay before reporting should be included when defining reporting timelines. Detailed delay values for AI/ML processing remain FFS (depending on RAN1 agreements, e.g., on related UE capabilities).

Proposal 2: For AI/ML-based predictive use cases(time-domain CSI prediction, time-domain beam prediction), the reporting delay can be relaxed, i.e., when the prediction time span long, UE does not need to complete reporting immediately.

Delay Requirements for activation/deactivation:

Regarding the delay requirement for activation, RAN4 should consider it to prevent performance degradation caused by overly slow activation of AI/ML functionalities/models. Detailed delay value for model activation remains FFS. As for the delay requirement for deactivation, we think that RAN4 does not need to define it. UE can autonomously execute deactivation of AI/ML functionalities/models when deemed necessary(or be indicated by NW).

Proposal 3: RAN4 should define activation delay to prevent performance degradation caused by overly slow activation of AI/ML functionalities/models.

Proposal 4: RAN4 does not need to define delay requirements for deactivation.

Delay Requirements for switching

When defining delay requirement for model switching (e.g., from Model A to Model B), it is necessary to consider whether the process requires deactivating Model A before activating Model B, or if Model B can be activated directly, e.g.,

- Case1: under constrained computing resources, if insufficient resources are available to activate Model B immediately, the switching process may require first deactivating Model A to free up sufficient computing capacity before activating Model B, introducing additional timing constraints.

- Case2: With sufficient computing resources, Model A deactivation and Model B activation can occur simultaneously, minimizing switching delay.

When specifying delay requirements for model switching, above two cases should be taken into account.

Proposal 5: When specifying delay requirements for model switching, following two cases should be taken into account:

Case1: Under constrained computing resources, if insufficient resources are available to activate Model B immediately, the switching process may require first deactivating Model A to free up sufficient computing capacity before activating Model B, introducing additional timing constraints.

Case2: With sufficient computing resources, Model A deactivation and Model B activation can occur simultaneously, minimizing switching delay.

Performance monitoring:

For performance monitoring, stability of the performance monitoring and decision-making mechanism should be considered in RAN4, to mitigate the impact of random effects on monitoring outcomes, includes: (1) obtaining a consistent monitoring result by considering multiple evaluating samples within an evaluation window, (2) assessing whether model monitoring should be handled at the UE level or the cell level.

When it comes to UE-level monitoring, each user must evaluate and make independent monitoring decisions regarding the utilization, updating, switching, and fallback of AI/ML models. This approach offers flexibility to users, but at the cost of preventing the sharing of evaluation and monitoring results among different users. On the other hand, cell-level monitoring assesses whether the current AI/ML model can be utilized, updated, switched, or fallback on a broader scale. The advantage of this approach is that when a significant number of users within a cell found that the current AI/ML model is no longer suitable, the evaluation and decision outcomes can be applied to other users in the same cell. This streamlines the process, saving monitoring time and effort for users who no longer need to individually identify and address potential performance issues with their respective models.

Proposal 6: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Delay requirements for inference/monitoring reporting are necessary. Both measurement delay and AI/ML based processing delay before reporting should be included when defining reporting timelines. Detailed delay values for AI/ML processing remain FFS (depending on RAN1 agreements, e.g., on related UE capabilities).

Proposal 2: For AI/ML-based predictive use cases(time-domain CSI prediction, time-domain beam prediction), the reporting delay can be relaxed, i.e., when the prediction time span long, UE does not need to complete reporting immediately.

Proposal 3: RAN4 should define activation delay to prevent performance degradation caused by overly slow activation of AI/ML functionalities/models.

Proposal 4: RAN4 does not need to define delay requirements for deactivation.

Proposal 5: When specifying delay requirements for model switching, following two cases should be taken into account:

Case1: Under constrained computing resources, if insufficient resources are available to activate Model B immediately, the switching process may require first deactivating Model A to free up sufficient computing capacity before activating Model B, introducing additional timing constraints.

Case2: With sufficient computing resources, Model A deactivation and Model B activation can occur simultaneously, minimizing switching delay.

Proposal 6: Stability of performance monitoring and decision-making mechanism should be considered to mitigate the impact of random effects on monitoring outcomes.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2