3GPP TSG-RAN WG4 Meeting #116																		R4-2510806

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.17.4

Source: 	OPPO

Title: 	RRM core requirement and testing framework for beam management

Document for:	Approval

- Discussion

Regarding the testing framework on BM, from our understanding, following aspects need to be considered.

- The number of FR2 beams that RAN4 tests can support remains unclear. It is necessary to clarify the limitations regarding the FR2 beams or probes that TE vendors could support. Based on this limitation(if any), RAN4 can further consider how to reach a consensus on designing/conducting tests for BM use cases and how to set/form beam set A and B of different sizes for different BM cases.

- A potential test setup could be (as shown in Figure 1)

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

As agreed in RAN4 #114 meeting, we can start with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Figure 1 A potential test setup for BM tests

- Besides, it's also crucial to further consider that, as a data/scenario-driven solution, the chamber and beam patterns provided by different TEs may impact the test results of BM (e.g., due to variations in beam settings, different considerations on multipath, and other implementations). Besides, even for a same TE, different DUT hardware implementations may also lead to variations in receiving behavior and beam measurement results. Attention must be paid to ensure that the BM model constructed on the DUT side(e.g. by corresponding training dataset) can match(or approximate match) and be utilized in the chamber environment on the TE side. Otherwise, it becomes challenging for a DUT to pass the test under unknown chamber conditions/scenarios. This may require a close collaboration between DUT and TE to ensure that the model under test is compatible with the testing environment.

- In RAN4 #114 meeting, RAN4 agreed to use CDL-based channel model as starting point. Following issues should be taken into account when identifying the BM test settings, 1) to model correlation between the measurements of different beams to ensure that the beams remain predictable, 2) the predictability of the beams must not be too strong, as an extreme case where there is no distinction between different beams would undermine the necessity for BM testing, 3) the test should not be overly complex and should optimally utilize the existing test environment.

Proposal 1: For BM testability, RAN4 needs to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 2: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Regarding the TCI state issue, in RAN4 #113 meeting, we have got agreements as below:

- Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS

- RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not.

- Other way to know UE Rx beams is not precluded

- UE capability is one of the solutions

Based on our understanding, for the BM test, the TCI state associated with a predicted Tx beam is better to be known to DUT. Otherwise, it is difficult to evaluate the performance uncertainty due to different RX beam assumptions and implementation differences among different UEs. To address how DUT can determine an optimal receiving beam, a beam pair (TX-RX pair) matching process may have to be introduced into the test procedure to ensure that the DUT has opportunities to obtain the TCI state associated with a predicted Tx beam.

In RAN4 #115 meeting, regarding Rx beam knowledge and TCI handling, we have got agreements as below:

Agreement:

    - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known.

    - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown.

    - Known Rx beam means no additional Rx beam sweeping is needed

    - Unknown Rx beam means additional Rx beam sweeping is needed.

    - Rx beam is known mean TCI state is known if conditions below are also met

The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis.

  - The target TCI state is known when the following is met:

    - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

    - Applies for case 1

    - FFS for case 2

    - Whether RS resource is only from set B should be clarified

      - FFS on whether prediction also impacts if a TCI state is known or unknown

        - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown

        - FFS on the time delay if prediction is used as a condition for a known TCI state

According to our understanding, for both BM-Case1 and BM-Case2, it can be defined that " the target TCI state is known when the TCI state switch command is received within 1280 ms after the last transmission of the reference signal (RS) resource for beam reporting or measurement, and the UE has sent at least one L1-RSRP report for the target TCI state".

- For BM-Case1, as illustrated in Figure 2(a), similar to the non-predictive scheme, if the Rx beam for the target TCI state is known at time T1, then the target TCI state can be considered known at time T2 (1280 ms later).

- For BM-Case2, as shown in Figure 2(b), the key difference from BM-Case1 is that the L1-RSRP report submitted at time T1 contains the predicted BM information for time T3. Here, there could be two different interpretations: (1) the 1280 ms calculation starts from T1, or (2) the 1280 ms calculation starts from T3. In BM-Case2, the actual measurement occurs at T1, and the UE determines the receive beam that eventually becomes the target TCI state at T1. Therefore, it more reasonable to calculate the 1280 ms time instance start from T1, rather than T2.

(a) BM-Case1

(b) BM-Case2

Figure 2 Rx beam knowledge and TCI handling for BM-Case1 and BM-Case2

Proposal 3: For both BM-Case1 and BM-Case2, the target TCI state is known when the following is met: The TCI state switch command is received within 1280ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L1 RSRP report of the target TCI state.

- Conclusions

In this contribution, we have discussed the testing framework for beam management, and got the following proposals:

Proposal 1: For BM testability, RAN4 needs to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 2: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Proposal 3: For both BM-Case1 and BM-Case2, the target TCI state is known when the following is met: The TCI state switch command is received within 1280ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L1 RSRP report of the target TCI state.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2