3GPP TSG-RAN WG4 Meeting #116									     R4-2510813

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	7.10.3.2

Source:	CAICT

Title:	Discussion on performance metric of NTN OTA

Document for:	Approval

1	Introduction

During RAN4#116, the following conclusions were reached regarding NTN OTA test methodologies [1]. This contribution provides our further views to address unresolved aspects.

Sub-topic 2-1 UE type, usage scenarios and performance metric for NTN (NR-NTN and IoT-NTN) Issue 2-1-1: Details on NTN usage scenario declaration Agreements: - RAN4 to adopt the following rules for scenario declaration in NTN OTA testing: - DUT shall declare to support only one scenario - The declaration of scenario applicability shall be consistent with product manual usage scenario - The declaration of scenario shall be clearly stated in the test report Issue 2-1-2: alignment of understanding on UE positioning guideline for Scenario 2 Agreements: - For Scenario 2 UEs: - The DUT shall be mounted in a suitable hand phantom - The elevation tilt of the default position of the DUT can be: - 0 degree (similar to Figure 6.1-1 of TR 38.870, but with hand phantom) or as shown below - [±15, ±30, ±45] degrees; Other tilt angles are not precluded - NOTE 1: the granularity of the elevation tilt to be further discussed next meeting - The elevation tilt of the hand phantom & DUT is determined based on OEM declaration Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center Issue 2-1-5: What is the angle range of the partial sphere-after concluding center of the sphere Agreements: - RAN4 further discuss the theta angle range of partial sphere metric, from ±30°, ±60°, ±90°. - Option 1: single range is selected and captured in TR - Option 2: multi-range in TR but based on UE declaration, details are FFS - Note1: the phi range should be 360 degrees - Note2: if multiple ranges are captured in TR, the metric applicability rule should also be defined. The target is selecting one for NTN scenario 2. Issue 2-1-6: Whether Using integration of EIRP/EIS or CDF? Agreements: - Integration is baseline. - If different range is declared in issue 2-1-5, how to ensure the metric comparable should be discussed and resolved. Sub-topic 2-2 NTN OTA test methodologies Issue 2-2-2: Whether Dwell time is considered during NTN OTA testing Agreements: - RAN4 further discuss and align understanding of NTN antenna tuning, if any. - RAN4 further discuss whether Dwell time for NTN OTA test case is needed or not. a decision in RAN4 is needed. Issue 2-2-3: NTN MU Agreements: - For NTN OTA, there is no MU update identified. Partial sphere metric related MU may be further discussed, if needed.

2	Discussion

- For Issue 2-1-2, using 0° elevation tilt as the default position can simplify the test setup, and improves comparability of results across different devices and test labs. This position also reflects the most neutral mounting orientation and minimizes additional variables in performance measurement. Therefore, we believe adopting 0° elevation tilt as the default position can provide a clear and consistent reference for NTN OTA measurements. It is recommended to use the 0° tilt as default position, and highly recommend that OEMs use this position unless there is a specific product design or usage scenario that requires a different tilt.

- Proposal 1: Adopt 0° elevation tilt as the default position for NTN OTA testing. Other tilt angles may be declared only if specifically required by the product design or intended usage scenario.

- For Issue 2-1-5, it is suggested to select a single theta angle range for the partial sphere metric, rather than multiple ranges. The obvious disadvantage of defining multiple angles in performance metric is that it will significantly reduce the comparability between test results, which will limit the reference value of NTN OTA test results for the industry, and will also increase the complexity of the test.

- When selecting a single theta angle range for partial sphere metric, a small theta range can be limiting, as some devices are capable of maintaining satellite connections over wider angular spans, and a narrow range would fail to capture this performance advantage. Choosing the range covering the upper hemisphere can be more compatible with various device designs and satellite link conditions, providing a simpler and more representative performance evaluation.

- Proposal 2: Adopt a single theta angle range for the partial sphere performance metric, the theta angle range is define as [-90°,90°].

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: Adopt 0° elevation tilt as the default position for NTN OTA testing. Other tilt angles may be declared only if specifically required by the product design or intended usage scenario.

- Proposal 2: Adopt a single theta angle range for the partial sphere performance metric, the theta angle range is define as [-90°,90°].

References

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3