3GPP TSG-RAN WG4 Meeting #116									     R4-2510817

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:	7.10.4.1

Source:	CAICT

Title:	Discussion on TRP TRS requirements

Document for:	Approval

1	Introduction

In this contribution, we provide our views on Rel-19 TRP TRS requirements.

2	Discussion

- In RAN4#115 meeting, it was agreed that a single set of TRP/TRS requirements will apply to all devices with width ≥56mm and ≤92mm (merging Size 1 and Size 2) for non-defined requirements [1]. Measurement data from both device sizes will populate a unified CDF for developing requirements.

Issue 4-2-1: offset between Size 1 and Size 2 device TRP/TRS requirements Agreements: - For already defined requirements: - - n1: 0.5 dB tighten - - n28, n41, n78: 0 dB - For non-defined requirements, Single set of requirements for device width ≥56mm and ≤92mm. - Note: both device size results can be used for data pool to generate CDF.

- Since devices of different sizes require distinct hand phantoms for testing (Wide hand for Size 1, PDA hand for Size 2), indicating the device size type in measurement reports is necessary to preserve traceability to test configurations during unified data pool analysis.

- Proposal 1: All submitted TRP/TRS measurement results shall indicate device size (Size 1 or Size 2). The Rel-19 Template for TRP/TRS Performance Test Campaign shall be adjusted accordingly.

3	Conclusion

This contribution makes the following observations and proposals.

- Proposal 1: All submitted TRP/TRS measurement results shall indicate device size (Size 1 or Size 2). The Rel-19 Template for TRP/TRS Performance Test Campaign shall be adjusted accordingly.

References

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3