TSG-RAN Working Group 4 (Radio) meeting #116	R4-2510827

Bengaluru, India, 25th to 29th August 2025

Agenda item:			10.1

Source:	Ericsson

Title:	Discussion on simplification of tables with requirements for co-existence and co-location

Document for:	Approval

# Introduction

During RAN4 #115 a way forward document was approved in [2] and the proposal was to Maintain tables for co-existence and co-location in each specification but simplify them.

Companies were encouraged to present the possible implementations of these options during this meeting, identifying pros and cons of the above options.

The following aspects need be considered when drafting the CRs:

- All notes deleted from the legacy tables are properly reflected in new table format.

- For conformance testing specifications, special care needs to be taken to correctly reflect frequency-dependant MU/TT values.

- FFS: how to reassure that unnecessary co-location requirements are NOT unintentionally added with this new framework, e.g., IMT BS and FRMCS BS, IMT BS and broadcasting BS, V2X, Unlicensed bands, etc.

- FFS: how to reassure that unnecessary co-existence requirements are NOT unintentionally added with this new framework.

- FFS: how to consider potential BS-class specific co-existence limits – refer to the BS Evo objective (TFES request).

During RAN4#115 a number of proposals were presented. A WF was agreed in R4-2508664.

# Discussion

In the WF the following options were outlined:

- Have a column in the table that contains frequency ranges which are subject to co-existence and co-location

- Add a new Annex to TS 37.104 containing all the frequency bands and respective frequency ranges which are in the scope for co-existence and co-location requirements (see example below)

An example of possible implementation is presented in R4-2508759.

The proposed target format of co-ex and co-location tables is described below, using TS 38.104 and BS Type 1-C for exemplification:

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type to co-exist with | Frequency range for co-existence requirement | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 | 100kHz | NOTE 1 |
|  | 824 - 849 MHz | -61 |  |  |
| GSM900 | 921 - 960 MHz | -57 |  |  |
|  | 876 - 915 MHz | -61 |  |  |
| DCS1800 | 1805 - 1880 MHz | -47 |  |  |
|  | 1710 - 1785 MHz | -61 |  |  |
| PCS1900 | 1930 - 1990 MHz | -47 |  |  |
|  | 1850 - 1910 MHz | -61 |  |  |
| Other system type | Frequency range of downlink operating band of the BS to co-exist with (NOTE 7, 8) | -52 | 1MHz | NOTE 1 |
|  | Frequency range of uplink operating band of the BS to co-exist with (NOTE 8) | -49 |  | NOTE 1, 4, 5, 6 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 6.6.5.2.3 -1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see table 5.2-1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 6.6.5.2.3 -1 assumes that two operating bands, where the frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	For NR Band n28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band.

NOTE 5:	For NR Band n29 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

NOTE 6:	For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

NOTE 7:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104, table 5.5-1.

NOTE 8:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 37.104 annex X.

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

NOTE 1:	As defined in the scope for spurious emissions in this clause, the co-location requirements in table 6.6.5.2.4-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 5.2-1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [4].

NOTE 2:	Table 6.6.5.2.4-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in table 5.2-1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	Does not apply for co-location with V2X operation defined in TS 36.104, table 5.5-1.

NOTE 5:	UTRA, E-UTRA, NR as described in annex X in TS 37.104.

Observation1: A new Annex X in TS 37.104 is an acceptable way fwd for us to solve the issue of describing the frequency range for different operating bands. A proposal is attached at the end of this document.

Proposal 1: In some cases (e.g. TS 37.105, TS 36.104) we need to delete 3 tables (e.g for WA, MR and LA)  while introducing a new one with all the information. In this case the present table description will be set to “void” and a new table will be introduced.

Proposal 2: In some specification the Out-of-band blocking requirements for co-location need to be compressed as well in one table. This aspect was not mentioned in the WF. See proposals in R4-2510829 and -30.

Proposal 3: Use the proposal in R4-2508759 as the reference for implementation.

Proposal 4: Simplify the Blocking requirements for co-location following the template used in TS 38.104 clause 7.5.3 (Conducted) and 10.6.2 (OTA)

# Summary

Observation1: A new Annex X in TS 37.104 is an acceptable way fwd for us to solve the issue of describing the frequency range for different operating bands. A proposal is attached at the end of this document.

Proposal 1: In some cases (e.g. TS 37.105, TS 36.104) we need to delete 3 tables (e.g for WA, MR and LA)  while introducing a new one with all the information. In this case the present table description will be set to “void” and a new table will be introduced.

Proposal 2: In some specification the Out-of-band blocking requirements for co-location need to be compressed as well in one table. This aspect was not mentioned in the WF. See proposals in R4-2510829 and -30.

Proposal 3: Use the proposal in R4-2508759 as the reference for implementation.

Proposal 4: Simplify the Blocking requirements for co-location following the template used in TS 38.104 clause 7.5.3 (Conducted) and 10.6.2 (OTA)

Proposal for Annex X in 37.104 :

- Annex X (informative): List of 3GPP operating bands

Operating bands are defined for the different RATs throughout 3GPP specifications. All bands are summarized in the tables below based on the following references:

-	UTRA and NB-IoT in TS 25.104 [2] and TS 25.105 [3].

-	E-UTRA in TS 36.104 [4].

-	GSM/EDGE in TS 45.005 [5].

-	NR in TS 38.104 [17].

-	MSR bands in the present specification.

The tables can be used for reference to 3GPP operating bands, where bands in 410 MHz – 7125 MHz are summarized in Table X-1 and bands in 24250 MHz – 71000 MHz are summarized in Table X-2.

Table X-1: 3GPP operating bands in 410 MHz – 7125 MHz

| MSR Band number | Supported RATs and Band Numbers |  |  |  |  | Uplink (UL) BS receive, UE transmit (MHz) | Downlink (DL) BS transmit, UE receive (MHz) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA | NB-IoT | UTRA | GSM/EDGE |  |  |  |
| 1 | n1 | 1 | X | I | - | 1920 – 1980 | 2110 – 2170 | FDD |
| 2 | n2 | 2 | X | II | PCS 1900 | 1850 – 1910 | 1930 – 1990 | FDD |
| 3 | n3 | 3 | X | III | DCS 1800 | 1710 – 1785 | 1805 – 1880 | FDD |
| 4 | - | 4 | X | IV | - | 1710 – 1755 | 2110 – 2155 | FDD |
| 5 | n5 | 5 | X | V | GSM 850 | 824 – 849 | 869 – 894 | FDD |
| 6 | - | - | - | VI | - | 830 – 840 | 875 – 885 | FDD |
| 7 | n7 | 7 | X | VII | - | 2500 – 2570 | 2620 – 2690 | FDD |
| 8 | n8 | 8 | X | VIII | E-GSM | 880 – 915 | 925 – 960 | FDD |
| 9 | - | 9 | - | IX | - | 1749.9 – 1784.9 | 1844.9 – 1879.9 | FDD |
| 10 | - | 10 | - | X | - | 1710 – 1770 | 2110 – 2170 | FDD |
| 11 | - | 11 | X | XI | - | 1427.9 – 1447.9 | 1475.9 – 1495.9 | FDD |
| 12 | n12 | 12 | X | XII | - | 699 – 716 | 729 – 746 | FDD |
| 13 | n13 | 13 | X | XIII | - | 777 – 787 | 746 – 756 | FDD |
| 14 | n14 | 14 | X | XIV | - | 788 – 798 | 758 – 768 | FDD |
| 15 | - | - | - | - | - | Reserved |  | - |
| 16 | - | - | - | - | - | Reserved |  | - |
| 17 | - | 17 | X | - | - | 704 – 716 | 734 – 746 | FDD |
| 18 | n18 | 18 | X | - | - | 815 – 830 | 860 – 875 | FDD |
| 19 | - | 19 | X | XIX | - | 830 – 845 | 875 – 890 | FDD |
| 20 | n20 | 20 | X | XX | - | 832 – 862 | 791 – 821 | FDD |
| 21 | - | 21 | X | XXI | - | 1447.9 – 1462.9 | 1495.9 – 1510.9 | FDD |
| 22 | - | 22 | - | XXII | - | 3410 – 3490 | 3510 – 3590 | FDD |
| 23 | - | 23 | - | - | - | 2000 – 2020 | 2180 – 2200 | FDD |
| 24 | n24 | 24 | X | - | - | 1626.5 – 1660.5 | 1525 – 1559 | FDD |
| 25 | n25 | 25 | X | XXV | - | 1850 – 1915 | 1930 – 1995 | FDD |
| 26 | n26 | 26 | X | XXVI | - | 814 – 849 | 859 – 894 | FDD |
| 27 | - | 27 | - | - | - | 807 – 824 | 852 – 869 | FDD |
| 28 | n28 | 28 | X | - | - | 703 – 748 | 758 – 803 | FDD |
| 29 | n29 | 29 | - | - | - | N/A | 717 – 728 | SDL |
| 30 | n30 | 30 | - | - | - | 2305 – 2315 | 2350 – 2360 | FDD |
| 31 | n31 | 31 | X | - | - | 452.5 – 457.5 | 462.5 – 467.5 | FDD |
| 32 | - | 32 | - | XXXII | - | N/A | 1452 – 1496 | FDD |
| 33 | - | 33 | - | a) | - | 1900 – 1920 | 1900 – 1920 | TDD |
| 34 | n34 | 34 | - | a) | - | 2010 – 2025 | 2010 – 2025 | TDD |
| 35 | - | 35 | - | b) | - | 1850 – 1910 | 1850 – 1910 | TDD |
| 36 | - | 36 | - | b) | - | 1930 – 1990 | 1930 – 1990 | TDD |
| 37 | - | 37 | - | c) | - | 1910 – 1930 | 1910 – 1930 | TDD |
| 38 | n38 | 38 | - | d) | - | 2570 – 2620 | 2570 – 2620 | TDD |
| 39 | n39 | 39 | - | f) | - | 1880 – 1920 | 1880 – 1920 | TDD |
| 40 | n40 | 40 | - | e) | - | 2300 – 2400 | 2300 – 2400 | TDD |
| 41 | n41 | 41 | X | - | - | 2496 – 2690 | 2496 – 2690 | TDD |
| 42 | - | 42 | X | - | - | 3400 – 3600 | 3400 – 3600 | TDD |
| 43 | - | 43 | X | - | - | 3600 – 3800 | 3600 – 3800 | TDD |
| 44 | - | 44 | - | - | - | 703 – 803 | 703 – 803 | TDD |
| 45 | - | 45 | - | - | - | 1447 – 1467 | 1447 – 1467 | TDD |
| - | n46 | 46 | - | - | - | 5150 – 5925 | 5150 – 5925 | TDD |
| - | - | 47 | - | - | - | 5855 – 5925 | 5855 – 5925 | TDD |
| 48 | n48 | 48 | X | - | - | 3550 – 3700 | 3550 – 3700 | TDD |
| - | - | 49 | - | - | - | 3550 – 3700 | 3550 – 3700 | TDD |
| 50 | n50 | 50 | - | - | - | 1432 – 1517 | 1432 – 1517 | TDD |
| 51 | n51 | 51 | - | - | - | 1427 – 1432 | 1427 – 1432 | TDD |
| 52 | - | 52 | - | - | - | 3300 – 3400 | 3300 – 3400 | TDD |
| 53 | n53 | 53 | - | - | - | 2483.5 – 2495 | 2483.5 – 2495 | TDD |
| 54 | n54 | 54 | X | - | - | 1670 – 1675 | 1670 – 1675 | TDD |
| 64 | - | - | - | - | - | Reserved |  | - |
| 65 | n65 | 65 | X | - | - | 1920 – 2010 | 2110 – 2200 | FDD |
| 66 | n66 | 66 | X | - | - | 1710 – 1780 | 2110 – 2200 | FDD |
| 67 | n67 | 67 | - | - | - | N/A | 738 – 758 | SDL |
| 68 | n68 | 68 | - | - | - | 698 – 728 | 753 – 783 | FDD |
| 69 | - | 69 | - | - | - | N/A | 2570 – 2620 | SDL |
| 70 | n70 | 70 | X | - | - | 1695 – 1710 | 1995 – 2020 | FDD |
| 71 | n71 | 71 | X | - | - | 663 – 698 | 617 – 652 | FDD |
| 72 | n72 | 72 | X | - | - | 451 – 456 | 461 – 466 | FDD |
| 73 | - | 73 | X | - | - | 450 – 455 | 460 – 465 | FDD |
| 74 | n74 | 74 | X | - | - | 1427 – 1470 | 1475 – 1518 | FDD |
| 75 | n75 | 75 | - | - | - | N/A | 1432 – 1517 | SDL |
| 76 | n76 | 76 | - | - | - | N/A | 1427 – 1432 | SDL |
| - | n77 | - | - | - | - | 3300 – 4200 | 3300 – 4200 | TDD |
| - | n78 | - | - | - | - | 3300 – 3800 | 3300 – 3800 | TDD |
| - | n79 | - | - | - | - | 4400 – 5000 | 4400 – 5000 | TDD |
| - | n80 | - | - | - | - | 1710 – 1785 | N/A | SUL |
| - | n81 | - | - | - | - | 880 – 915 | N/A | SUL |
| - | n82 | - | - | - | - | 832 – 862 | N/A | SUL |
| - | n83 | - | - | - | - | 703 – 748 | N/A | SUL |
| - | n84 | - | - | - | - | 1920 – 1980 | N/A | SUL |
| 85 | n85 | 85 | X | - | - | 698 – 716 | 728 – 746 | FDD |
| - | n86 | - | - | - | - | 1710 – 1780 | N/A | SUL |
| 87 | n87 | 87 | X | - | - | 410 – 415 | 420 – 425 | FDD |
| 88 | n88 | 88 | X | - | - | 412 – 417 | 422 – 427 | FDD |
| - | n89 | - | - | - | - | 824 – 849 | N/A | SUL |
| - | n90 | - | - | - | - | 2496 – 2690 | 2496 – 2690 | TDD |
| - | n91 | - | - | - | - | 832 – 862 | 1427 – 1432 | FDD |
| - | n92 | - | - | - | - | 832 – 862 | 1432 – 1517 | FDD |
| - | n93 | - | - | - | - | 880 – 915 | 1427 – 1432 | FDD |
| - | n94 | - | - | - | - | 880 – 915 | 1432 – 1517 | FDD |
| - | n95 | - | - | - | - | 2010 – 2025 | N/A | SUL |
| - | n96 | - | - | - | - | 5925 – 7125 | 5925 – 7125 | TDD |
| - | n97 | - | - | - | - | 2300 – 2400 | N/A | SUL |
| - | n98 | - | - | - | - | 1880 – 1920 | N/A | SUL |
| - | n99 | - | - | - | - | 1626.5 – 1660.5 | N/A | SUL |
| - | n100 | - | - | - | - | 874.4 – 880 | 919.4 – 925 | FDD |
| - | n101 | - | - | - | - | 1900 – 1910 | 1900 – 1910 | TDD |
| - | n102 | - | - | - | - | 5925 – 6425 | 5925 – 6425 | TDD |
| - | - | 103 | - | - | - | 787 – 788 | 757 – 758 | FDD |
| - | n104 | - | - | - | - | 6425 – 7125 | 6425 – 7125 | TDD |
| - | n105 | - | - | - | - | 663 – 703 | 612 – 652 | FDD |
| 106 | n106 | 106 | X | - | - | 896 – 901 | 935 - 940 | FDD |
| - | - | 107 | - | - | - | N/A | 612 – 652 | SDO |
| - | - | 108 | - | - | - | N/A | 470 – 698 | SDO |
| - | n109 | - | - | - | - | 703 – 733 | 1432 – 1517 | FDD |
| - | n110 | - | - | - | - | 1390 - 1395 | 1432 - 1435 | FDD |
| 111 | - | 111 | X | - | - | 1800 - 1810 | 1820 - 1830 | FDD |

Table X-2: NR operating bands in 24250 MHz – 71000 MHz

| Supported RAT and Band Numbers | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive (MHz) | Duplex mode |
| --- | --- | --- |
| NR |  |  |
| n257 | 26500 – 29500 | TDD |
| n258 | 24250 – 27500 | TDD |
| n259 | 39500 – 43500 | TDD |
| n260 | 37000 – 40000 | TDD |
| n261 | 27500 – 28350 | TDD |
| n262 | 47200 – 48200 | TDD |
| n263 | 57000 – 71000 | TDD |