3GPP TSG-RAN WG4 Meeting #116	R4-2510837

Bengaluru, India, 25 August – 29 August 2025

Agenda Item:	7.18.3

Source:	Ericsson

Title:	Discussion on the 2-sided CSI study

Document for:	Discussion

# 1	Introduction

RAN4#115 agreed with the way forward on two-sided CSI compression [1]. This contribution discusses the open issues to complete the study item.

# 2	Additional simulation results for Option 3 Track 2

The remaining open issue for the two-sided CSI compression is the consideration of the low complexity encoders.

Further simulations (not agreement): In order to further elaborate option 3 track 2 considering lower complexity encoders, interested companies may contribute simulations to RAN4#116. The following guidance, and also the guidance in R4-2508084 is provided and recommended for companies to follow in order to obtain aligned simulations. For option 3, track 2 consider the following investigations for lower complexity encoder structures. - Performance assuming training and testing with the Samsung frozen decoder - Performance achieved with a jointly trained low complexity encoder and decoder - Performance achieved when a frozen decoder is trained based on all of the encoder structures (i.e. different encoder complexity) - Consider transformer, CNN, MLP

## 2.1	Evaluation with 2 CNN, 1 Transformer, and 1 MLP encoders with the agreed decoder

Table 1 summarizes our simulation results of Option 3 Track 2 for the case of 2 CNN, 1 Transformer, and 1 MLP encoders with the agreed decoder. The results are presented, according to the simulation assumption reporting format [2]. Note that in the second column of Table 1, we also present the result when we use ‘Ericsson’ own decoder instead of the frozen decoder, which is written inside the parenthesis.

The detailed procedure to derive SGCS-1/2/3/4 is described as below:

- (1-to-1) Joint training - For each pair of Encoder-N (N=1,2,3,4) and Agreed decoder, train both the encoder and decoder based on the mixed dataset. - Run training for each encoder-decoder pair. - For each pair of trained encoder-N and trained agreed decoder, calculate SGCS (SGCS-0 for Encoder-1 and SGCS-3 for Encoder-2/3/4). - New encoder training with Samsung’s frozen decoder - For each pair of Encoder-N (N=1,2,3,4) and Samsung’s (frozen) decoder, train the encoder based on the mixed dataset. - Run training for each encoder-decoder pair. - For each pair of trained encoder-N and Samsung’s decoder, calculate SGCS (SGCS-1 for Encoder-1 and SGCS-2 for Encoder-2/3/4). - (N-to-1) Joint training SGCS-4 - Step-1 - Jointly train all the encoders towards the agreed decoder based on the mixed dataset. - Run one training, that is, four encoders are trained at the same time. - For each pair of trained Encoder-N and trained agreed decoder, calculate SGCS (SGCS-4 step 1). - New encoder training with frozen decoder from Step-1 SGCS-4 - Step-2 - Jointly train all the encoders towards the agreed decoder based on the mixed dataset. - Run one training, that is, four encoders are trained at the same time. - The purpose is to train the decoder. - Using the trained agreed decoder, for each pair of Encoder-N and trained agreed decoder, train the encoder based on the mixed dataset. - Run training for each pair. - For each pair of trained Encoder-N and trained agreed decoder, calculate SGCS (SGCS-4 step 2).

Table 1	Simulation results of low-complexity encoders with different backbones. The values in () in the second column is the results using ‘Ericsson’ own decoder instead of the frozen decoder.

|  | FLOPs (M) | (1-to-1) Joint training | New encoder training with Samsung’s frozen decoder | (N-to-1) Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 |
| --- | --- | --- | --- | --- | --- |
|  |  | SGCS-0/3 | SGCS-1/2 | SGCS-4 step 1 | SGCS-4 step 2 |
| Encoder-1 (Agreed encoder) | 47 | 0.6937 | 0.7000 (0.6944) | 0.7012 | 0.7017 |
| Encoder-2 (Transformer) | 3.63 | 0.7112 | 0.7022 (0.7049) | 0.7130 | 0.7156 |
| Encoder-3 (Convolutional neural network, CNN) | 4.14 | 0.6790 | 0.6810 (0.6821) | 0.6898 | 0.6910 |
| Encoder-4 (Multi-layer perceptron, MLP) | 0.22 | 0.6677 | 0.6344 (0.6553) | 0.6640 | 0.6753 |

From the above table, we can observe that, compared with the 1-to-1 joint training, training with N-to-1 joint training has a performance difference of +1.1%, +0.3%, +1.6%, and -0.6%, for the Agreed encoder (Encoder-1), Transformer encoder (Encoder-2), lower complexity CNN encoder (Encoder-3), and MLP encoder (Encoder-4), respectively.

Comparing the second column (SGCS-1/2, New encoder training with Samsung’s frozen decoder) and the fourth column (SGCS-4 step 2, New encoder training with frozen decoder from Step-1), we can observe that, compared to training a new encoder with a frozen decoder obtained from 1-to-1 joint training of the agreed encoder (Encoder-1), training a new encoder with a frozen decoder obtained from the N-to-1 joint training has a performance difference of +0.2%, +1.9%, +1.5%, and +6.4% for the Agreed encoder (Encoder-1), Transformer encoder (Encoder-2), lower complexity CNN encoder (Encoder-3), and MLP encoder (Encoder-4), respectively.

Observation 1: For the agreed decoder architecture and training hyper parameters, SGCS-4 step 1 shows better SGCS compared with SGCS-0/3. That is, N-to-1 joint training gives a better performance than 1-to-1 joint training.

Different percentages of improvement can be seen when the Ericsson own decoder is used instead of the Samsung frozen decoder, i.e. +1.1%, +1.5%, +1.3%, and +3.1% for the Agreed encoder (Encoder-1), Transformer encoder (Encoder-2), lower complexity CNN encoder (Encoder-3), and MLP encoder (Encoder-4), respectively. This may suggest that the Samsung decoder fitted better toward the Encoder-1 architecture but less accommodating toward other (low-complexity) encoders.

Observation 2: For the agreed decoder architecture and training hyper parameters, SGCS-4 step 2 shows better SGCS compared with SGCS-1/2. That is, encoders trained with a frozen decoder obtained from N-to-1 joint training have a better performance than the encoders trained with a frozen decoder obtained from 1-to-1 joint training of Encoder-1.

## 2.2	Evaluation with 4 CNN encoders with the low-complexity decoder

In this contribution paper, except evaluations for the case of the four encoders are 2 CNN, 1 Transformer, and 1 MLP with the agreed decoder, we also present evaluation results for the case of different CNN models with a lower complexity decoder, i.e., a convolution size of 32 is used instead of 128. Observation from such evaluations is important to make sure that the conclusion drawn from the evaluation/observations applies across different scenarios.

We have also evaluated 3 CNN encoders, called Encoder-5/6/7, together with agreed encoder (Encoder-1). The configurations and complexities of these encoders are summarized in Table 2. The block diagram is according to the agreed diagram in A.2.1 (only differ in the sizes of the CNN parameters).

Since the currently agreed decoder has an extremely high complexity, especially in terms of FLOPs (around 1000M FLOPs), we have used a decoder with a lower complexity than the agreed decoder. The configuration of the decoder architecture is the same as the agreed CNN-based decoder (See A.3.1), but the convolution size is 32 instead of 128.

Table 2	Configurations and complexities used in the evaluation with 4 CNN-based encoders.

| Encoder name | Backbone | Conv. Size | # Resnet Blocks | Params (M) | Storage (MB) | FLOPs (Relative to Encoder-1) |
| --- | --- | --- | --- | --- | --- | --- |
| Encoder-1 (Agreed encoder) | CNN | 32 | 6 | 0.13 | 0.50 | 1.00 |
| Encoder-5 | CNN | 16 | 2 | 0.02 | 0.09 | 0.09 |
| Encoder-6 | CNN | 32 | 2 | 0.05 | 0.21 | 0.34 |
| Encoder-7 | CNN | 32 | 4 | 0.09 | 0.35 | 0.67 |

The evaluation results using this setup can be seen in Table 3.

Table 3	Simulation results of 4 CNN encoder architectures with a lower complexity decoder.

|  | (1-to-1) Joint training | New encoder training with Ericsson’s frozen low-complexity decoder | (N-to-1) Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 |
| --- | --- | --- | --- | --- |
|  | SGCS-0/3 | SGCS-1/2 | SGCS-4 step 1 | SGCS-4 step 2 |
| Encoder-1 (Agreed encoder) | 0.6971 | 0.6977 | 0.6959 | 0.6955 |
| Encoder-5 (CNN Config 1) | 0.6873 | 0.6777 | 0.6793 | 0.6798 |
| Encoder-6 (CNN Config 2) | 0.6898 | 0.6851 | 0.6847 | 0.6847 |
| Encoder-7 (CNN Config 3) | 0.6947 | 0.6936 | 0.6921 | 0.6926 |

From Table 3, we can observe that compared with the 1-to-1 joint training, training with N-to-1 joint training has a performance difference of -0.2%, -1.2%, -0.7%, and -0.4%, for the Agreed encoder, CNN Config 1 encoder, CNN Config 2 encoder, and CNN Config 3 encoder, respectively.

Observation 3: For the (low-complexity) decoder with convolution size of 32 and low complexity CNN encoders, SGCS-4 step 1 shows worse SGCS compared with SGCS-0/3. That is, N-to-1 joint training has a lower performance than 1-to-1 joint training.

Comparing the second column (New encoder training with Ericsson’s frozen decoder) and the fourth column (New encoder training with frozen decoder from Step-1), we can observe that compared to training a new encoder with a frozen low-complexity decoder obtained from 1-to-1 joint training of the agreed encoder (Encoder-1), training a new encoder with a frozen low-complexity decoder obtained from the N-to-1 joint training has a performance difference of -0.3%, +0.3%, -0.1%, and -0.1% for the Agreed encoder, CNN Config 1 encoder, CNN config 2 encoder, and CNN config 3 encoder, respectively.

Observation 4: For the (low-complexity) decoder with convolution size of 32, SGCS-4 step 2 for Encoder-5 only shows better SGCS compared with SGCS-2.

Considering that two different observations are obtained from two different setups, a rigid conclusion related to the evaluation from Table 1 and Table 3 shall be avoided. Since we think it is difficult to conclude the benefit of training the decoder with multiple encoder configurations and/or backbone, RAN4 should revisit it during in Rel-20 WI.

Observation 5: It is difficult to conclude the benefit of training the decoder with multiple encoder configurations and/or backbone.

# 3	Issues to be discussed in WI phase

RAN#108 has approved a new WI on AI/ML for NR air interface enhancements in Rel-20 [3], where the objective includes CSI feedback enhancement with two-sided model and inter-vendor training collaboration for two-sided model. We will bring issues to be discussed in the Rel-20 WI phase.

## 3.1	What/how to specify the model/dataset in 3GPP

Option 3: Standardized Decoder Agreements from RAN4#114bis - The encoder that corresponds to the standardized test decoder is stored within 3GPP - TBC whether in a TR, or in any TS, whether in RAN4 or RAN5, or whether simply kept in a 3GPP database (with no reference from TR or TS) - FFS whether several encoders would be captured (e.g. due to different complexity) New agreement from RAN4#115: - The training dataset used to train the test decoder in 3GPP should be captured - FFS whether channel, Eigenvectors or something else Option 4a: Standardized Dataset Agreements from RAN4#114bis - Dataset is specified for option 4a. - Assume reference decoder is captured. New agreements from RAN4#115: - Reference encoder should be captured - FFS whether the labelled dataset is based on Eigenvector, channel estimation or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated Option 4b: Standardized reference Encoder Agreements from RAN4#114bis - Dataset is specified for option 4a. - FFS on dataset is captured for option 4b. - Assume reference decoder is captured. - FFS on reference encoder for option 4a - Test decoder structure is (i) fully specified or (ii) partially specified - NOTE: RAN4 understands that further decision on option 4a/4b and option 3 will be made in the future WI phase. New agreements from RAN4#115: - Specify dataset to train frozen encoder and decoder - TE vendor shall use the dataset to train the test decoder - This is the same dataset as used to train the frozen encoder - FFS whether the datasets consist of Eigenvector, channel or something else - The study has only considered the case of a common assumption on model structure for the “own” test decoder. This corresponds to standardized model structure. - Structure refers to backbone, numbers of layers, type of layers and all description of the model, but not the parameters. - Interoperability in the case that the structure of the TE decoder is not specified has not been investigated

One of the issues that should be discussed in Rel-20 WI is what/how to specify the model(s)/dataset(s) to support two-sided CSI compression functionalities in 3GPP. In Rel-19 SI, RAN4 so far discussed and evaluated three options.

Option 3: Standardized decoder

In this option, test decoder (including structure and parameters) is stored within 3GPP. During the UE conformance test, TE uses this test decoder to decode the CSI from the UE encoder.

It was also agreed that 3GPP should capture:

- the training datasets used to train the test decoder, and

- the reference encoder(s) (structure only) associated with the decoder.

Option 4a: Standardized datasets

In this option, the dataset is specified so that the TE vendors can train their test decoder. In this option, the ‘labeled dataset’ should be the latent space, e.g., eigenvectors, channel estimates, etc. RAN4 agreed to capture the reference encoder structure so that TE vendors train their test decoder. It may need to capture the reference encoder parameters also.

During the conformance test, TE uses their trained test decoder to decode the CSI from the UE encoder.

Option 4b: Standardized reference encoder

In this option, the reference encoder is standardized. TE vendors first develop their test decoder towards the standardized reference encoder. RAN4 agreed to specify the dataset to train the reference encoder and to train TE vendors’ test decoder.

During the conformance test, TE uses their trained test decoder to decode the CSI from the UE encoder.

Table 4	Summary of what to be captured in 3GPP.

|  | Option 3 | Option 4a | Option 4b |
| --- | --- | --- | --- |
| What is standardized | Test decoder | Dataset with the latent space | Reference encoder |
| What to be ‘captured’ | - Standard test decoder (structure and parameters) - Dataset | - Reference encoder (structure and parameters) - Dataset with the latent space - Reference decoder (structure) | - Reference encoder (structure and parameters) - Dataset - Reference decoder (structure) |
| TE vendor | - Use the standard test decoder to decode CSI from the UE encoder. | - Train both their test decoder and a reference encoder with the dataset (with latent space). - Use their test decoder to decode CSI from the UE encoder. | - Train their test decoder towards the standard reference encoder with the dataset. - Use their test decoder to decode CSI from the UE encoder. |

Table 4 summarizes what to be captured in 3GPP for each option. Regardless of the options, in our understanding, 3GPP needs to capture the encoder, decoder and dataset together with latent space in the end.

As agreed in the WF, 3GPP needs further discussion on how to specify/capture the dataset for training, reference encoder, and/or reference decoder in the WI.

If 3GPP agrees to store the reference encoder and/or reference decoder in 3GPP database (e.g., FTP server), RAN4 needs to discuss the data format. For Rel-19 SI, RAN4 uses ONNX (Open Neural Network Exchange [4]) to exchange reference models. We also observed some companies proposed to use PyTorch [5].

We understand these frameworks are commonly used in AI/ML area in 2025. However, since ONNX and PyTorch are developed by an open-source community, it is not ensured the framework is available throughout the entire lifecycle of 5G. Even if the framework is available, the data format could become obsolete. We think it will be difficult to discuss it again in the future (e.g., new WI or TEI) and it might be an issue if ONNX/PyTorch are not maintained anymore. Even if it is maintained, as it is outside of the 3GPP control, ONNX/PyTorch could evolve into some undesired direction. This means 3GPP needs to be very careful to point towards the correct version and the version needs to not become obsolete the captured/standardized reference models.

If RAN4 is going to specify the reference encoder and/or reference decoder structures (i.e., backbone, numbers of layers, type of layers and all the description of the model) in TS or in TR, RAN4 needs to discuss how to write the models in the TS or in TR, e.g., JSON-based serialized text data or data flow graph. On the other hand, we don’t think it is realistic to specify all the model parameters (i.e., weights) in TS or in TR; it should be stored in 3GPP database.

3GPP also needs to revisit for dataset format. RAN4 so far uses NumPy data format to share the dataset among companies. Although NumPy is commonly used, it is also developed by the open-source community as well as ONNX/PyTorch, RAN4 may need to discuss the data format of the dataset, e.g., binary data with 32bits floating point format (a.k.a. IEEE 754 Single-precision floating-point format). This agreed data format can be reused to store the model parameters. In our understanding, RAN1 will discuss the dataset format in Rel-20 WI.

Proposal 1: RAN4 needs to discuss how to capture the reference encoder and reference decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model) in Rel-20 WI.

Proposal 2: RAN4 needs to discuss how to capture the reference encoder and reference decoder parameters (i.e., model weights) in Rel-20 WI.

Proposal 3: RAN4 needs to revisit the dataset format including latent space in Rel-20 WI.

## 3.2	Performance requirements

In the WI phase, RAN4 needs to specify the CSI reporting performance requirements with CSI compression. RAN4 needs to discuss the test metric and generalization aspect when the WI performance part starts.

Test metric

In the study item, RAN4 only evaluated SGCS between encoder input and decoder output to study the testability/inter-operability of two-sided CSI compression. Since TE does not know the raw channel data before encoding at UE, we cannot use SGCS for the test metric of CSI reporting requirements. In our understanding, the benefit of two-sided CSI compression feature is to reduce the CSI feedback data size. Since RAN1 will discuss the CSI quantity for two-sided models and performance monitoring, RAN4 needs to discuss the test metric of PMI reporting requirements with two-sided CSI compression in Rel-20 WI based on the RAN1 agreements.

Proposal 4: RAN4 needs to discuss the test metric of PMI reporting requirements with two-sided CSI compression model in Rel-20 WI.

Generalization aspects

The main difference of AI/ML-based functions from the non-Al/ML-based functions is that the performance depends on the training dataset. This means even if the AI/ML model shows better performance compared with the traditional algorithm in some scenarios, the same model may not show better performance in other scenarios. Traditionally RAN4 has defined the performance requirements by choosing one or two scenarios (e.g., channel model, antenna configuration, SNR etc.), we think this is the risk the AI/ML model only works well in the specific scenario specified in the performance requirements. Therefore, it is important to ensure the generalization of AI/ML-based CSI compression model.

We think new encoder/decoder to be considered in WI, which should work enough in the real field. In Rel-19 SI, RAN4 uses CNN as the reference encoder/decoder architecture, but probably other architecture (e.g., transformer-based or lower complexity CNN-based) could be better than the one taken.

Moreover, the training dataset should not be generated from a single scenario. In Rel-19 SI, dataset is based on the Urban macro scenario with 32 Tx antenna ports at gNB, but it may not be sufficient to ensure the generalization.

Proposal 5: RAN4 needs to discuss new encoder/decoder architecture in Rel-20 WI.

Proposal 6: RAN4 needs to introduce new dataset(s) ensuring the model generality in Rel-20 WI.

# 4	Summary

Observations:

Observation 1: For the agreed decoder architecture and training hyper parameters, SGCS-4 step 1 shows better SGCS compared with SGCS-0/3. That is, N-to-1 joint training gives a better performance than 1-to-1 joint training.

Observation 2: For the agreed decoder architecture and training hyper parameters, SGCS-4 step 2 shows better SGCS compared with SGCS-1/2. That is, encoders trained with a frozen decoder obtained from N-to-1 joint training have a better performance than the encoders trained with a frozen decoder obtained from 1-to-1 joint training of Encoder-1.

Observation 3: For the (low-complexity) decoder with convolution size of 32 and low complexity CNN encoders, SGCS-4 step 1 shows worse SGCS compared with SGCS-0/3. That is, N-to-1 joint training has a lower performance than 1-to-1 joint training.

Observation 4: For the (low-complexity) decoder with convolution size of 32, SGCS-4 step 2 for Encoder-5 only shows better SGCS compared with SGCS-2.

Observation 5: It is difficult to conclude the benefit of training the decoder with multiple encoder configurations and/or backbone.

Proposals:

Proposal 1: RAN4 needs to discuss how to capture the reference encoder and reference decoder structures (i.e., backbone, numbers of layers, type of layers and all description of the model) in Rel-20 WI.

Proposal 2: RAN4 needs to discuss how to capture the reference encoder and reference decoder parameters (i.e., model weights) in Rel-20 WI.

Proposal 3: RAN4 needs to revisit the dataset format including latent space in Rel-20 WI.

Proposal 4: RAN4 needs to discuss the test metric of PMI reporting requirements with two-sided CSI compression model in Rel-20 WI.

Proposal 5: RAN4 needs to discuss new encoder/decoder architecture in Rel-20 WI.

Proposal 6: RAN4 needs to introduce new dataset(s) ensuring the model generality in Rel-20 WI.

# References

- R4-2508051, “WF on AI/ML part 2”, Ericsson.

- R4-2508084, “Simulation assumptions on AI/ML CSI compression”, Huawei, et al.

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm.

- https://onnx.ai/

- https://pytorch.org/

- R4-2417207, “WF on Data Sharing framework”, Qualcomm Incorporated.

- R4-2411786, “Summary of e-mail discussion on AI/ML Model for CSI testing Option 3 feasibility study”, Moderator (Qualcomm Incorporated).

# Appendix

## A.1	Simulation result reporting format

Table 5:	Format of simulation results of Option 3 to submit in RAN4#116 [2].

|  | (1-to-1) Joint training | New encoder training with (Samsung’s) frozen decoder | (N-to-1) Joint training SGCS-4 Step-1 | New encoder training with frozen decoder from Step-1 SGCS-4 Step-2 |
| --- | --- | --- | --- | --- |
| Encoder-1 | SGCS-0 | SGCS-1 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-2 | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-3 | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |
| Encoder-4 | SGCS-3 | SGCS-2 | SGCS-4 (step-1) | SGCS-4 (step-2) |

## A.2	Encoder structure options

### A.2.1	Encoder 1 (CNN-based agreed encoder) [7]

### A.2.2	Encoder 2 (Transformer based lower-complexity encoder)

### A.2.3	Encoder 3 (CNN based lower-complexity encoder)

### A.2.3	Encoder 4 (MLP based lower-complexity encoder)

## A.3	Decoder structure options

### A.3.1	Agreed decoder (CNN-based agreed decoder) [7]