3GPP TSG-RAN WG4 Meeting #116	R4-2510839

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.24.7

Source:	Ericsson

Title:	UE demodulation requirements for LP-WUS/WUR

Document for:	Discussion

# 1	Introduction

RAN4#116 agreed with the way forward on LP-WUS demodulation requirements [1]. This contribution discusses the open issues on the UE demodulation requirements for LP-WUS/WUR.

# 2	Discussion

## 2.1	Scope of requirements

False alarm rate (FAR) for wake-up signal reception

RAN4 agreed to specify the wake-up signal reception performance requirements, where RAN4 will use the missed detection rate (MDR) as a test metric. RAN4 also discussed whether to use false alarm rate (FAR) as an additional test metric.

Whether to introduce FAR test - Option 1: Yes. - Option 2: No.

False alarm is the case where the receiver incorrectly detects a signal when nothing is transmitted. Higher false alarm rate (FAR) of wake-up signals wakes up the main receiver (MR) unnecessarily and results in consuming more UE battery.

RAN4 has the FAR requirements for PUCCH and PRACH, where it is called ‘DTX to ACK probability requirements’ and ‘PRACH false alarm probability’ for PUCCH and PRACH, respectively. According to TS 38.104, the false alarm rates shall not exceed 1% and 0.1% for PUCCH and PRACH, respectively. It is important to ensure minimizing the false alarm rates of PUCCH and PRACH, otherwise gNB may allocate unnecessary DL resources, e.g., random access response.

Although the false alarm of DL wake-up signal reception could increase the UE power consumption, it does not increase the UL interference. From the network performance point of view, it does not affect the network performance. We think it is sufficient to define missed detection rate requirements for wake-up signals.

Proposal 1: For Wake-up signal demodulation performance requirements, the requirements are specified as the SNR to achieve 1 % of missed detection rate (MDR) of Wake-up signal. RAN4 does specify the Wake-up signal demodulation requirements with false alarm rate (FAR).

LP-SS reception performance

Whether to define performance requirements for LP-SS - Option 1: No - Option 2: Postpone further study on LP-SS until decided if performance requirements are to be introduced for LP-WUR/LP-WUS (OOK receiver)

In LP-WUS WI, RAN1 introduced low-power synchronization signal (LP-SS). The main purpose is that OOK-WUR uses this signal to re-synchronize to the network after the sleep. RAN1 also assumes the OOK-WUR may use LL-SS for RRM measurements.

RAN4 RRM session has already decided to specify LP-RSRP/LP-RSRQ measurement performance requirements.

Observation 1: RAN4 RRM has already decided to define LP-SS based LP-RSRP/LP-RSRQ measurement performance requirements.

Since RAN4 will define LP-RSRP/RSRQ measurement performance requirements, it is not necessary to define the dedicated LP-SS reception requirements in the demodulation performance requirements. However, if RAN4 specifies the wake-up signal reception requirements for OOK-WUR, we may need to configure LP-SS, e.g., every 320ms, to ensure that the OOK-WUR can re-synchronize to the network after the deep sleep. This implicitly verifies the (re-)synchronization functionality performance.

Proposal 2: RAN4 should configure LP-SS for the WUS reception requirements.

## 2.2	Test parameters

RAN4 agreed to define the wake-up signal missed detection requirements, but RAN4 has not decided the detailed test parameters.

Wake-up signal configuration

LP-WUS waveform - Option 1: Consider both OOK-1 and OOK-4 Coding/decoding algorithms for LP-WUS - Option 1: 1 or 2 information bits for small block coding and 3 or more information bits for RM

RAN1 agreed to use On-Off Keying (OOK) to modulate the LP-WUS. When gNB transmits the LP-WUS information bits, it encodes the information bits by Reed-Muller code, performs rate-matching, and then encodes by Manchester code. The gNB applies DFT to the coded bits over 11 RBs (=132 subcarriers) and then performs IFFT together with other DL channel/signals. This means the gNB transmitter overlays the LP-WUS OOK symbols with NR downlink OFDM sequence.

When the gNB applies DFT to map the coded bits into 132 subcarriers, it processes MWUS bits per DFT processing, where MWUS = 1, 2 or 4. For example, in the case of MWUS=1, called OOK-1, one coded bit is transmitted in one OFDM symbol duration. In the case of MWUS=4, gNB transmits four coded bits in one OFDM symbol duration.

TS 38.212 calls the information carried by a wake-up signal as the wake-up information, and the number of wake-up information (K) is up to 5 bits. The wake-up information is channel coded, where the channel coding scheme depends on the wake-up information is K=1, K=2, or K≥3, according to TS 38.212 5.3.3. Especially the wake-up information with K≥3 uses Reed-muller coding.

From the discussion in the last meeting, companies are interested in defining the requirements with both OOK-1 and OOK-4. Considering the different channel coding methods with K=1, 2 and K≥3, we propose to define the WUS reception requirements with the configurations (MWUS, K) = (1, 1), (2, 2), and (4, 5).

Proposal 3: RAN4 defines the wake-up signal reception requirements with (MWUS=1, K=1), (MWUS=2, K=2), and (MWUS=4, K=5).

Frequency range

Frequency range - Option 1: Consider both FR1 and FR2. - Option 2: Prioritize FR1; FFS FR2.

Since RAN4 RF are going to define the WUR RF requirements for both FR1 and FR2, we support defining the wake-up signal reception requirements for FR2 also. According to the UE feature list [3], only OOK-1 is supported for FR2. Therefore, we propose to define the wake-up signal reception requirements with OOK-1 for FR2.

Proposal 4: Define the wake-up signal reception requirements with OOK-1 for FR2.

Channel model

Channel model - Option 1: TDLA30-10 only - Option 2: TDLC300-100 only - Option 3: Both TDLC300-100 and TDLA30-10

UE may use LP-WUR to receive LP-WUS to save the UE battery in any scenario. For example, RAN1 evaluated both the Rural and Urban scenarios. We think RAN4 should evaluate both the channel models for FR1. For FR2, we should use TDLA30-10.

Proposal 5: Evaluate both TDLA30-10 and TDLC300-100 for FR1. Use TDLA30-10 for FR2.

frequency/timing offsets

Time offsets - Option 1: T= 5us for OOK-1, T= 2us for OOK-4 with M=2, T= 1us for OOK-4 with M=4 Frequency offsets - Option 1: For OFDM based receiver [5] ppm and for OOK based receiver [10] ppm - Option 1: up to 20 ppm

Since LP-WUR implements a less accurate oscillator compared with MR, the frequency error and timing error after the wake-up is huge. RAN1 therefore defined a new synchronization signal called LP-SS to help LP-WUR to synchronize with the network.

We agree to assume certain frequency and timing errors when we derive the Wake-up signal reception requirements, but RAN4 does not need to specify in the test parameters in TS 38.101-4, because the Rx frequency error and Rx timing error are part of UE Rx impairments. Based on the TR [2], we propose to assume the reception timing error as T=5us for MWUS=1, T=2us for MWUS=2, and T=1us for MWUS=4. For the frequency error, we propose to consider 10ppm.

Observation 2: Receiver frequency error and timing error are considered as Rx impairments.

Proposal 6: Set the reception timing error as TO = 5us for MWUS=1, TO = 2us for MWUS=2, TO = 1us for MWUS=4, in the simulation assumption.

Proposal 7: Set the reception frequency error as FO = 10ppm in the simulation assumption.

Proposal 8: RAN4 should not configure the reception timing error and frequency error in TS 38.101-4.

Table 1 summarizes our proposal on simulation assumptions for the wake-up signal reception requirements.

Table 1	Proposed simulation assumption for the wake-up signal reception requirements.

| Parameters | Test 1 (Mwus=1) | Test 2 (Mwus=2) | Test 3 (Mwus=4) |
| --- | --- | --- | --- |
| MWUS | 1 | 2 | 4 |
| Wake-up information length (K) | 1 bit | 2 bits | 5 bits |
| Number of sequences (Nseq) | 1 | 1 | 1 |
| Number of root sequence numbers (Nroot) | 1 | 1 | 1 |
| Number of OFDM symbols for LP-WUS (Nos) | 12 symbols | 12 symbols | 12 symbols |
| Number of bits after line coding (G0 = Nos x MWUS, in TS 38.212 7.4.3) | 12*1 = 12 bits | 12*2 = 24 bits | 12*4 = 48 bits |
| Code rate (K/G0) | 0.083 | 0.083 | 0.10 |
| Reception time offset | 5us | 2us | 1us |
| Reception frequency offset | 10ppm | 10ppm | 10ppm |
| Channel model | Option 1: TDLA30-10 Option 2: TDLC300-100 (Only for FR1) | Option 1: TDLA30-10 Option 2: TDLC300-100 | Option 1: TDLA30-10 Option 2: TDLC300-100 |
| Frequency range and SCS | FR1 SCS=15kHz/30kHz FR2 SCS=120kHz | FR1 SCS=15kHz/30kHz | FR1 SCS=15kHz/30kHz |
| Antenna configuration | 1x1 | 1x1 | 1x1 |
| LP-SS | Configured | Configured | Configured |

## 2.3	Specification structure

Since the Wake-up signal is a new downlink signal defined in Rel-19 LP-WUR WI, RAN4 should create a new clause to specify the wake-up signal reception requirements in TS 38.101-4. Since RAN1 uses ‘Wake-up signal, WUS’ in TS 38.211 7.1.2, RAN4 should use this terminology.

For FR1 conducted requirements, RAN4 should add the new clause in 5.7.

Proposal 9: RAN4 add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4.

If RAN4 also agree to define the wake-up signal reception requirements for FR2, we could also add a new clause in TS38.101-4 7.7.

Proposal 10: If RAN4 agrees to define the wake-up signal reception requirements in FR2, add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4.

# 3	Summary

Observations:

Observation 1: RAN4 RRM has already decided to define LP-SS based LP-RSRP/LP-RSRQ measurement performance requirements.

Observation 2: Receiver frequency error and timing error are considered as Rx impairments.

Proposals:

Proposal 1: For Wake-up signal demodulation performance requirements, the requirements are specified as the SNR to achieve 1 % of missed detection rate (MDR) of Wake-up signal. RAN4 does specify the Wake-up signal demodulation requirements with false alarm rate (FAR).

Proposal 2: RAN4 should configure LP-SS for the WUS reception requirements.

Proposal 3: RAN4 defines the wake-up signal reception requirements with (MWUS=1, K=1), (MWUS=2, K=2), and (MWUS=4, K=5).

Proposal 4: Define the wake-up signal reception requirements with OOK-1 for FR2.

Proposal 5: Evaluate both TDLA30-10 and TDLC300-100 for FR1. Use TDLA30-10 for FR2.

Proposal 6: Set the reception timing error as TO = 5us for MWUS=1, TO = 2us for MWUS=2, TO = 1us for MWUS=4, in the simulation assumption.

Proposal 7: Set the reception frequency error as FO = 10ppm in the simulation assumption.

Proposal 8: RAN4 should not configure the reception timing error and frequency error in TS 38.101-4.

Proposal 9: RAN4 add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4.

Proposal 10: If RAN4 agrees to define the wake-up signal reception requirements in FR2, add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4.

# References

- R4-2508626, “WF for [115][332] NR_LPWUS_Demod”, Moderator (Qualcomm).

- TR 38.869, “Study on low-power wake-up signal and receiver for NR”, 3GPP.

- R1-2504673, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #121”, Moderators (AT&T, NTT DOCOMO, INC.)