3GPP TSG-RAN WG4 Meeting #116	R4-2510841

Bangaluru, India, 25 August – 29 August 2025

Agenda Item:	7.22.9

Source:	Ericsson

Title:	BS Demodulation requirements for Ambient IoT

Document for:	Discussion

# 1	Introduction

This contribution shows our view on the BS demodulation performance requirements for Ambient IoT (A-IoT).

# 2	Rel-19 Ambient-IoT overview

The scope of Rel-19 WI Ambient IoT is as follows [1]:

- Deployment scenario 1 (D1): Indoor micro BS and indoor A-IoT devices.

- Topology 1 (T1): A-IoT device directly communicates with BS (reader)

- Device is a passive device without an amplifier, which relies on backscattering a carrier wave (CW)

- FR1 licensed spectrum in FDD

  - RAN4 RF assumed band 8 (900MHz) as an example band.

Our companion paper [2] summarizes the detailed physical layer design and procedure for Rel-19 Ambient IoT.

# 3	Demodulation performance requirements

## 3.1	Scenario

The WID mentions the Ambient IoT is operated in FR1 FDD licensed spectrum [1], and RAN1 assumes PDRCH/PRDCH uses only SCS=15kHz. If RAN4 defines PDRCH demodulation requirements, RAN4 should define the requirements with FR1 FDD SCS=15kHz only.

Proposal 1: If RAN4 defines PDRCH demodulation requirements, RAN4 should define PDRCH demodulation requirements only with FR1 FDD SCS=15kHz.

The WID also mentions the Rel-19 Ambient IoT targets indoor deployment scenario (D1). In such a scenario, we expect the device to be stationary or low mobility. We therefore propose considering TDLA30-10 for PDRCH demodulation requirements.

Proposal 2: If RAN4 defines PDRCH demodulation requirements, use TDLA30-10.

Since RAN4 RF session assumes Ambient IoT device has one Tx/Rx chain and BS also have one antenna element. We propose the demodulation requirements follow the RF assumption.

Proposal 3: Ambient IoT BS demodulation requirements are set based on 1Tx/1Rx antenna configuration.

## 3.2	Reader (Ambient IoT BS) demodulation

A candidate for the reader demodulation performance requirement is the PDRCH demodulation. As summarized in [2], PDRCH is be used to transmit Message 1 and Message 3. Considering the PDRCH configuration options such as repetition and channel coding, RAN4 should consider defining the PDRCH demodulation requirements for Message 3. We are also open to consider defining the PDRCH demodulation requirements assuming Message 1.

Proposal 4: For BS demodulation performance, RAN4 should consider defining PDRCH demodulation requirements at least with Message 3 reception.

Since PDRCH information includes CRC, the test metric of PDRCH demodulation requirement can be SNR to achieve [1] % of PDRCH block error probability.

Proposal 5: Test metric of PDRCH demodulation requirement is the SNR to achieve [1] % of PDRCH block error probability (BLER).

As we summarized above, RAN1 agreed to apply several configuration options for PDRCH, such as convolutional coding and block repetition. To minimize the number of tests, we prefer to consider at least the following PDRCH configuration:

- With repetition,

- With convolutional coding, and

- With mid-amble.

Proposal 6: Consider applying block repetition, convolutional coding, and mid-amble insertion to PDRCH demodulation requirements.

## 3.3	Specification structure

RAN plenary agreed to create new specifications to define the Ambient IoT device and BS demodulation performance requirements [1]. If RAN4 agree to define the Ambient IoT BS demodulation requirements, they should be specified in the new specification “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception” and “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing” for performance core requirements and conformance test requirements, respectively.

Observation 1: 3GPP creates new specifications for Ambient IoT device and BS performance requirements.

Proposal 7: RAN4 should specify the Ambient IoT BS demodulation requirements in “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception” and “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing” if RAN4 agreed to define the Ambient IoT BS demodulation requirements.

# 4	Summary

Observations:

Observation 1: 3GPP creates new specifications for Ambient IoT device and BS performance requirements.

Proposals:

Proposal 1: If RAN4 defines PDRCH demodulation requirements, RAN4 should define PDRCH demodulation requirements only with FR1 FDD SCS=15kHz.

Proposal 2: If RAN4 defines PDRCH demodulation requirements, use TDLA30-10.

Proposal 3: Ambient IoT BS demodulation requirements are set based on 1Tx/1Rx antenna configuration.

Proposal 4: For BS demodulation performance, RAN4 should consider defining PDRCH demodulation requirements at least with Message 3 reception.

Proposal 5: Test metric of PDRCH demodulation requirement is the SNR to achieve [1] % of PDRCH block error probability (BLER).

Proposal 6: Consider applying block repetition, convolutional coding, and mid-amble insertion to PDRCH demodulation requirements.

Proposal 7: RAN4 should specify the Ambient IoT BS demodulation requirements in “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception” and “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing” if RAN4 agreed to define the Ambient IoT BS demodulation requirements.

# References

- RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.

- R4-2510840, “Device demodulation requirements for Ambient IoT”, Ericsson.