3GPP TSG-RAN WG4 Meeting #116		 			                           R4-2510843
Bengaluru, India, August 25th – 29th, 2025
 
Agenda Item: 7.7.5

Source:	SKY Perfect JSAT

Title: Discussion on UE RF requirements for NR NTN Ku band

Document for:	Discussion

- Introduction

A summary of the agreed Ku band UE/SAN RF requirements discussed at the RAN4#115 meeting is provided in R4-2508778 [1]. This contribution aims to share our perspective on the remaining issues of UE RF requirements that warrant further discussion.

- Analysis and Discussion

Clarification on the minimum peak EIRP requirements

As described in WF in the RAN4#115 WF in R4-2508778[1], it was discussed how to clarify the two minimum peak EIRP requirements for GEO and LEO in the specification. However, it was not completely agreed but agreed that the introduction of new type for LEO only will be considered.

If we proceed with the introduction of a new VSAT type for LEO only as VSAT Type 6, the definition of the NTN VSAT Type and minimum peak EIRP would be like the tables below. We will call these tables Option 1.

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only defined for n509, n508 and n248, n247. |  |  |

Table 1: Option 1 of The definitions of NTN VSAT Types

Table 2 : Option 1 of Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| n509, n508 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| n248, n247 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

On the other hand, if we do not introduce a new VSAT type for LEO only, we will define the VSAT type and minimum peak EIRP as the tables below, which we will call Option 2.

Table 3: Option 2 of The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247. |  |  |

Table 4: Option 2 of Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |  |
| --- | --- | --- | --- |
| n512, n511 | 4 | 70 |  |
|  | 5 | 70 |  |
|  |  | [When communicating with LEO only] | [When communicating with GSO and LEO] |
| n5092, n5082, n2483, n2473 | 4 | 60 | 70 |
|  | 5 | 60 | 70 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |  |

With Option 1, it is possible to define the minimum peak EIRP for each VSAT type, making it possible to have simple and elegant specifications. While the addition of UE Type 6 requires minor updates to TS38.101-5[3], the introduction of Type 6 will enhance clarity in the documentation. For example, in the case of OTA reference sensitivity power level EISREFSENS, there are currently two values for EISREFSENS as same as minimum peak EIRP, one for communicating with LEO and one for communicating with GEO, so introducing a new VSAT Type is also considered an effective solution in terms of the definition of EISREFSENS.

With Option 2, there is no need to consider the impact on other chapters of TS 38.101-5[3] by introducing a new VSAT Type, but the issue of how to specify in the specification the two minimum peak EIRPs for GEO and LEO, which values have been agreed upon in previous meetings, remains.

In our view, there are many benefits to introducing a new VSAT type for LEO only, such as being able to specify a simple specification.

Proposal 1: It is proposed to introduce a new VSAT Type for LEO only in the definition of NTN VSAT Types, like Option1 below.

Table 1: Option 1 of The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only defined for n509, n508 and n248, n247. |  |  |

Table 2 : Option 1 of Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| n509, n508 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| n248, n247 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

RB configuration for EIRP requirements

In the previous meeting, it was agreed that both minimum peak & maximum EIRP is considered to be defined using RB allocation around 10~20MHz as below.

In our understanding, the bandwidth for RB allocation 20MHz is based on the contribution R4-2506585[2] from SKY Perfect JSAT although the contribution mentions the RB allocation only for maximum EIRP not including for minimum peak EIRP.

On the other hand, the bandwidth for RB allocation 10MHz is based on the idea that it should be aligned with the parameter of the transmission bandwidth per UE for coexistence simulation. This idea is used when the RB allocation value, 13 RBs allocation with 120 kHz SCS which is captured in the NOTE of the maximum EIRP for Ka band, was defined.

In addition, we understand that the NOTE of Maximum EIRP for Ka-band regarding the number of RB allocations suggests that maximum EIRP can be scaled up if Channel BW is greater than this number of RB allocations to increase the maximum throughput. However, as for minimum peak EIRP, we think it would be appropriate to specify the number of RBs used for the test, if necessary, but we don't believe scaling according to the number of RBs is required since we would like to allow an operational mode where the bandwidth is increased to get throughput with keeping the EIRP.

Therefore, it is proposed to use around 10MHz for RB allocation bandwidth only for the definition of maximum EIRP considering the advantage of being able to adopt same approach for Ku-band as for the Ka-band standardization.

Proposal 2: It is proposed to use around 10MHz for RB allocation bandwidth only for the definition of maximum EIRP.

Next, it was agreed in the previous meeting that the number of RB allocation is considered to be suitable for DFT-s-OFDM at least. In case we use around 10MHz for the RB allocation bandwidth as the above proposal, we can pick up 60RBs with 15 kHz SCS and 30 RBs with 30kHz regarding FR1-NTN from the table in TS 38.101-5[3], “A.2.2 Reference measurement channels for FR1-NTN FDD”. However, the table in “A.2.3 Reference measurement channels for FR2-NTN FDD” doesn’t specify the suitable number of RBs allocation for FR2-NTN with 120 kHz SCS that matches 10 MHz; the closest values are 1 RB (1RB*12*120 kHz SCS=1.44MHz) or 16 RBs (16RBs*12*120 kHz SCS=23.04MHz). And the number of RB allocation for Ka-band, 13 RBs allocation with 120 kHz SCS, doesn’t follow the values in the table.

The RB allocation for the Ka-band was based on parameters used in coexistence studies. In the coexistence study for the Ku-band, 27 RBs per UE were allocated with 30 kHz SCS, so we can similarly select 27 RBs with 30 kHz SCS per UE as a baseline of the numbers of RB allocation, based on that approach. The number of RBs for other SCSs can also be defined based on the bandwidth obtained from this number of RB allocation.

To summarize above, it is proposed to not follow the Table of Reference measurement channels and to use the following numbers of RB allocation for maximum EIRP.

- 54 RBs allocation with 15 kHz SCS  (54RBs*12*15 kHz SCS=9.72MHz)

- 27 RBs allocation with 30 kHz SCS  (27RBs*12*30 kHz SCS=9.72MHz)

- 6 RBs allocation with 120 kHz SCS  (6RBs*12*120 kHz SCS=8.64MHz)

Therefore, the table of Maximum output power limits for Mobile VSAT can be defined as follows:

Table5 : TS38.101-5 Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.21 |
|  | 5 | 43 | 76.21 |
| n509, n508 | 4 | 47 | 88.22,4 |
|  | 5 | 47 | 88.22,4 |
|  | 6 | [40] | [76.62,4] |
| n248, n247 | 4 | 47 | 88.23,4 |
|  | 5 | 47 | 88.23,4 |
|  | 6 | [40] | [76.63,4] |
| NOTE 1: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. NOTE 2: Maximum EIRP is defined using [6]RBs allocation with 120kHz SCS. NOTE 3: Maximum EIRP is defined using [27]RBs allocation with 30kHz SCS or using [54]RBs allocation with 15kHz SCS. NOTE 4: For land ESIM operating in CEPT regions, the upper limits of EIRPmax is 84.5 dBm. |  |  |  |

Proposal 3: It is proposed to not follow the Table of Reference measurement channels and define the table as below by using RB allocation for maximum EIRP.

Table 5: TS38.101-5 Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.21 |
|  | 5 | 43 | 76.21 |
| n509, n508 | 4 | 47 | 88.22,4 |
|  | 5 | 47 | 88.22,4 |
|  | 6 | [40] | [76.62,4] |
| n248, n247 | 4 | 47 | 88.23,4 |
|  | 5 | 47 | 88.23,4 |
|  | 6 | [40] | [76.63,4] |
| NOTE 1: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. NOTE 2: Maximum EIRP is defined using [6]RBs allocation with 120kHz SCS. NOTE 3: Maximum EIRP is defined using [27]RBs allocation with 30kHz SCS or using [54]RBs allocation with 15kHz SCS. NOTE 4: For land ESIM operating in CEPT regions, the upper limits of EIRPmax is 84.5 dBm. |  |  |  |

OTA reference sensitivity requirement

- Issue 1-3-1: EIS requirement Wayforward: - Consider calculate the EIS requirement for 50MHz channel bandwidth based on the DL SNR target -1 dB (MCS for QPSK 1/3) - At least 2dB implementation margin and will be taken into account. - At least the beam gain reduction around [3.5dB] at the 30~35 degree elevation angle can be taken into account. - Companies are encourage to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting Wayforward: - Companies are encouraged to further check whether the OCNG Pattern is needed for the NTN EIS requirement in the next RAN4 meeting.

At the previous RAN4#115 meeting, as stated in the RAN4#115 WF in R4-2508778[1], the following was agreed.

As stated above, companies are encouraged to provide views on the assumed VSAT G/T value for the EIS requirement in the next RAN4 meeting.  This document proposes the G/T value of an NTN VSAT UE assuming communication with a Ku-band GEO satellite, as well as the resulting EIS reference sensitivity.

In TS 38.101-5[3], the following is specified for the Ka-band OTA reference sensitivity level, and it has been agreed that the calculation for the Ku-band will also be based on a 50 MHz channel bandwidth.

Table 6: Citation from TS38.101-5 Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | Full RB allocation NRB as specified in sub-clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

The OTA reference sensitivity is calculated using the following formula in TR 38.863 [4].

EIS=10*log10(k)+30+10*log10(BW)+SNR+IM-G/T BW is channel bandwidth. Note: It was agreed at the RAN4#115 meeting to set it to 50 MHz. IM is implementation margin. 2dB can be assumed. SNR is the required SNR for demodulation. -1dB can be assumed. Note: It was agreed at the previous meeting to set it to -1dB. G/T: Assumed G/T value K: is Boltzmann constant, 1.38*10-23 J/K

When considering the Minimum Requirement of OTA reference sensitivity, it is necessary to assume and take into account how small the G/T value of a VSAT UE could be.

For Ku-band antennas designed for GEO satellites, commercially available small GEO user terminals - such as Greenerwave's Ku-band GEO terminal [5] and the Kymeta GEO terminal [6] - have a boresight G/T of approximately 9 dB/K. As per the previous agreement, “At least the beam gain reduction around [3.5 dB] at the 30–35 degree elevation angle can be taken into account”, and considering low elevation angles, the G/T is expected to degrade by about 4 dB/K. Therefore, it is proposed to define the EIS reference sensitivity based on an assumed G/T of 5 dB/K for GEO connectivity.

Proposal 4: It is proposed to define the EISREFSENS_50MHz based on an assumed G/T of 5 dB/K for GEO connectivity.

According to the above formula, when G/T is set to 5 dB, the calculated EISREFSENS_50MHz is -125.6 dB.

Proposal 5: It is proposed to set the EISREFSENS_50MHz for GEO to -125.6 dB.

In Proposal 1, it was proposed to add UE Type 6 as a new VSAT type for LEO only. If this is agreed, the EISREFSENS_50MHz value for LEO operation can be defined in the specification of UE Type 6.

On the other hand, UE Type 4 and Type 5 cover both GEO and LEO, but since the EISREFSENS_50MHz value is smaller for GEO, the minimum requirement should be specified based on GEO operation.

Proposal 6: The EISREFSENS_50MHz for UE Type 4 and Type 5 should be specified based on the value for GEO operation.

- Conclusions

Proposal 1: It is proposed to introduce a new VSAT Type for LEO only in the definition of NTN VSAT Types, like Option1 below.

Table 1: Option 1 of The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to n512, n511, n510, n509, n508 and n248, n247; LEO support is only defined for n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only defined for n509, n508 and n248, n247. |  |  |

Table 2 : Option 1 of Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511 | 4 | 70 |
|  | 5 | 70 |
| n509, n508 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| n248, n247 | 4 | 70 |
|  | 5 | 70 |
|  | 6 | 60 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

Proposal 2: It is proposed to use around 10MHz for a common RB allocation bandwidth for the definition of both minimum peak EIRP and maximum EIRP.

Proposal 3: It is proposed to not follow the Table of Reference measurement channels and define the table as below by using RB allocation for maximum EIRP.

Table 5: TS38.101-5 Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510 | 4 | 35 | 76.21 |
|  | 5 | 43 | 76.21 |
| n509, n508 | 4 | 47 | 88.22,4 |
|  | 5 | 47 | 88.22,4 |
|  | 6 | [40] | [76.62,4] |
| n248, n247 | 4 | 47 | 88.23,4 |
|  | 5 | 47 | 88.23,4 |
|  | 6 | [40] | [76.63,4] |
| NOTE 1: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. NOTE 2: Maximum EIRP is defined using [6]RBs allocation with 120kHz SCS. NOTE 3: Maximum EIRP is defined using [27]RBs allocation with 30kHz SCS or using [54]RBs allocation with 15kHz SCS. NOTE 4: For land ESIM operating in CEPT regions, the upper limits of EIRPmax is 84.5 dBm. |  |  |  |

Proposal 4: It is proposed to define the EISREFSENS_50MHz based on an assumed G/T of 5 dB/K for GEO connectivity.

Proposal 5: It is proposed to set the EISREFSENS_50MHz for GEO to -125.6 dB.

Proposal 6: The EISREFSENS_50MHz for UE Type 4 and Type 5 should be specified based on the value for GEO operation.

- References

- R4-2508778, WF for [115][314] NR_NTN_Ku_Band_UE_SAN_RF; Moderator (CHTTL), RAN4#115

- R4-2506585,” Discussion on UE RF requirements for NR NTN Ku band,” SKY Perfect JSAT, RAN4#115

- 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)

- 3GPP TR 38.863, Solutions for NR to support non-terrestrial networks (NTN):Non-terrestrial networks (NTN) related RF and co-existence aspects(Release 18)

- Greenerwave, “Ku GEO Terminal,” Product Page, https://www.greenerwave.com/products/ku-geo-terminal/

- Kymeta, “Hawk u8,” Product Page, https://www.kymetacorp.com/products/hawk-u8