3GPP TSG-RAN WG4 Meeting # 116						            R4-2510844

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.22.3.3

Source: 	OPPO

Title: 	On AIOT CW requirement

Document for:	Approval

- Introduction

During the last RAN4 meeting, the AIOT CW requirement has been discussed and a WF [1] has been agreed. In this discussion paper we try to give further discussion and finalize the device RF requirements.

- Discussion

During the RAN4#115 meeting, the agreed WF is captured as below:

Issue 3-2-1: OFF power

- Agreement:

  - OFF power is -50dBm, RBW is FFS

Issue 3-4: Phase noise

- Agreement:

  - For BS with phase noise cancellation capability, no phase noise requirement is specified.

  - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

Issue 3-6: Spurious emissions

- Agreement:

  - The boundary between spectrum emission mask domain and spurious emission domain for CW node is specified as ΔF = 10 MHz

  - Spurious emissions limits are defined as in Table 2.2-5.

- Table 2.2-5: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |

The OFF power is agreed to be -50dBm but the bandwidth is not decided yet. For a CW node, the transmission bandwidth is hard to decide. From interference perspective, the off power of the CW node will influence the device channel bandwidth. Hence to use the maximum device channel bandwidth to define the OFF power for the CW node seems reasonable.

Proposal 1: To define the RBW of OFF power of the CW node as the maximum device channel bandwidth.

For the phase noise of the CW node, during the last meeting, we have proposed a typical CW phase noise which is based on a quite good CW source of test equipment and again we put it here in figure 1.

Figure 1 Typical phase noise

Some typical points with certain noise figure is captured below using 1GHz data from figure 2.

-77dBc/Hz at 10Hz point

-107dBc/Hz at 100Hz point

-119dBc/Hz at 1kHz point

-122dBc/Hz at 7.5kHz point

-147dBc/Hz at 2880 kHz point

As the largest channel bandwidth can be up to 2.88 MHz according to last RAN1 meeting agreement, hence we propose to also consider the phase noise up to 2.88MHz which will largely influent the reader RX performance. From the above 5 points, the change points of the whole phase noise figure have been all considered. For the exact phase noise value, we see that the phase noise difference between 1GHz to 6GHz can be up to 20dB. From implementation margin perspective, the commercial product for sure will much worse phase noise comparing to a test equipment. So we propose 20dB margin based on the TE CW source to define the phase noise requirement. Consider 33dBm output power of CW node, the phase noise at 2880kHz will be 33-77=-44dBm/100kHz which has already been lower than the spurious requirement as -36dBm/100kHz. The final requirement is as below table 1:

Proposal 2: The phase noise requirement of CW node is proposed to be defined as table 1:

Table 1 Phase noise requirement for the CW node

| Frequency distance toward the CW centre | Phase noise (dBc/Hz) |
| --- | --- |
| 10Hz | -57 |
| 100Hz | -87 |
| 1kHz | -99 |
| 7.5kHz | -102 |
| 2880kHz | -127 |

3	Conclusions

In this paper, we give discussion on the OTA aspect, the observations and proposals are as below:

Proposal 1: To define the RBW of OFF power of the CW node as the maximum device channel bandwidth.

Proposal 2: The phase noise requirement of CW node is proposed to be defined as table 1:

Table 1 Phase noise requirement for the CW node

| Frequency distance toward the CW centre | Phase noise (dBc/Hz) |
| --- | --- |
| 10Hz | -57 |
| 100Hz | -87 |
| 1kHz | -99 |
| 7.5kHz | -102 |
| 2880kHz | -127 |

4 References

[1] R4-2508101	WF on A-IoT BS and CW requirements, Huawei